# WinLALS User's Guide

WinLALS version 1.0 for MS-Windows

WinLALS user's Guide was the update version of LALS users manuals (May 19,2000, R. Chandrasekaran) and the LALS users manual (1983, P. J. Campbell Smith).

1 Introduction

2 Detailed Input Instructions

3 Output Descriptions

4 Users Helpful Description

References

Appendix

A  Summary of Cycle Control Data

B  Examples of Cycle Control Data

C  Error Messages

D  WinLALS Source files and subroutine name

E  Structure and Calling Sequences

F  COMMON Header files

G  Array Specification

H  Sample Input/Output

2001.11.09

Kenji Okada, Ricoh Co. Ltd.

Keiichi Noguchi, Tokyo University of Agriculture and Technology

Kenji Okuyama, Tokyo University of Agriculture and Technology

Contents

1 Introduction	4

1.1 File usage	4

1.2 General Notes	5

2 Detailed Input Instructions	6

2.1  Title 	6

2.2  Molecular symmetry 	6

2.3  Stereochemistry	7

2.4  Root definition	8

2.5  Terminator	8

2.6  Theta equivalence	8

2.7  Terminator	9

2.8  X-ray refinables	9

2.9  Position definitions	9

2.10 Terminator	10

2.11 Equivalent positions in the unit cell	10

2.12 Terminator	11

2.13 Generated molecule definitions	11

2.14 Terminator	12

2.15 Transformations, constraints and restraints	12

2.16 Terminator	18

2.17 Unit cell dimensions	18

2.18 If X-ray data are present, go to 19. If not, supply a terminator 	18

2.19 Cruickshank weights and divisor for Fobs	18

2.20 Scattering factor codes 	19

2.21 X-ray data	20

2.22 Terminator	21

2.23 Rejection factors and selection indicator	21

2.24 Overall elastic weight	21

2.25 If short contact data are present, go to 26. If not, supply a terminator	22

2.26 Contact options	22

2.27 Source unit specifications	22

2.28 Terminator	23

2.29 Target unit specifications	23

2.30 Terminator	23

2.31 Atom redundancy	23

2.32 Terminator	24

2.33 Type I bond list	24

2.34 Terminator	24

2.35 Type II bond list	25

2.36 Terminator	25

2.37 Type III bond list	25

2.38 Terminator	26

2.39 Attractive bond specifications	26

2.40 Attractive bond donors and acceptors	26

2.41 Terminator	27

2.42 Contact limit table	27

2.43 Terminator	27

2.44 Overall contact weights	28

2.45 Individual contact weights	28

2.46 Cycle control line	28

2.47 Cycle data lines	32

2.48 Set and step data (ST)	33

2.49 Terminator, if ST data is present	38

2.50 Model fitting data (MF)	36

2.51 Terminator, if MF data are present	38

3 Output Descriptions	40

3.1 Input Data Listing	40

3.2 Output From CYCLER	41

3.3 Output From COORDS	41

3.4 Output From CONELA	42

3.5 Output From CNTCTS	42

3.6 Output From SFACTR	43

3.7 Output From FILMAT	44

3.8 Output from SOLVE	44

4 Users Helpful Description	46

4.1 Data Notes	46

4.2 Tree Geometry	47

4.3 Generation of Molecular Coordinates	48

4.4 Contact Calculation	48

4.5 Uncertainties	50

References	50

Appendix	51

A	Summary of Cycle Control Data	51

B	Examples of Cycle Control Data	53

C	Error Messages	55

D	WinLALS Source Files, Subroutine Name and Structure	59

E	Array Specification and COMMON Header files	63

F	Sample Input/Output	70

1 Introduction

The Linked-Atom Least-Squares concept was first applied to helical biopolymers by Struther Arnott at Kings College, London, in the mid 1960s. The original programming was in Algol by Alan Wonacott, and that version was subsequently expanded, altered and translated into Fortran by several people including David Dover, David Hukins, Bill Scott and R. Chandrasekaran. An IBM compatible version of the program was developed by Peter Smith at Purdue and completed in 1977. These tasks were accomplished with the aid of helpful suggestions and critical comments from many users. A fairly comprehensive mathematical description of LALS has been published in Acta Crystallographica, A34, 3-11 (1978). Citation of this paper would be appreciated in any published work that uses LALS. The final touch to this UNIX version was given by Thom Hendrixson, Purdue University.

WinLALS was reconstructed from the dimensioned version of LALS of May 19, 2000 that was prepared mainly by Dr. R Chandrasekaran, Purdue University.  Main features are a) WinLALS runs on MS-Windows PC, b) User can selectable the structure factor calculation from Trigonometric or Bessel function, c) Input/Output assignments are amended from LALS2000 version, d) Update file has all output parameters of each cycle with SC cycle control code, e) Add local title line (comment of atomic coordinates) in 2.3 stereochemistry data for user conveniences.  Our group is developing the graphical user interface PreLALS workbench for WinLALS that is able to prepare the input of WinLALS by the conversation with Windows PC.

1.1 File usage

Logical unit assignments and default file names are as follows.

Unit  FileName  	Contents                          	Default File Name

3   -          	Scratch file, binary mode

5   -          	CON (Keyboad)

6   -          	CON (CRT display)

9   user input 	Input 3.Stereo Chemistry + 4.Root 	WinLALSAtom.dat*

Atom                          	usually 15

11   user input 	Input 21.X-ray data               	WinLALSFobs.dat*

usually 15

12   user input 	Input 46-51.Control lines          	WinLALSCont.dat*

usually 15

14   user input 	Error Output File                 	WinLALS.err

15   user input 	Standard input file                	WinLALS.dat

16   user input 	Standard print file                	WinLALS.prn

17   user input 	Standard punch file               	WinLALS.pnc

20   user input 	For Gnu plot file                  	WinLALSGnu.pnc

The above * means that these files are depend on the first character of 1.Title.

1.2 General Notes

a) Formats given for input are to be interpreted in the normal Fortran sense. In particular, if more variables are to be input than the format implies, input continues on the next line. One exception concerns blank fields indicated by, for example, 5x. While this usually means the field is ignored and can contain anything, WinLALS sometimes depends on these fields being blank because they are read with something other than x format. Such instances are noted when they occur.

b) Terminator- means a line with 99999 in columns 1 to 5 and blank elsewhere.

c) Columns beyond 72 are ignored by WinLALS.

d) All input angles are in degrees and should be in the range -180 to +180. Distances are in Angstroms.

e) In many cases a choice is given between atom names and numbers. It is better to use the name. No two atoms may have the same name. If both name and number are given, the number is used and the name ignored.

## 2 Detailed Input Instructions

2.1 Title. Format (18A4)

(CHKAP(i),i=1,18)

This title is printed at the top output

columns

1-72 	CHKAP     : this run title consisting of any desired alphanumeric identification  information

Usually all input data are prepared in the same file (logical unit 15).

If the first character of CHKAP, CHKAP(1)(1:1), is '*', then the following three data sets are read from another input file. The last line of these files should terminate with End of file [EOF].

a) 3.Stereo Chemistry + 4.Root Atom (logical unit 9),

b) 21.X-ray Data (logical unit 11) and

c) 46-51.Control lines (logical unit 12).

2.2 Molecular symmetry. Format (5I5)

NHEL, JHAND, NRES, NTRN, IFLABET

columns

1- 5  	NHEL   	: = 0, =1 for a single helix

= 2 for a double helix having a dyad along the molecular x-axis (i.e. the root atom lies on the dyad) and affects only the structure factor calculation, and speeds up Bessel.

6-10 	JHEL     	: = 0, =1 for right-handed system

= -1 for left-handed system

11-15 	NRES      	: Number of residues per c-repeat, default is 1. Zero implies the default.

16-20 	NTERN    	: Number of helix turns per c-repeat, default is 1. Zero implies the default.

21-25 	IFLABET 	: structure factor calculation

= 0,1 for Trigonometric function use

= 2  for Bessel function use

A six-fold left-handed helix (e.g.) may alternatively be given by JHEL=-1, NRES=6, NTRN =1 or JHEL=1, NRES=6, NTRN=5. The c-repeat is defined as the translation required parallel to the helix axis to superpose one residue on another in the same strand. This may be a multiple of the crystallographic c for certain parallel multiple helices.

Specifying NHEL=2 implies that there is a dyad along the molecular x-axis (i.e. The root atom lies on the dyad). This is of hysterical importance for nucleic acid structures, but more flexibility is obtained by defining a double helix by other means and using NHLE=1. Using NHEL=2 affects only the structure factor calculation, and speeds up only Bessel.

2.3 Stereochemistry. Format (7A10,A2/A4,1x,I5,A4,1x,I5,4F10.0)  Logical unit is 15 or 10.

(TITLE(i),I=1,10)

(CHKNAM(i), LP(i), CHKP(i), LTYPE(i), OCC(i), D(i), TAU(i), THETA(i), i=1,NAT)

NAT+1 lines are required.  The first line is the stereochemistry coordinate title that is prepared by user. The second to NAT line are actual atomic coordinates.

Usually all input data are prepared in the same file (logical unit 15). If the first character of CHKAP (2.1 Title) is ‘*’, this 2.3 Stereochemistry and next 2.4 Root Atom data are read from logical unit 10.

columns, first line. Format (7A10,A2)

1-72 	ITLE     : this stereochemistry title consisting of any desired alphanumeric identification information

columns, second line. Format(A4,1x,I5,A4,1x,I5,4F10.0)

1- 4 	CHKNAM(i): name of this atom

6-10	LP(i)     	: number of precursor atom -or-

11-14	CHKP(i)   	: precursor atom name

16-20	LTYPE(i)  	: atom type

21-30 	OCC(i)    	: occupancy (see 4.1 Data Notes a)

31-40 	D(i)       	: bond length (distance)

41-50 	TAU(i)    	: bond angle

51-60 	THETA(i) 	: dihedral angle in the tree geometry of the I-th atom

One line for each atom until the whole tree structure is defined. The root definition comes next. If there are two or more structures, sets of stereochemistry and root definition are input for each structure. The first LP(1), CHKP(1), D(1), TAU(1), THETA(1), the second TAU(2), THETA(2) and the third THETA(3) are meaningless in every set and ignored.

Numbering starts from 1 for the first atom of the first structure and continues sequentially through the last structure. Atom names may not be wholly blank. THETA(i)s and TAU(i)s may be refined and they are numbered according to the number of the atom of p3(i) for THTA(i)s and p2(i) for TAU(i)s (See 4.2 Tree Geometry)

If the atom names CHNAM(i) are not unique, atoms may not be subsequently referenced by name but only by number. The numbering starts at 1 for the first atom of the first structure and continues sequentially through all the structures. Atom names may not be wholly blank.

Any of the THETA(i) or TAU(i) may be refined, and they are subsequently numbered according to the number of the atom that they define. Thus the first theta is THETA(4) and so on.

2.4 Root definition. Format (5x,I5,A4,6x,4F10.0/3I5)

IRUT, CHKRUT, EPX, EPY, EPZ, S

I1, I2, I3

Two lines are required.

Usually all input data are prepared in the same file (logical unit 15). If the first character of CHKAP (2.1 Title) is ‘*’, previous 2.3 Stereochemistry and this 2.4 Root Atom data are read from logical unit 9.

columns, first line Format(5x,I5,A4,6x,4F10.0)

6-10 	IRUT      	: number of root atom for this structure -or-

11-14 	CHKRUT  	: root atom name

21-30 	EPX       	: Eulerian angle for x-rotation, εx

31-40 	EPY       	: Eulerian angle for y-rotation, εy

41-50 	EPZ      	: Eulerian angle for z-rotation, εz

51-60 	S          	: distance (cylindrical radius) of root atom from the helix axis

columns, second line Format(3I5)

1- 5 	I1        	: rotation to be applied first

6-10 	I2        	: rotation to be applied second

11-15 	I3        	: rotation to be applied third

The choice of root atom is not critical in most cases, but may give some physical  significance to the epsilons. See 4.1 Data Notes f for additional information. I1, I2, I3 will normally be 1, 2, 3, implying that EPX is applied first, EPY second, EPZ third. In general, the rotation I1 is applied first and so on. Repeat input data in items 3 and 4 for defining each additional structure. If there are more structures to be defined, input continues with the next stereochemistry lines, above.

2.5 Terminator of 2.4. Format (I10)

columns

1-10 	ITERM    	: 9999999999

2.6 Theta equivalence. Format (6(I5,A4,1x))

(LPQi1(i),CHLPQi1(i),LPQi2(i),CHLPQi2(i),i=1,NQP)

NQP/3 lines are required.

columns

1- 5  	LPQ11(i)   	: ITH11, number of theta -or-

6- 9  	CHLPQ11(i)	: KTH11, atom name to be equivalenced to

11-15 	LPQ12(i)   	: ITH12, number of theta -or-

16-19 	CHLPQ12(i)	: KTH12, its atom name

21-25 	LPQ21(i)   	: ITH21, number of theta -or-

26-29 	CHLPQ21(i) 	: KTH21, atom name to be equivalenced to

31-35 	LPQ22(i)   	: ITH22, number of theta -or-

36-39 	CHLPQ22(i)	: KTH22, its atom name

41-45 	LPQ31(i)   	: ITH31, number of theta -or-

46-49 	CHLPQ31(i) 	: KTH31, atom name to be equivalenced to

51-55 	LPQ32(i)   	: ITH32, number of theta -or-

56-59 	CHLPQ32(i)	: KTH32, its atom name

Theta equivalence allows the initial difference between any two thetas to be exactly maintained throughout a refinement, without the use of constraints. This is useful for describing, for example, an unrefined group or ring along a chain. Three equivalences per line are able to input. Only the second theta in the equivalence may be refined. The program will accept without comment unused equivalences.

Note that, with the root atom considered fixed in space, no pair of equivalenced thetas may affect any common atom. I.e. they must be in different branches of the tree.

2.7 Terminator of 2.6. Format (I10)

columns

1-10 	ITERM    	: 999999999

2.8 X-ray refinables. Format (5F10.0)

SCLE0, ATTN0, SCLE1, ATTN1, EDM

columns

1-10 	SCLE0   	: Initial scale factor for Fobs, i.e. we are fitting Scle0*Fobs to Fcalc

11-20 	ATTN0    	: Initial overall isotropic attenuation (temperature) factor for Bragg Fobs data

21-30 	SCLE1    	: Initial scale factor for continuous Fobs X-ray data

31-40 	ATTN1    	: Initial attenuation factor for continuous data

41-50 	EDM      	: Electron density multiplier for water weighted scattering factors (= 1.0 by default)

The calculated structure factors are multiplied by exp−ATTNsinθλ2. If there are no X-ray data, presence of the values in the line does not interfere with other refinement.

2.9 Position definitions. Format (6F10.0)

(FMU(i), U(i), V(i), W(i), POCC(i), PSNS(i), i = 1, NMOL)

NMOL lines are required.

columns

1-10 	FMU(i)    	: Axial orientation angle for this position

11-20 	U(i)       	: fractional x-coordinate of helix axis

21-30 	V(i)       	: fractional y-coordinate of helix axis

31-40 	W(i)       	: fractional displacement of root atom along z from the unit cell base

41-50 	POCC(i)   	: site occupancy (multiplier to all the atomic occupancies in item 3)

51-60 	PSNS(i)   	: = +1.0 for "up"

= -1.0 for "down" sense of the ith helix.

If NHEL = 2, only up molecules are permitted.

This completes the input of all the refinable parameters. Those other than the thetas and Taus are termed "other parameters" and are numbered in a single sequence as follows. Other(1) = SCLE0, Other(2) = ATTN0, Other(3) = SCLE1 and Other(4) = ATTN1.

This numbering is retained even if there are no X-ray data, but in that case Other(1) to Other(4) cannot be refined. Other(5) to Other(8) are epsilon-x, epsilon-y, epsilon-z and S for the first structure, Other(9) to Other(12) are epsilon-x, epsilon-y, epsilon-z and S for the second, and so on till the m-th structure. Other(4+4*m+1) to Other(4+4*m+4) are FMU(1), U(1), V(1), W(1) for first structure, Other(4+4*m+5) to Other(4+4*m+8) are FMU(2), U(2), V(2), W(2) for second structure and so on.

2.10 Terminator of 2.9. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.11 Equivalent positions in the unit cell. Format (6F10.0) (If none, go to 2.12)

(SSYM(i,j),i=1,12),j=1,NEQUIV)

NEQUIV lines are required.

columns, first line

1-10 	SSYM(1,j)  	: a1

11-20 	SSYM(2,j)  	: a2

21-30 	SSYM(3,j)  	: a3

31-40 	SSYM(4,j)  	: a4

41-50 	SSYM(5,j)  	: a5

51-60 	SSYM(6,j)  	: a6

columns, second line

1-10 	SSYM(7,j)  	: a7

11-20 	SSYM(8,j)  	: a8

21-30 	SSYM(9,j)  	: a9

31-40 	SSYM(10,j)	: b1

41-50 	SSYM(11,j) 	: b2

51-60 	SSYM(12,j)	: b3

This defines an equivalence of International Tables, i.e. the equivalence moves an atom positioned at fractional (x, y, z), it to (xx, yy, zz) given below in a matrix notation.

xxyy

Note that a3, a6, a7 and a8 must be zero and a9 must be +1.0 or -1.0, i.e. the only rotation other than that parallel to the helix axis is one of 180 degrees about a line perpendicular to it.

If no equivalence is given, the identity operation is assumed (a1 = a5 = a9 = 1.0). If it is not assumed and must be specified explicitly if any others are specified.

2.12 Terminator of 2.11. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.13 Generated molecule definitions. Format (4I5)

(IPOS(i), IEQV(i), ISTR(i), MHEL(i), i = 1, NMOL)

NMOL lines are required.

columns

1- 5  	IPOS(i)  	: position number of this i-th molecule

6-10 	IEQV(i)   	: equivalence number of this i-th molecule

11-15 	ISTR(i)   	: structure number of this i-th molecule

16-20 	MHEL(i)   	: = 0 if helical symmetry applies to this i-th molecule

= 1 if not, to this i-th molecule

Positions, equivalences and structures, and similarly the generated molecules, are numbered in the same order they are defined above. If no definition is given, the program assumes every possible combination (all with MHEL(i) = 0) such that the structure number varies the fastest, then the equivalence, finally the position.

The Bessel routine cannot be used for non-helical molecules.

2.14 Terminator of 2.13. Format (I10)

columns

1-10 	ITERM    	: 999999999

2.15 Transformations, constraints and restraints. Header Format (A3,6x,I1,6F10.0)

CHKSRT, NP, X1, X2, X3, X4, X5, X6

Each constraint or restraint definition consists of a header line that specifies the type of constraint or restraint, and possibly the following lines dependent on the type.

columns

1- 3  	CHKSRT 	: a three-letter code specifying the type of constraint or restraint. The meanings of the other variables depend on the type as given below

ANG 	= Bond angle relationship (two lines input).

COI 	= Coincidence relationship (two lines input).

COL 	= Collinearity relationship (four lines input).

COO 	= Coordinate relationship (NP+1 lines).

DIH 	= Dihedral angle relationship (three lines input).

DIS 	= Distance relationship (two lines input).

EQU 	= General transformation (two lines input).

GAM 	= Gamma relationship (two lines input).

PAR 	= Parameter relationship (NP+1 lines).

TRA 	= Helix transformation or cell translation (one header line format input).

10    	NP      	: used only by types "PAR" and "COO" to indicate how many elements are involved in the relationship.

11-20 	X1       	: the weight of a restraint or zero for a constraint

21-30 	X2       	: the desired value for distance or angle

The exceptions of EQU and TRA are the input for constraints and restraints. In all cases, a constraint is constructed if x1 is zero; otherwise, it is a restraint with weight x1.

Several types use "standard atom" format to read input off the following lines. These lines contain (IAT(i),CHKAT(i),ITM(i),IML(i),i=1,NAT) in format (10x, 3(I5, A4, 1x, 2I5)).

columns

11-15 	IAT1     	: first atom number -or-

16-19 	CHKAT1 	: first atom name

21-25 	ITM1    	: transform number of first atom or zero if none

26-30 	IML1     	: molecule number of first atom

31-35 	IAT2     	: second atom number -or-

36-39 	CHKAT2 	: second atom name

41-45 	ITM2    	: transform number of second atom or zero if none

46-50 	IML2    	: molecule number of second atom

51-55 	IAT3     	: third atom number -or-

56-59 	CHKAT3 	: third atom name

61-65 	ITM3    	: transform number of third atom or zero if none

66-70 	IML3    	: molecule number of third atom

Atom involved in a constraint may be on any generated molecule. Provision exists also for atom in a different helical repeat and/or different unit cell, or for a new atom related to an existing atom by a complicated matrix relationship. Before they are called in the constraint/restraint, such transforms must be sequentially defined by types "TRA" and "EQU" as follows.

a)  Bond angle relationship (two lines input).

columns, first line. Format(A3,7x,2F10.0)

1- 3  	CHKSRT 	: ANG

10    	NP       	: (ignored)

11-20 	X1        	: weight or zero

21-30 	X2        	: desired value of bond angle

columns, second line. Format (10x, 3(I5, A4, 1x, 2I5))

11-15 	IAT1     	: first atom number -or-

16-19 	CHKAT1 	: first atom name

21-25 	ITM1    	: transform number of first atom or zero if none

26-30 	IML1     	: molecule number of first atom

31-35 	IAT2     	: second atom number -or-

36-39 	CHKAT2 	: second atom name

41-45 	ITM2    	: transform number of second atom or zero if none

46-50 	IML2    	: molecule number of second atom

51-55 	IAT3     	: third atom number -or-

56-59 	CHKAT3 	: third atom name

61-65 	ITM3    	: transform number of third atom or zero if none

66-70 	IML3    	: molecule number of third atom

Second line contains three atoms in standard atom format. This relationship is to maintain the bond angle 1-2-3 close to X2.

b)  Coincidence relationship (two lines input).

columns, first line. Format(A3,7x,F10.0)

1- 3  	CHKSRT 	: COI

10    	NP       	: (ignored)

11-20 	X1       	: weight or zero

columns, second line. Format (10x, 2(I5, A4, 1x, 2I5))

11-15 	IAT1     	: first atom number -or-

16-19 	CHKAT1 	: first atom name

21-25 	ITM1    	: transform number of first atom or zero if none

26-30 	IML1     	: molecule number of first atom

31-35 	IAT2     	: second atom number -or-

36-39 	CHKAT2 	: second atom name

41-45 	ITM2    	: transform number of second atom or zero if none

46-50 	IML2    	: molecule number of second atom

Second line contains two atoms in standard atom format. This defines 3 relationships, between x, y and z coordinates of the two atoms that are to be made coincident.

c)  Collinearity relationship.

columns, first line. Format(A3,7x,F10.0)

1- 3  	CHKSRT  	: COL

10    	NP        	: (ignored)

11-20 	X1         	: weight or zero

columns, second to five lines. Format (10x, I5,A4, 1x, 2I5)

11-15 	IAT     	: atom number -or-

16-19 	CHKAT  	: its atom name

21-25 	ITM     	: transform number or zero if none

26-30 	IML     	: molecule number, for the ith atom

Second to five lines contain the three atoms in standard atom format. This relationship is to keep the three atoms collinear. The order of the atoms is immaterial.

The parametric constraint is given by x, y, z permutations of G=(X1-X2)/(X2-X3) - (Y1-Y2)/ (Y2-Y3). The order of the atoms is immaterial.

d)  Coordinate relationship

columns, first line. Format(A3,6x,I1,2F10.0)

1- 3  	CHKSRT  	: COO

10    	NP        	: number of coordinates involved (must be 1,2,3 or 4)

11-20 	X1        	: weight or zero

21-30 	X2        	: desired value (see below).

columns, second to NP+1 lines. Format (10x, I5, A4, 1x, 2I5, 5x, A1, 4x, F10.0)

11-15 	IAT(i)     	: atom number whose coordinate is affected -or-

16-19 	CHKAT(i) 	: its atom name

21-25 	ITM(i)    	: transform number

26-30 	IML(i)     	: molecule number

36    	CHKEL(i) 	: X, Y or Z for the respective Cartesian coordinate,

R or P   for the radial or angular cylindrical coordinate

41-50 	COEF(i)   	: coefficient of the coordinate

This relationship is to closely maintain X2=COEF(i)∗COO(i). If the last coordinate involved has CHKEL = P, X2 and X1 should be in degrees**(-2) and degrees. The units assumed for each COEF(i) agree with the type of the coordinate. While the R coordinate of an atom is independent of ITM(i) and IML(i), valid values for them must be given for operational reasons.

e)  Dihedral angle relationship (three lines input).

columns, first line. Format(A3,7x,2F10.0)

1- 3  	CHKSRT	: DIH

10    	NP      	: (ignored)

11-20 	X1       	: weight or zero

21-30 	X2       	: desired dihedral angle

columns, second line. Format (10x, 3(I5, A4, 1x, 2I5))

11-15 	IAT1     	: first atom number -or-

16-19 	CHKAT1 	: first atom name

21-25 	ITM1    	: transform number of first atom or zero if none

26-30 	IML1     	: molecule number of first atom

31-35 	IAT2     	: second atom number -or-

36-39 	CHKAT2 	: second atom name

41-45 	ITM2    	: transform number of second atom or zero if none

46-50 	IML2    	: molecule number of second atom

51-55 	IAT3     	: third atom number -or-

56-59 	CHKAT3 	: third atom name

61-65 	ITM3    	: transform number of third atom or zero if none

66-70 	IML3    	: molecule number of third atom

Second line contains the first three atoms in standard atom format.

columns, third line. Format (10x, 3(I5, A4, 1x, 2I5))

11-15 	IAT4     	: fourth atom number –or-

16-19 	CHKAT4 	: fourth atom name

21-25 	ITM4     	: transform number of fourth atom or zero if none

26-30 	IML4     	: molecule number of fourth atom

Third line contains the fourth atom in standard atom format. This relationship is to maintain the dihedral angle 1-2-3-4 close to X2.

f)  Distance relationship (two lines input).

columns, first line. Format(A3,7x,2F10.0)

1- 3  	CHKSRT  	: DIS

10    	NP        	: (ignored)

11-20 	X1        	: weight or zero

21-30 	X2        	: desired distance

columns, second line. Format (10x, 2(I5, A4, 1x, 2I5))

11-15 	IAT1     	: first atom number -or-

16-19 	CHKAT1 	: first atom name

21-25 	ITM1    	: transform number of first atom or zero if none

26-30 	IML1     	: molecule number of first atom

31-35 	IAT2     	: second atom number -or-

36-39 	CHKAT2 	: second atom name

41-45 	ITM2    	: transform number of second atom or zero if none

46-50 	IML2    	: molecule number of second atom

Second line contains two atoms in standard atom format. This relationship is to maintain the distance between two atoms close to X2.

g)  General transformation (two lines input).

columns, first line. Format(A3,7x,6F10.0)

1- 3  	CHKSRT 	: EQU

10    	NP        	: (ignored)

11-20 	X1        	: This defines a transform to generate an atom at xx, yy, zz

21-30 	X2        	: from one at x, y, z (Cartesian). Thus

31-40 	X3        	:    (xx)    (x1 x2 x3) (x) 	(x10)

41-50 	X4        	:    (yy) =  (x4 x5 x6)*(y) + 	(x11)

51-60 	X5        	:    (zz)    (x7 x8 x9) (z)   	(x12)

61-70 	X6        	:

columns, second line. Format (10x, 6F10.0)

11-21 	X7        	:

21-31 	X8        	:

31-41 	X9        	:

41-51 	X10       	:

51-61 	X11       	:

61-71 	X12       	:

Standard atom input (above) then allows a transform number to be specified, or zero if a transform is not needed.

h)  Gamma relationship

columns, first line. Format(A3,7x,2F10.0)

1- 3  	CHKSRT  	: GAM

10    	NP        	: (ignored)

11-20 	X1         	: weight or zero

21-30 	X2        	: desired value for gamma

columns, second line. Format (10x, 3(I5, A4,1x))

11-15 	IAT1    	: first atom number -or-

16-19 	CHKAT1 	: its name

21-25 	IAT2    	: second atom number -or-

26-29 	CHKAT2 	: its name

31-35 	IAT3     	: third atom number -or-

36-39 	CHKAT3 	: its name defining the plane

Gamma, the angle between the normal to this plane within a molecule and the helix axis, is always in the range 0 to 90 degrees.

i)  Parameter relationship.

Columns, first line. Format(A3,6x,I1,2F10.0)

1- 3  	CHKSRT  	: PAR

10    	NP        	: number of parameters involved (must be 2, 3 or 4)

11-20 	X1        	: weight or zero

21-30 	X2        	: desired value (see below)

columns, second to NP+1 lines. Format (10x, A2, 5x, I3, F10.0)

11-12 	CHKPAR(i)	: TH for a theta, TA for a Tau, OT for an "other parameter"

18-20 	IPAR(i)    	: number of the parameter

21-30 	COEF(i)   	: coefficient of the parameter

This relationship is to closely maintain X2=COEF(i)∗PAR(i). If the parameter is an angle, X2 should be in degrees and x1 in degrees**(-2). Each coefficient should be in degrees**(-1) for angle and Angstroms**(-1) for distance.

j) Helix transformation or cell translation (one header line format input).

columns. Format(A3,7x,3F10.0)

1- 3  	CHKSRT 	: TRA

10    	NP      	: (ignored)

11-20 	X1       	: number of cell edge a translation

21-30 	X2        	: number of cell edge b translation

31-40 	X3        	: number of helix repeat operation. Note that X1, X2, X3 may be zero, negative or positive. They will normally be integral.

2.16 Terminator of 2.15. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.17 Unit cell dimensions. Format (6F10.0)

(UCELL(i),i=1,6)

columns

1-10 	UCELL(1) 	: a, length of a edge of the unit cell, in Angstroms

11-20 	UCELL(2) 	: b, length of b edge

21-30 	UCELL(3) 	: c, length of c edge

31-40 	UCELL(4) 	:α, angle between b and c, in degrees

41-50 	UCELL(5) 	:β, angle between c and a

51-60 	UCELL(6) 	:γ, angle between a and b

By convention (and necessarily for this program) c (or possibly c*) is parallel to the helix axis. A blank line defaults to a = b = c = 1.0, α=β=γ=90.0and this may be used for a non-crystallographic study.

2.18 If X-ray data are present, go to 2.19. If not, supply a terminator and proceed to 2.24 Overall elastic weight.

2.19 Cruickshank weights and divisor for Fobs. Format (5F10.0)

(CRUIK(i),i=1,3), CRUICK(5), CRUICK(4)

columns

1-10 	CRUICK(1)	: A

11-20 	CRUICK(2)	: B

21-30 	CRUICK(3)	: C

31-40 	CRUICK(5)	: D, coefficient for continuous data [weight = 1/D]

41-50 	CRUICK(4)	: SUB, subdivision factor for statistical analysis of Fobs

(= 2.0 by default)

A, B and C coefficients for Bragg data [weight w = 1/(A + B * Fobs + C * Fobs**2)]. Thus a constant weight (1/A) can be generated by setting B=C=0.0. If A (or D) is negative, the weight becomes (-w)*individual weight of each Bragg (or continuous) datum. A = 0 (or D = 0) is equivalent to A = -1 (or D = -1). If a value A just greater than 1.0 (say 1.1) is given to subdivision, the program resets it to the minimum possible value. For details see 3.6 structure factor statistics in the output description. SUB (which defaults to 2.0 if omitted) is subdivision factor used in the statistical analysis of the Fobs values.

2.20 Scattering factor codes. Format (12I5)

(LSCT(i),i=1,NTYPE)

NTYPE/12 lines are required. LSCT(i)=-1 is terminator.

columns

1- 5  	LSCT(1)   	: a 4-digit code "CBAA"

6-10 	LSCT(2)   	: a 4-digit code "CBAA"

11-15 	LSCT(3)   	: a 4-digit code "CBAA"

....

55-60 	LSCT(12)  	: a 4-digit code "CBAA"

LSCT(i) is a 4-digit code "CBAA" meaning that atoms of type I (given on 2.3 stereochemistry lines) have a scattering effect as follows.

C = 0 normal atomic scattering, do to allow for water in the gaps of the structure

= 1 gives water weighting with van der Waals volume as in the Smith and Arnott paper

= 2 gives the obsolete water weighting scheme of Arnott and Hukins, J. Mol. Biol. (1973) 81, 93-105

= 3 gives water weighting as in C = 1 but with excluded volume given by Langridge et al. J. Mol. Biol. (1960) 2, 38-64

= 4 gives the recommended water weighting given by Arnott et al. J. Mol. Biol. (1981) 149, 489-505. Built in excluded volumes (Angstroms**3) are: hydrogen = 5.15, carbon = 16.44, nitrogen = 2.49, oxygen = 9.13, oxygen 1/2- = 5.00, oxygen - =  5.00, phosphorus = 23.50, sulfur = 25.50.

B = number of bound hydrogen atoms scattering from the same atomic center.

AA = 00 null atom (also implied by type of 0 or zero occupancy)

= 01 hydrogen

= 02 carbon

= 03 nitrogen

= 04 oxygen, neutral

= 05 oxygen, half negative charge

= 06 oxygen, one negative charge

= 07 phosphorus

= 08 sulfur

= 09-99 may be assigned by the user for other species

If any AA exceeds 08, there follow for the additional species.

((X(i,j),j=1,9), RAD(i), HBND(i),i=9, NSPEC)  Format (6F10.0/5F10.0)

((X(i,j),j=1,9)	= set of 9 numbers from (or analogous to) the International Tables, Vol IV, pp. 99-101,

RAD(i) 	= van der Waals radius of the atom (needed only for water weighting)

HBND(i)	= length of the H-atom bond (needed only if hydrogen atoms are to be attached and water weighting applied)

The first two lines refer to species 09, the next two to 10 and so on.

Note that the water weighting schemes for C = 3 and 4 are currently valid only for B = 0 and AA = 00 through 08.

2.21 X-ray data. Format (A:4I5, 2F10.0, B:2I5,10X,F10.0 or C:I5,15X,3F10.0)

For Trigonometric calculation, 2.21A data are required. For Continuous transform calculation, 2.21B or 2.21C data are required.

Usually all input data are prepared in the same file (logical unit 15). If the first character of CHKAP (2.1 Title) is ‘*’, this 2.21 X-ray data are read from logical unit 11.

2.21A Observed structure factors (Bragg data)

(ITLHH(i), ITLHK(i), ITLHL(i), ITLHM(i), FOBS(i), FWT(i), i = 1, NREFL)

NREFL lines are required.

columns, Format(4I5,2F10.0)

1- 5  	ITLHH(i)  	: IH, Miller index h, IH1000

6-10 	ITLHK(i)  	: IK, Miller index k

11-15 	ITLHL(i)  	: IL, Miller index l

16-20 	ITLHM(i) 	: IM, multiplicity of reflection, zero being taken as 1

21-30 	FOBS(i)  	: = 0, if this reflection is part of a composite spot with the next reflection(s)

> 0, observed structure amplitude if this is the last or only reflection in this spot

< 0, negative value if the observed structure amplitude is to be set at zero

31-40 	FWT(i)    	: = 0 means unit weight by default

> 0, weight to be applied to this spot if this is the last or only reflection in this spot

< 0, negative value to signals an unobserved spot

2.21B Indicate Continuous Diffraction 1

(ITLHH(i), ITLHK(i), FOBS(i),i=1,NREFL)

NREFL lines are required.

Only a Bessel routine can handle these data under IFLABET=2 (see 2.2)

columns, Format(2I5,10X,F10.0)

1- 5  	ITLHH(i)	: 1000

6-10 	ITLHK(i)  	: 1000 * RR (cylindrical polar radius in reciprocal space)

21-30 	FOBS(i)  	: must be greater than zero in each datum. If this value less than 0.001, this reflection treats as zero.

2.21C Indicate Continuous Diffraction 2

ITLHH, RSTART,RSTEP,REND

Only a Bessel routine can handle this datum under IFLABET=2 (see 2.2)

Continuous diffraction data from RRstart to RRend (step is RRstep) are generated.

columns, Format(I5,15X,3F10.0)

1- 5  	ITLH  	: 2000

21-30 	RRSTART 	: RR (cylindrical polar radius in reciprocal space) start value

31-40 	RRSTEP  	: RR step value

41-50 	RREND  	: RR end value

2.22 Terminator of 2.21. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.23 Rejection factors and selection indicator. Format (3I5)

IREJ, IREJT, IABN

columns

1- 5  	IREJ    	: r1, then reject from the least squares any spot for which r1 * Fcalc is less than Fobs

6-10 	IREJT  	: r2, then reject from the least squares any unobserved spot for which Fcalc is less than r2 * Fobs

10-15 	IABN   	: = 0, use all Bessel orders between -20 and +20 in SF calculation

= 1, use m = 0 in the selection rule for Bragg data and m not equal to 0 for continuous data.

Spots with zero weight are always rejected.

2.24 Overall elastic weight. Format (F10.0)

ELAWT

columns

1-10 	ELAWT   	: Individual weights of elasticated parameters (see 2.46 Cycle control line), model fitting data (see 2.50 Model fitting data) and restraints (see 2.15 Transformations, constraints and restraints) are further multiplied by ELAWT

2.25 If short contact data are present, go to 26. If not, supply a terminator and proceed to 46.

2.26 Contact options. Format (2I5)

NMAXC, MINCP

columns

1- 5  	NMAXC   	: maximum number of contacts to find. Any beyond this number will be ignored with a non-fatal warning message. Zero implies the default 400.

6-10 	MINCP   	: > 0, contacts differing from the limiting value by less than MINCP/100.0 Angstrom will not be printed

= 0, contacts differing by less than 0.4 Angstrom *100 will not be printed. Zero implies the default 40.

= -1, all contacts will be listed

Note that MINCP affects only the output listing of contacts. They are still applied whatever the difference is.

2.27 Source unit specifications. Format (4I5)

((LSU(i,j),i=1,4), j=1,NSM)

NSM lines are required.

columns

1- 5  	LSU(1,j)  	: IM1, molecule number of source element

6-10 	LSU(2,j)  	: IM2, 	= 0, only IM1 is to be used

0, molecules IM1 to IM2 inclusive are included

11-15 	LSU(3,j)  	: IR1, first residue number

16-20 	LSU(4,j)  	: IR2, last residue number, i.e. residues IR1 through IR2 are used

Note that IM2  IM1 and IR1  IM1 (if IM2 is not zero) are necessary.

Residue 0 is the one constructed from the input (i.e. it has its root atom at z = w(i) * c of 2.9 Position definitions). Residue +1 is derived from the base molecule residue proceeding in the +z direction, and residue -1 conversely. Positive residue numbers therefore increase towards +z in the generated molecule if it has sense "up" and its equivalence parameter PSNS(i) (2.9 Position definitions) is +1.0, and so on.

Positive residue numbers increase towards -z in the generated molecule whose sense is "down" (either by positioning or equivalence or any other means). Negative molecule numbers may be used to specify the (NHEL = 2) double helix mate.

With each line of source specification is associated an automatic number termed the S-code whose purpose will be described below (2.29 Target unit specifications) with T-code. The nth line has an S-code of 2**(n-1), i.e. they are successively 1, 2, 4, 8, 16 ...

2.28 Terminator of 2.27. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.29 Target unit specifications. Format (8I5, I10)

((LTU(i,j), i=1,9), j=1,NTM)

NTM lines are required.

columns

1- 5  	LTU(1,j)  	: IM1, molecule number of target element

6-10 	LTU(2,j)  	: IM2, 	= 0, only IM1 is to be used

0, molecules IM1 to IM2 inclusive are included

11-15 	LTU(3,j)  	: IR1, first residue number

16-20 	LTU(4,j)  	: IR2, last residue number, i.e. residues IM1 through IM2 are used

21-25 	LTU(5,j)  	: IA1, number of unit cell a edge translations to be applied

26-30 	LTU(6,j)  	: IB1, number of b translations

31-35 	LTU(7,j)  	: IC1, number of c translations

36-40 	LTU(8,j)  	: multiplicity of interactions involving this target element

41-45 	LTU(9,j) 	: T-code, being the sum of the S-codes of source elements whose interactions with this target element is considered. Zero T-code implies all the source elements.

Any target unit specification may be followed by:

Columns, Format(20x,3I5)

21-25 	IA2(i)  	: IA2, number of unit cell a edge translations to be applied

26-30 	IB2(i)    	: IB2, number of b translations

31-35 	IC2(i)    	: IC2, number of c translations

This implies that all numbers of a translation from IA1 to IA2 inclusive are to be used, and so on.

2.30 Terminator of 2.29. Format (I10)

columns

1-10 	ITERM  	: 999999999

2.31 Atom redundancy. Format (I5, A4, 1x, I5, A4, 1x, I5)

LABEL(1,i), CHKA1(i), LABEL(2,i), CHKA2(i), LABEL(3,i), i=1,NADEL)

NADEL lines are required.

columns

1- 5  	LABEL(1,i)	: IAT1, atom number to be made redundant -or-

6- 9  	CHKAT1(i) 	: its atom name

11-15 	LABEL(2,i) : IAT2, atom number which IAT1 duplicates -or-

16-29 	CHKAT2(i) 	: its atom name

21-25 	LABEL(3,i) : IRES, number of residues IAT2 must be displaced to make it IAT1.

If only the first atom is specified, it becomes a dummy atom for contact purposes and is completely ignored. If both atoms are given, considered to duplicate the second, the first is ignored. However, any bond implied by precursion of or to the first will apply to the second. A non-zero value of IRES is used when the first atom duplicates a helical repeat of the second  (i.e. IRES is usually +1 or -1 in this case).

Note that a zero occupancy atom is ignored in the contact search, but its precursion-implied bonds still exist. This is not true of atoms dummied here with IAT2 omitted.

2.32 Terminator of 2.31. Format (I10)

columns

1-10 	ITERM  	: 999999999

2.33 Type I bond list. Format (I5, A4, 1x, I5, A4)

(LBDIN1(1,i), CHKAT1(i), LBDIN1(2,i), CHKAT2(i), i = 1, NIB)

NIB lines are required.

columns

1- 5  	LBDIN1(1,i) 	: IAT1, number of atom at one end of bond -or-

6- 9  	CHKAT1(i) 	: its atom name

11-15 	LBDIN1(2,i) 	: IAT2, number of atom at other end -or-

16-19 	CHKAT2(i) 	: its atom name

Type I bonds are bonds within one residue. Some of them may already have been  accounted by atom redundancies (above). Those between atoms and their precursors should not be input here.

2.34 Terminator of 2.33. Format (I10)

columns

1-10 	ITERM 	: 999999999

2.35 Type II bond list. Format (I5, A4, 1x, I5, A4, 1x, I5)

(LBDIN2(1,i),CHKAT1(i),LBDIN2(2,i),CHKA2(i),LBDIN2(3,i), i=1,NIIB)

NIIB lines are required.

columns

1- 5  	LBDIN2(1,i) 	: IAT1, number of atom at one end of bond -or-

6- 9  	CHKAT1(i) 	: its atom name

11-15 	LBDIN2(2,i)	: IAT2, number of atom at other end -or-

16-19 	CHKAT2(i) 	: its atom name

21-25 	LBDIN2(3,i)	: IRES, residue number of IAT2 if IAT1 is on residue 0

Type II bonds are between atoms on different residues in a molecule. Those already implied by atom redundancy list need not be repeated here.

2.36 Terminator of 2.35. Format (I10)

columns

1-10 	ITERM     	: 999999999

2.37 Type III bond list. Format (I5, A4, 1x, 3I5, A4, 1x, 5I5)

(LBDIN3(1,i),CHKAT1(i),LBDIN3(3,i),LBDIN3(4,i),LBDIN(5,i),CHKA2(i),LBDIN3(6,i),LBDIN(7,i),LBDIN(8,i),LBDIN(9,i),LBDIN(10,i),LBDIN(11,i),i=1,NIIIB)

NIIIB lines are required.

columns

1- 5  	LBDIN3(1,i)	: IAT1, number of atom at one end of bond -or-

6- 9  	CHKAT1(i) 	: its atom name

11-15 	LBDIN3(3,i)	: IRES1, residue number of IAT1

16-20 	LBDIN3(4,i)	: IMOL1, molecule number of IAT1

21-25 	LBDIN3(5,i)	: IAT2, number of atom at other end -or-

26-29 	CHKAT2(i)	: its atom name

31-35 	LBDIN(7,i)	: IRES2, residue number of IAT2

36-40 	LBDIN(8,i)	: IMOL2, molecule number of IAT2

41-45 	LBDIN(9,i)	: IA, number of A edge displacements separating IAT2 from the source unit

46-50 	LBDIN(10,i)	: IB, number of similar B edge displacements

51-55 	LBDIN(11,i) 	: IC, number of similar C edge displacements

Type III bonds are between different molecules.

2.38 Terminator of 2.37. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.39 Attractive bond specifications. Format (6F10.0). If none, go to 2.41.

((HBPAR(i,j), i=1,6) ,j=1,NHB)

NHB data sets (2.39 + 2.40) are required.

columns

1-10 	HBPAR(1,j)	: RMI, minimum distance for attractive bond application

11-20 	HBPAR(2,j)	: RMA, maximum distance for attractive bond application

21-30 	HBPAR(3,j)	: REQ, optimum distance for attractive bond

31-40 	HBPAR(4,j)	: W, weight to be given to interaction, in degrees**(-2)

41-50 	HBPAR(5,j)	: ANG, angle desired for precursor-donor-acceptor

51-60 	HBPAR(6,j)	: DEV, deviation from ANG to be tolerated

If ANG and DEV are omitted, no angle test is made. If the angle test fails, or the distance is between RMI and RMA, the interaction is treated as a normal contact. It is necessary that RMI is REQ  RMA  DLIM (given 2.42 Contact limit table). W is affected by the overall contact weight (see 2.44 Overall contact weights).

2.40 Attractive bond donors and acceptors. Format (6(I5, A4, 1x))

(LBUF(i,j), CHKAT(i,j), i = 1, INF)

(INF+2)/6 lines are required. LBUF(i,j)=-1 or CHKAT(1,j) has blank are each terminator of donors and accepters, respectively.

columns

1- 5  	LBUF(1,j)  	: IAT1, donor or acceptor atom number -or-

6- 9  	CHKAT(1,j)	: KAT1, its atom name

11-15 	LBUF(2,j)  	: IAT2, donor or acceptor atom number -or-

16-19 	CHKAT(2,j)	: KAT2, its atom name

...

41-45 	LBUF(5,j)  	: IAT5, donor or acceptor atom number -or-

46-49 	CHKAT(5,j)	: KAT5, its atom name

51-55 	LBUF(6,j)  	: IAT6, donor or acceptor atom number -or-

56-59 	CHKAT(6,j)	: KAT6, its atom name

All atoms which are possible donors are listed, then either IATn = -1 or a blank KATn, followed by all possible acceptor atoms, and ended by IATn = -1 or a blank KATn.

Note that there is no real distinction between donors and acceptors except when the interaction is angle dependent.

If there are different types of attractive interaction, repeat 2.39 and 2.40 as needed. The program numbers the types sequentially.

2.41 Terminator of 2.40. Format (I10)

columns

1-10 	ITERM    	: 999999999

2.42 Contact limit table. Format (6F10.0)

((VRAD(i, j), i = 1, j), j = 1, NTYPE)

NTYPE lines are required.

Columns, first line

1-10 	VRAD(1,1) 	: DLIM11, limit below which a distance between atom types i and j is considered a contact

Columns, second line

1-10 	VRAD(1,2) 	: DLIM12, limit below which a distance between atom types i and j is considered a contact

11-60 	VRAD(2,2) 	: DLIM22

...

Columns, NTYPE-th line

1-10 	VRAD(1,NTYPE) 	: DLIM1,NTYPE, limit below which a distance between atom types i and j is considered a contact

11-20 	VRAD(2,NTYPE) 	: DLIM2,NTYPE

...

nn-nn	VRAD(NTYPE,NTYPE)	: DLIMNTYPE,NTYPE

There will be one distance on the first line, two on the next and so on. After the sixth, two or more lines are required per type. The atom type is adopted from the stereochemistry lines.

2.43 Terminator of 2.42. Format (I10)

columns

1-10 	ITERM    	: 999999999

2.44 Overall contact weights. Format (2F10.0)

WCON, X

columns

1-10 	WCON   	: CNW, non-zero multiplier to all individual contact weights (including the attractive interactions in 2.39)

11-20 	X         	: ALW, = 0, if individual weights for each contact type are to be given in 2.45 Individual contact weights

0, so that all contact types have the same weight ALW and go to 2.46 Cycle control line

2.45 Individual contact weights. Format (6F10.0)

(WRAD(i, j), i = 1, j), j = 1, NTYPE)

NTYPE lines are required.

Columns, first line

1-10 	WRAD(1,1) 	: CWT11, limit below which a distance between atom types i and j is considered a contact

Columns, second line

1-10 	WRAD(1,2) 	: CWT12, limit below which a distance between atom types i and j is considered a contact

11-60 	WRAD(2,2) 	: CWT22

...

Columns, NTYPE-th line

1-10 	WRAD(1,NTYPE) 	: CWT1,NTYPE, limit below which a distance between atom types i and j is considered a contact

11-20 	WRAD(2,NTYPE) 	: CWT2,NTYPE

...

nn-nn	WRAD(NTYPE,NTYPE)		: CWTNTYPE,NTYPE

This table is in exactly the same format as the DLIM table in 2.42 Contact limit table.

2.46 Cycle control line. Format (72A1)

CHKARD

Please refer Appendix A (Summary) and B (Examples) of cycle control data.

Usually all input data are prepared in the same file (logical unit 15). If the first character of CHKAP (2.1 Title) is ‘*’, this 2.46 Cycle control line data to 2.51 Terminator are read from logical unit 12.

columns

1-72 	CHKARD  	: option codes

This line contains one or more option codes separated by commas and terminated by a period. If necessary, it may be continued on to a following line by interrupting the information after a comma and continuing on the next line.

Option codes consist of either two letters, or two letters followed by an equal sign followed by an unsigned integer, for example, CY or CY = 3.

Note that CY is equivalent to CY = 1, and the absence of CY to CY = 0.

The following are the option codes. Operations of which there must be precisely one.

CL = n 	Perform a run to convergence with a limit of n cycles.

CY = n 	Perform a run of n cycles.

CO     	Proceed as far as coordinate calculation, constraint and restraint evaluation,  model fitting and elastication calculation only.

SF = n 	Proceed as far as structure factor calculation only. If Cruickshank A is non-zero and B is zero, a Cruickshank statistical analysis will be done.

If n = 3, the calculated Cruickshank weights will be used in subsequent cycles.

If n = 2 or if A calculates negative, the B and C values for the given A will be used.

If n = 1, none will be used.

CX     	Proceed as far as contact calculation only.

ST = n 	Set or step parameters. (see 2.48 Set and setup data ST)

ZZ     	Stop (and ignore any succeeding lines)

PU     	Copy the following lines to a file (see 4.1 Data Notes c)

a) Parameter Controls

These set the parameters to be varied and the constraints and/or restraints to be applied. This information is in effect, as a whole, unless and until changed by a subsequent cycle control line. This also applies to the MF data (see below d).

Parameters, which are elasticated, need not also be specified as refined.

VH = n 	Refine n thetas (as subsequently detailed)

VT = n 	Refine n Taus

VO = n 	Refine n other parameters

EH = n 	Elasticate n thetas

ET = n 	Elasticate n Taus

EO = n 	Elasticate n others

AC = n 	Apply n constraints and miscellaneous elastic relations

VN     	Vary nothing. this may be used to set all of the above back to zero.

b) Print Controls (default is none, beware)

CR = n 	Print coordinates where

n	= 1 of base molecule

= 2 of generated molecules

= 3 of both base and generated molecules

= 4 of generated molecules and their double helix mates (i.e. Those related by a dyad in the base molecule)

= 5 as 4, plus the base molecule

= 6 of the generated molecules and all their helical repeats in one unit cell

= 7 as 6, plus the base molecule

= 8 of the union of 4 and 6

= 9 of the lot, i.e. 8 plus the base molecule, See also miscellaneous controls CE

CN   	Print constraints and restraints

EL   	Print details of parameter elastication

BO   	Print details of bonding detected in the contact calculation. This is effective only in the first contact calculation per run, and is a useful data debugging aid.

CD = n	Print details of the contact calculation where

n	= 1 all the useful stuff

= 2 more information, generally unnecessary note that CD = 2 implies BO (above)

CS   	Print statistics of the contact calculation

XR   	Print x-ray data and structure factors

XS   	Print statistics concerning x-ray calculation.

MA   	Print normal matrix summary and right hand side vector

SH = n	Print eigenvalues, data analysis and shifts where

n	= 1 usually sufficient information

= 2 include Lagrange multipliers and error analysis of shifts. This is only meaningful for a converged run.

OP   	Suppress printing of current values of parameters at start of each cycle.

DG = n	Print various diagnostics throughout execution where

n	= 1 print in output file

= 2 print in output file and error file.

AL   	This is the same as CN, EL, CS, XR, XS, MA, and also CR = 3, CD, SH unless any of these is also specified.

c) Update File (Punch ) Controls (default is none)

SC = n	Specify a file to update the data set to the new parameter values where

n	= 1 the stereochemistry lines have atom names for precursors

= 2 the stereochemistry lines have atom numbers for precursors

If SC accompanies CY = n or CL = n, it has effect only in the last cycle performed. If no SC is encountered in a run, lines for SC = 1 are automatically generated (even if the run fails due to a WinLALS error), if any shifts have been applied.

CC = n	Create the coordinates specified in the CR print option where

n	= 1 in ORTEP Cartesian format

= 2 in ORTEP cylindrical polar format

= 3 in Format (3F10.4,A4,1x,F10.4,I5), giving x,y,z,name,occupancy,type

= 4 in ORTEP fractional coordinates format, for the generated molecules only.

OZ   	Omit zero occupancy atoms when making this file.

PO   	Make this file but omit the printing specified by the CR option.

FC   	Fourier data available only with Trig version. Fourier coefficients are prepared during the structure factor calculation and are intended for input to a Fourier program. The first line contains (6F10.4) a, b, c, alpha, beta, gamma (angles in radians). Then follow reflection lines, one per reflection, containing IH, IK, IL, IM, Z, PHSE, X NOV and Y in Format (4I5, 5F10.4) where

IH, IK, IL are the Miller indices,

IM	= multiplicity,

Z  	= MOD(Fcalc) * SQRT(IM) where Fcalc is prior to applying temperature factor,

PHSE	= phase of Fcalc in degrees,

X    	= Fobs for this SPOT,

NOV 	= number of reflections in this spot

Y    	= X/SQRT(NOV).

Following all the reflection lines for a spot, a spot line containing IfFLAG, IZ,	NH in Format (3I5) where

IFLAG	= 1 for an accepted, observed spot,

= 2 for a rejected observed spot

= 3 for an accepted unobserved spot

= 4 for a rejected unobserved spot and

IZ = -999 (provided for identification purposes)

NH  	No headings. Normally each set of all the above lines is preceded by two lines of title, which are suppressed by this option (see 4.1. Data Notes c).

d) Miscellaneous Controls

FI    	Change the filtering, scaling, damping and/or convergence conditions.

OS   	Switch on optimized shifts (see 4.1 Data Notes b). This remains in effect until

NS   	Resume normal shifts.

FO   	Force continuation even if divergent. This causes the check for WinLALS	error 18 to be bypassed, and remains in effect until it encounters

NF  	No forcing and resume the divergence checks.

NX  	No x-ray calculation. This is used if x-ray data are supplied, but are not to be used. This remains in effect until

YX  	Yes x-ray. If a previous NX has been given, this switches the use of x-ray data back on.

NC  	No contact calculation. See NX (above) for details

YC  	Yes contacts. See YX (above) for details.

LI = n	This causes a contact search to be done at least every n cycles (see 4.1 Data Notes e)

TA = n	Tabulate function n (valid only with ST)

TB = n	Tabulate function n (valid only with ST)

SP    	Suppress print. (valid only with ST and TA and/or TB)

MF = n	Use model-fit data where

n 	= 1  read and apply the data

= 2  apply previously read data

MF is counted with parameter controls in the sense that it need only be given when parameter controls are present. Equally, if given on its own, it nullifies any previous parameter controls.

ND = n	Use n as the number of data for the error calculation for shifts and/or coordinates. See "uncertainties" below.

CE   	Calculate uncertainties in coordinates as given by the CR parameter. This is valid only in a CO, CX or SF cycle, and the preceding cycle must have included SH = 2. None of the following may appear with CE, VH, EH, VT, ET, VO, EO, AC, VN.

2.47 Cycle data lines.

If any of VH=nVH, VT=nVT, VO=nVO, EH=nEH, ET=nET, EO=nEO, AC=nAC, FI, ST=nST or MF=nMF is given, the appropriate data follow on subsequent lines in the following order, regardless of the order on the control line. Each set only appears if the appropriate option was specified on 2.46 Cycle control line.

2.47A VH: Varied thetas. Format (12I5)

(IVH(i), i = 1, nVH) where IVH(i) is the number of theta to be varied.

columns

1- 5  	IVH(1) 	: IVH(i) is the number of theta to be varied.

6-10  	IVH(2) 	: i=1,nVH

11-15 	IVH(3) 	:

16-20 	IVH(4) 	:

……       	:

IVH(i) 	:

2.47B EH: Elasticated thetas.

(IEH(i), i = 1,nEH) in Format (12I5)

(BEH(i), i = 1, nEH) in Format (6F10.0)

(WEH(i), i = 1, nEH) in Format (6F10.0)

columns, first line, Format(12I5)

1- 5  	IEH(1) 	: IEH(i) is the number of theta to be elasticated.

6-10  	IEH(2) 	: i=1,nEH

11-15 	IEH(3) 	:

16-20 	IEH(4) 	:

……       	:

IEH(i) 	:

columns, second line, Format(6F10.0)

1- 5  	BEH(1) 	: BEH(i) is the value to which theta is bound,

6-10  	BEH(2) 	: in the same order as the IEH(i).

11-15 	BEH(3) 	: i=1,nEH

16-20 	BEH(4) 	:

……       	:

BEH(i) 	:

columns, third line, Format(6F10.0)

1- 5  	WEH(1) 	: WEH(i) is the weight with which theta is to be bound,

6-10 	WEH(2) 	: in the same order as the IEH(i).

11-15 	WEH(3) 	: i=1,nEH

16-20 	WEH(4) 	: If BEH(i) is greater than 360, it causes binding in every cycle

……       	: to the current value. If BEH(i) is less than -360, binding is

WEH(i) 	: always to the value at the time this control line is read, and

it is not updated in subsequent cycles.

Note that the data for IEH(i), BEH(i), and WEH(i) each start on a new line. Thetas need not be specified as refined as well as elastically bound; the latter will suffice.

2.47C VT: Varied Taus. These have the same format as VH.

columns

1- 5  	IVT(1) 	: IVT(i) is the number of Taus to be varied.

6-10  	IVT(2) 	: i=1,nVT

11-15 	IVT(3) 	:

16-20 	IVT(4) 	:

……       	:

IVT(i) 	:

2.47D ET: Elasticated Taus. These have the same format as EH.

(IET(i), i = 1, nET) in Format (12I5)

(BET(i), i = 1, nET) in Format (6F10.0)

(WET(i), i = 1, nET) in Format (6F10.0)

columns, first line, Format(12I5)

1- 5  	IET(1) 	: IET(i) is the number of Taus to be elasticated.

6-10  	IET(2) 	: i=1,nET

11-15 	IET(3) 	:

16-20 	IET(4) 	:

……       	:

IET(i) 	:

columns, second line, Format(6F10.0)

1-10 	BET(1) 	: BET(i) is the value to which Taus is bound,

11-20  	BET(2) 	: in the same order as the IET(i).

21-30 	BET(3) 	: i=1,nET

31-40 	BET(4) 	:

……       	:

BET(i) 	:

columns, third line, Format(6F10.0)

1-10  	WET(1) 	: WET(i) is the weight with which Tau is to be bound,

11-20 	WET(2) 	: in the same order as the IET(i).

21-30 	WET(3) 	: i=1,nEH

31-40 	WET(4) 	: If BET(i) is greater than 360, it causes binding in every cycle

……       	: to the current value. If BET(i) is less than -360, binding

WET(i) 	: is always to the value at the time this control line is read, and

it is not updated in subsequent cycles.

2.47E VO: Varied "other parameters". Format (12I5)

(IVO(i), i = 1, nVO) where IVO(i) is the number of "other parameters" to be varied.

VO and EO: Varied and elasticated "other parameters". These have the same format as VH and EH. The numbering of the "other parameters" is given earlier under 2.9 "position definitions". Weights for other parameters are in the reciprocal square of the units of the parameter.

columns

1- 5  	IVO(1) 	: IVO(i) is the number of "other parameters" to be varied.

6-10  	IVO(2) 	: i=1,nVO

11-15 	IVO(3) 	:

16-20 	IVO(4) 	:

……       	:

IVO(i) 	:

2.47F EO: Elasticated "other parameters".

(IEO(i), i = 1,nEO) in Format (12I5)

(BEO(i), i = 1, nEO) in Format (6F10.0)

(WEO(i), i = 1, nEO) in Format (6F10.0)

columns, first line, Format(12I5)

1- 5  	IEO(1) 	: IEO(i) is the number of "other parameters" to be elasticated.

6-10  	IEO(2) 	: i=1,nEO

11-15 	IEO(3) 	:

16-20 	IEO(4) 	:

……       	:

IEO(i) 	:

columns, second line, Format(6F10.0)

1-10  	BEO(1) 	: BEO(i) is the value to which "other parameter" is bound,

11-20  	BEO(2) 	: in the same order as the IEO(i).

21-30 	BEO(3) 	: i=1,nEO

31-40 	BEO(4) 	:

……       	:

BEO(i) 	:

columns, third line, Format(6F10.0)

1-10  	WEO(1) 	: WEO(i) is the weight with which "other parameter" is to be

11-20 	WEO(2) 	: bound, in the same order as the IEO(i).

21-35 	WEO(3) 	: i=1,nEO

16-20 	WEO(4) 	:

……       	:

WEO(i) 	:

2.47G AC: Applied constraints and restraints. Format (12I5)

(IAC(i), i = 1, NAC)

columns

1- 5  	IAC(1) 	: IAC(i) is the number of constraint or restraint to be applied.

6-10  	IAC(2) 	: Constraints and restraints are numbered by the program

11-15 	IAC(3) 	: sequentially in the order in which they are defined.

16-20 	IAC(4) 	:

……       	: i=1,nAC

IAC(i) 	:

Note that some definitions generate two or more constraints. e.g. COI generates three with 3 consecutive numbers.

2.47H FI: Filtering etc. information. Format (5F10.0)

SANG, SDIS, DAM, FIL, CNV

columns

1-10 	SANG 	: maximum shift in degrees to be allowed in any angle (20.0 by default)

11-20 	SDIS  	: maximum shift in Angstroms to be allowed in any distance (i.e. S, U, V, W). It is converted to a unit cell fraction by the program for U, V, W (0.5 by default)

21-30 	DAM    	: damping factor (1.0 by default)

31-40 	FIL    	: < 1.0, filter size for eigenvalues (i.e. those of lesser magnitude  will be filtered)

1.0, it is the number of eigenvalues to be filtered (0.001 by                         default)

41-50 	CNV   	: a run is deemed to have converged when the mean shift is less than CNV. For this purpose angles are in radians and distances in angstroms (0.001 by default).

Additional information on filtering is in 4.1 Data Notes g, and on shift scaling in 4.1 Data Notes h.

2.48 Set and step data (ST). Format (2I5, 4F10.0)

(IWH(i), IND(i), SET(i), CHA(i), STE(i), UNT(i), I = 1, nST)

This allows any parameter(s) to be explicitly reset or changed by a fixed amount. It also allows a series of non-refinement cycles to be performed in which specified parameter(s) are stepped systematically. In the latter case, one or two functions of each cycle may be collected and tabulated. The data lines are as follows.

nST lines are required.

columns

1- 5  	IWH(i)   	: = 1 This line pertains to a theta

= 2 to a Tau

= 3 to an other parameter

6-10 	IND(i)    	: number of theta, Tau or other parameter

11-20 	SET(i)    	: new value for the parameter or

= 400.0 to keep the parameter as is

21-30 	CHA(i)   	: value (negative, zero or positive) to be added to the parameter immediately (normally used only with SET=400.0)

31-40 	STE(i)    	: step size for the stepping process or

= 0.0 to stop stepping this parameter

41-50 	UNT(i)    	: continue until the next step would go beyond UNT by half a step or more

There may be any number of parameters set, but a maximum of four may be stepped  together. Normally if more than one parameter is stepped, the stepping is nested, i.e. the second parameter is stepped over its entire range for each value of the first, and so on. But if UNT is given as 400.0, the stepping of that parameter is linked to that of the previous one, the number of joint steps being determined by its UNT. The step sizes in this case need not be identical.

The part of the WinLALS cycle to be performed during stepping is specified by the control line option.

ST = 1 for the constraint and restraint, model fitting and elastication or

= 2 for the contact calculation is done or

= 4 for the structure factor calculation.

Combinations are possible by summing the n values, i.e. N = 7 means everything.

Tabulation allows the values of one or two functions to be saved and printed in tabular format at the end of the entire stepping sequence. This is specified by the control line options TA = n and/or TB = n, where n may take the following values.

1  sum of applied constraints

2  worst constraint

3  sigma (w * d**2) for elasticated parameters

4  sigma (w * d**2) for contacts

5  weighted R-factor (including unobserved reflections)

6  conventional R-factor (including unobserved reflections)

7  overall sigma (w * d**2)

8  shift scaling factor

10  sum of all constraints

11  mean (w * d**2) for elasticated parameters

12  shortest contact

13  worst contact (greatest difference)

14  mean (w * d**2) for contacts

15  sigma (w * d**2) for x-ray data

16  mean (w * d**2) for x-ray data

17  sigma (w * d**2) for restraints and model fitting

18  mean (w * d**2) for restraints and model fitting

101  if any contact (see below)

102  number of contacts in list

103  number of contacts applied

104  total number of data

Function 101 has the value 1 if any contact is found and 0 otherwise. This is much faster than a complete contact search. Note that it is the programmer’s responsibility to choose the correct value for ST for a given TA or TB value (e.g. ST = 4 for TA = 6). Although no parameters need be specified as varied - using the usual VH, VT, VO parameters - for a stepping run, it may sometimes be advantageous to do so if a contact tabulation is being done. This is because  invariants cannot be identified unless there are entries in the varied parameter list. During a tabulation run only, all the intermediate print may be suppressed by using SP. Beware, though, that if you don't give enough time for the run you will get nothing out.

2.49 Terminator, if 2.48 is present. Format (I10)

columns

1-10 	ITERM   	: 999999999

2.50 Model fitting data (MF).

IMOL,CHKMOL in Format (I5,A3)

or

(IAT(i),CHKAT(i),X(i),Y(i),Z(i),W(i),i=1,nMF) in Format (I5,A4,1x,4F10.0)

columns, Format (I5,A3)

1- 5 	IMOL     	: molecule number on which atoms to be fitted are located

6- 8 	CHKMOL 	: (the three characters)

or

columns, Format (I5,A4,1x,4F10.0)

1- 5 	IAT(i)    	: atom number -or-

6- 9 	CHKAT(i) 	: its atom name

11-20 	X(i)      	: x,

21-30 	Y(i)      	: y, and

31-40 	Z(i)      	: z are the coordinates in the fixed model.

41-50 	W(i)     	: weight to be applied to this term

If model fitting is done for two or more molecules, the input is repeated from the IMOL line. The given atom names (or numbers) must correspond to the original stereochemistry input, but they may be in any order and not all the atoms for the specified molecule need be present. Model fitting is exactly equivalent to a series of distance restraints to points fixed in space, and is commonly used to make a model with standard bond lengths and angles as close to another non-standard model as possible.

2.51 Terminator, if 2.50 is present. Format (I10)

columns

1-10 	ITERM  	: 999999999

3 Output Description

The output from WinLALS is intended to be self-explanatory. The following notes expand on some of the more obscure parts. Warning messages start with +++ and indicate possible problems with  which the program can deal, but which may be symptomatic of data or other errors.

Error messages (listed separately) begin with *** and indicate faults with which the program cannot cope (refer Appendix C). In most cases execution ceases immediately. During execution, however, the program continues to the end of input routines, and then stops. No attempt is made to fix errors (including array bound checks), which occur during this phase, so that one error may well lead to other spurious errors. The numbered errors are listed in the following section. Unnumbered errors and warnings are listed here in the appropriate context. There follows a description of the output from a mythical run, which uses every facility in WinLALS, with all the print options on.

Subroutine names and their calling sequences are summarized in Appendix D and E, respectively.

3.1 Input Data Listing

The main input data (i.e. down to but not including the cycle control data) is listed in an interpreted form, and a certain amount of checking done. In particular, atom names are checked for validity.

Errors and Warnings

a) Illegal NHEL or JHAND.

In the present manual these are referred to as NHEL and JHEL.

b) Position nun has illegal sense viz. nun.

Down molecules are illegal for a NHEL = 2 double helix.

c) Illegal constraint code - aura.

The first three characters are unrecognized as a constraint type.

e) Non-integral number of occupied atoms viz. nun (warning)

f) Illegal root atom nun - structure nun.

See 4.1 Data Notes e.

g) Illegal precursor for atom nun viz. nun.

The precursor must be an earlier atom in the same structure, and the first 4 atoms of each structure must each procure the next, and no other atom may have any of the first 2 as precursor.

h) Premature end of atom data for structure nnn.

No atoms have been defined for this structure.

i) Illegal number of components in an aaa constraint/restraint viz. nnn.

See the description of COO and PAR constraints.

j) Illegal code - aa.

WinLALS expects TH or TA or OT here.

k) Bad order numbers, structure nnn.

The epsilon order numbers are not a permutation of 1, 2, 3.

l) No target specifications.

m) No source molecules.

n) Bond between atoms nnn and nnn not of type stated.

A type II bond input is actually type I, or a type III is actually type I or II.

o) Misplaced target to-line.

The first line cannot be an inclusive cell-edge translation line.

p) Array specifier - aaaa - is too small.

Like it says. This may occur at various points throughout the run.

q) Bad atom nnn(aaaa) - xxxxxxxxx.

Either the supplied atom name is unknown, or the supplied number is outside the appropriate range. This error may occur during model fitting input too. The xxxxxxxxx says where it occurred.

3.2 Output From subroutine CYCLER

CYCLER is the routine called at the start of every cycle. It processes a cycle control line (if one is not already being processed) and reads and lists any cycle control data. Also, the values of all parameters at the start of each cycle are listed.

In a stepping run with print off (SP) and tabulation (TA/TB), the only non-error output from CYCLER is a message to the effect that the appropriate parameter has been updated.

Errors and Warnings

a) Illegal or duplicate applied constraint - nnn.

The number is out of range or listed twice.

b) Syntax error on control line.

This is indicated by underlining the offending characters.

c) Aaaa(nnn) is specified twice - xxxx.

Aaaa may be Thta, Tau or Othr, and xxxx may be refined or elasticated.

d) Aaaa(nnn) is illegal - xxxx.

This includes the possibility of refining undefined parameters such as theta(1).

3.3 Output From subroutine COORDS

In a CE cycle (coordinate errors), the line labeled SE after each line of coordinates is the standard error in each coordinate. The value given by u at the end of the line is the rms positional uncertainty in the units of the Cartesian coordinates.

Errors and warnings

a) Theta and Tau nnn flipped. (warning)

Through shifting or as input, Tau is not in the range 0 to 180 degrees. In order to preserve the description, the theta had to be altered by 180 degrees. For example, theta = 40, Tau = 200 would be flipped to theta = -140, Tau = 160.

b) Dangerous value for Tau nnn made safe. (warning)

Any Tau closer than .005 degrees to 0 or 180 is shifted minimally to preserve precision.

c) The value given as zero in the next line calculated as a negative variance of nnnnnnn (warning).

This occurs in a CE cycle occasionally. See SOLVE, below.

3.4 Output From subroutine CONELA

CONELA performs the constraint, restraint, elastication and model- building functions.

Errors and warnings

a) Relation nnn has unvaried components. (warning)

Not all the elements of a PAR constraint are varied.

b) Too small, set at .001. (warning)

A distance is constrained to less than .001. This will probably lead to instability, if intended, and a COI constraint should be substituted.

c) Radius too small for r or phi constraint - fudged. (warning)

An atom very close to the helix axis has its r or phi constrained in a COO constraint/restraint. This may lead to instability.

d) Constraint nnn has no non-zero derivative. (warning)

Therefore it has no effect.

e) Restraint nnn has no non-zero derivative. (warning)

Neither does it.

3.5 Output From subroutine CNTCTS

CNTCTS performs the contact search. It is in 3 parts. The first, CONSET, is performed only once per run and it sets up the bonding arrays and various other things. The second, CONTAB, is the actual search (which may be omitted in some cycles by the use of the LI option), and the last, CONUSE, is the application of the contact list.

Some of the following output appears only if BO or CD = 2 is specified. CONSET lists source and target molecules, numbering them S1 ... and  T1 ..., and these numbers are referred to subsequently in CNTCTS rather than the original molecule numbers.

The bonding lists should list every existing bond in either direction (i.e. C1---C2 as well as C2---C1). The attractive interaction list includes the type (in the order read in), whether the atom is an acceptor (A) or donor (D), and a plus (+) indicates angle-dependence.

CONTAB does nested cycles over all source and target residues, and prints one line per pair (if CD = 1, pairs with no contacts are omitted). The list includes the highest (i.e. greatest z) and lowest atoms in each residue, whether the swal (sorted and weeded atom list) was scanned up or down, the separation between the helix axes, the reason that no search was done, viz. that the axes were too far apart for any possible contact (axes), that the target was too high above the source for any contact (high) or too low (low). Under relation is given, possibly, that the residues are in fact the same residue (same) or that they are diadically related (diad). In the latter case, only the unique pairs are searched and the multiplicity doubled for contacts not between corresponding atoms. The number of contacts tested will often be fewer than the product of the number of atoms in source and target because of the economies in searching allowed by the swal order. CONTAB finishes by listing some mildly interesting statistics.

CONUSE tests every contact in the list to see if it has any non-zero derivative with respect to a varied parameter. If not, it is marked as inactive and subsequently ignored. The fact is listed if CD = 2. Relieved contacts are similarly ignored. After any contact, which might be an angle-dependent interaction, the angle is listed (whether or not the test succeeded). Contacts with zero weights are treated similarly to invariant ones.

Errors and warnings

a) Contact list limit reached. (warning)

The contact list is full and searching stops. This may not be serious at the start of a run since many of the contacts may later be relieved.

b) Contact list empty. (warning)

There were no contacts.

c) Less than a third of the contact list applied - trying again. (warning)

An old contact list has been used and two-thirds of the non-invariant contacts have been relieved. Because the normal matrix will have been messed up, it may be necessary for it to repeat from constraints and restraints.

3.6 Output From subroutine SFACTR

The two structure factor routines give similar output, except that the Bessel routine can also calculate continuous transform. User can able to select Trigonometry or the Bessel routine in WinLALS.

The first time SFACTR is executed, it lists the type it thinks each atom is. Under core it gives the element, o* being oxygen half-charged and o- oxygen negatively charged. User defined types are given xx. Under h is the number of hydrogen atoms bound, h* meaning more than 4 (an unlikely occurrence). Under w is * if normal water-weighting is applied and + if the old-fashioned (i.e. wrong) scheme is.

In the following list of structure factors, the Fobs values have been multiplied by scale, and the Fcalc values have had attenuation applied. Under the flag column, r indicates a spot rejected from the least squares, u an unobserved spot.

The two sets of R-factor etc. Statistics are on the left, all except rejected unobserved spots, and on the right only observed spots.

They follow a count of the electrons in the unit cell, and the number implied by water-weighting, if any. The continuous transform calculation lists the transform for each rr (reciprocal cylindrical radius).

Errors and warnings

a) Fobs range has caused division factor to be nnnnn. (warning)

The Fobs octave range for Cruickshank statistics has been increased to get the whole range into ten divisions.

b) Cylindrical transform not possible in Trig. (warning)

Like it says, and the data is ignored.

3.7 Output From subroutine FILMAT

This routine illogically but practically contains the structure factor Cruickshank statistics routine (used only in a SF cycle), and also the normal matrix completion and summary procedure.

Errors and warnings

a) Matrix diagonal element nn is zero. (warning)

A curious phenomenon, likely to be followed by

b) Indeed, no datum has a non-zero weighted derivative with respect to this parameter. (warning)

That is, unless this parameter is determined by constraints, it is not defined by the conditions of the problem.

c) Singularity in Cruickshank calculation least squares matrix. (warning)

Possibly all the eggs are in one basket. Not serious.

d) Need zero Cruickshank B for statistical analysis. (warning)

SF = 2 or 3 requires that the Cruickshank B be zero.

e) Negative Cruickshank B or C generated. (warning)

This may lead to negative spot weights in future cycles. In any case, the physical significance bears pondering.

3.8 Output from subroutine SOLVE

SOLVE solves the normal equations, updates the parameters, and returns to CYCLER. The values designated goodliness and residual refer to the numerical precision of the matrix inversion. Goodliness should be less than 1 (it is usually much less), residual should be of the order of x, where x is the largest number for which your machine believes that 1.0 + x = 1.0. In the weight analysis you might consider that ideally, in the final model, the mean w * d**2 for each category should be 1.0. This assumes that your weights are equal to the reciprocals of the variances of the data (and not just proportional to them). But the applicability of this philosophy to the contact calculation is debatable.

Errors and warnings

a) Theta and Tau nn flipped. (warning)

See discussion under output from COORDS, above.

b) Standard deviations etc. may be meaningless for a not converged run. (warning)

Because, inter alia, the total w * d**2 will have changed.

c) Shift optimization unsuccessful. (warning)

The parabola was upside down. The more successful of the two points is selected.

d) Optimized shifts impossible without constraints applied. (warning)

You goofed.

e) Negative inverted matrix diagonal element - check filtering. (warning)

The inverted normal matrix contains a negative diagonal element corresponding to the following parameter. This is quite impossible in reality, and probably arises from filtering the wrong (probably too many) number of eigenvalues. In any case, the calculated standard deviation would be imaginary.

4 User's Helpful Description

4.1 Data Notes

a) In general it is much more efficient to assign all the occupancy for multiply defined atoms to one of them. However, care should be taken. If there exists a varied theta or Tau which moves (assuming the root atom to be fixed) a subset of such equivalent atoms, then at  least one of this subset must have non-zero weight.

b) A run with optimized shifts is performed as follows. The shifts are calculated as usual and the shift maxima are used to calculate the shift scaling factor. Two trials are then made, using 1 and 2 times these shifts, wherein a new cycle is performed as far as the constraint calculation (with all printing automatically off). The two values of the sum of the absolute values of the  applied Lagrangian constraints, together with the zero shift value (from the start of the current cycle) are fitted to a parabola, and the shift fraction corresponding to the minimum of this sum is calculated. This is then used as the shift scaling factor, and to it is applied the damping factor, which can only meaningfully be 1.0. The interpolated fraction is restricted to the range be applied only to poorly constrained systems, since if the constraints are already well satisfied, the predicted shifts are small. A good strategy is to optimize shifts on the second half of a run only.

c) Operation PU in conjunction with SC or CC and NH can be used to create an input data set for another program run in the same job. PU will copy lines (all 80 columns) to the updating file as follows. If the first line after the control line containing PU does not contain the characters FLAG in columns 1 to 4, lines are copied up to but not including the first line encountered which does have FLAG in columns 1 to 4. If the first line does contain FLAG in columns 1 to 4, it is stored (and  not copied) and compared with each successive line, which is copied only if it differs in at least one of the first 72 columns with this stored line. When a match is found, copying ceases. In summary, therefore, if no flag line is to be copied, PU will copy everything up to a flag line. If a flag line is to be copied, then the line set must have two identical flag lines before and after.

d) After each refinement cycle (CL or CY), the current values of all the parameters are written in binary format on unit 3. This is indicated by the message =Dump record made (n)= where n th the cycle number. If the unit 3 file is saved, subsequent run may restart the refinement at any point by executing the cycle control line RE=n, where n is a cycle number for which a dump record was made. This parameter, RE merely reads in the stored values of the parameters, and does not of itself initiate any refinement cycles. Note that RE always rewinds the file, so that an RE line may be used to go back to some earlier stage in the current run. Note however, that all previous data of the file will be overwritten if this is done. Note too that the dump is made at the end of the cycle, so that it is nit possible to go back to the start of the run, before the first cycle. This can be overcome by the use of the DU operation. A cycle which executes DU = 1 will write a dump record without refining the dump file. DU=2 will rewind and then write A dump record, and DU=3 will simply rewind it.

e) A new contact list will be created (1) in the first cycle in which a contact calculation is requested, (2) thereafter, after the list has been depleted to the extent of two-thirds by the contacts having been relieved, (3) if the number of cycles specified by the LI parameter has been completed since the table was last created, or (4) if a cycle control line containing LI was actually read in this cycle. The default for the LI parameter is 1, i.e. the list is recreated every cycle. A larger value for LI can often save a lot of time.

f) Normally, the root atom may not be one of those for which no theta is defined (i.e. the first three of any structure). For structures containing fewer than 4 atoms, the root atom must be the last atom. If there is only 1 atom, no epsilon has any effect and the only root parameter that may be varied is S. If there are 2 atoms, epsilon-x has  no effect and may not be varied.

g) Eigenvalue filtering may be required because of redundancies, explicit or implicit, in parameters or constraints. For example, if a ring is joined up or a helix made continuous using 9 coincidence constraints, only 6 (at a maximum) of the constraints are linearly independent. If all 9 are applied, as is desirable to avoid other discrete solutions, 3 eigenvalues will have to be  filtered. Deciding how many eigenvalues to filter is sometimes difficult. By far the best way is to deduce from physical considerations how many redundancies there are. However, this is not always possible, and another way, good in well-behaved systems, is to look for the biggest gap in the spectrum of eigenvalues and assume that those below the gap are spurious (the eigenvalues are listed in decreasing order). In the worst case, it may be necessary to do trial runs filtering different numbers. Hopefully, using the correct number will lead to a convergent refinement without any negative inverted matrix diagonal elements (shown by warnings), and using a wrong number will give the opposite result. Inspection of the correlation coefficients given by SH = 2 may show why redundancies or near-redundancies arise, and may allow a difficult problem to be altered to avoid such troubles.

h) The calculated shifts are modified as follows.

1) If any angle shift exceeds Sang, or any distance shift exceeds Sdis, the entire set of shifts is scaled by a constant so that no shift exceeds these limits.

2) The optimized shifts procedure is then called to modify this scaling factor, if OS has been specified.

3) The factor is unconditionally multiplied by DAM (2.47H). This is on occasion useful for damping an oscillatory refinement (DAM less than 1.0), or for speeding an asymptotic one where the same shifts are predicted in each cycle (DAM greater than 1.0).

4.2 Tree Geometry

The principle of tree geometry is that any atom is defined by specifying an already-defined atom to which it is (notionally perhaps) attached, denoted its precursor, and giving its position in terms of its relation to its precursor. More precisely, if p(i) is the precursor of i and p2(i) the precursor of p(i) etc., atom i is fixed by giving distance p(i)-i, angle p2(i)-p(i)-i, and dihedral angle p3(i)-p2(i)- p(i)-i. For the first three atoms, this must be slightly modified, in that their spatial position is (initially) arbitrary. Consequently it follows that atoms 1 to 4 must be sequentially dependent (2 on 1, 3 on 2 and 4 on 3) and no other atoms may depend on atoms 1 or 2. Because of the method used, no bond length may be zero, nor any bond angle initially zero or 180 degrees. If a bond angle is subsequently shifted through 180, the description is preserved by changing it to 180 ? angle and the corresponding theta to 180 + theta, and similarly if it is shifted  through zero. If it is shifted to a value within .005 degrees of zero or 180, it is moved to .005 or 179.995 degrees respectively. In any of these cases a message is printed.

After all the atoms have been incorporated in a structure, it is

a) aligned with the root atom at the origin, its precursor on the positive x-axis, and its precursor in the (x,-y) half-plane,

b) rotated about the first-specified axis by the first specified Eulerian angle (positive rotation is anti-clockwise viewed from positive infinity),

c) similarly rotated about the second and third axes and

d) translated in the x-direction by S.

Note that relative to the molecule, the Eulerian rotations are thus not orthogonal. For degenerate structures the above is modified in the obvious way.

4.3 Generation of Molecular Coordinates

a) The base residue of each structure is generated using the stereochemistry data. Its root atom will be at (S, 0, 0) in the molecular coordinate system, and its helix axis will be along the z-axis.

b) Each molecule descriptor (structure number, position number, equivalence number) is taken in turn and operates on the base residue to produce a generated molecule. This is done as follows. G = C ( E ( D. O. I. B + U ) + T ) where G is the vector (x, y, z) defining the position of an atom in the generated molecule in the crystal Cartesian orthogonal system. B is the vector (x, y, z) defining the position of that atom in the base residue. C is a matrix that converts fractional crystal cell-edge-directed coordinates to crystal Cartesian orthogonal coordinates. E is the equivalence matrix from the input data. D is the inverse of C. O is a z-rotation matrix which rotates by Mu (from the position definition) anti-clockwise viewed from positive infinity. I is an x-rotation matrix of zero for an up or 180 for a down molecule. U is the fractional crystal cell-edge-directed vector (u, v, w) from the input position definition. T is the fractional crystal cell-edge-directed vector from the equivalence definition from the input.

c) The relationship between the crystal Cartesian orthogonal and crystal fractional cell-edge directed coordinate systems is that z is along the c-edge of the cell, y is in the plane formed by the b and c edges, and x completes the right-handed set.

Note that, in general, the term "residue" is used to mean a part of one strand of the helix containing one of each of the defined atoms, whereas the term "molecule" refers to a residue plus any number of helical repeats in either direction.

4.4 Contact Calculation

The contact search looks for short contacts between atoms in specified residues and treats them similarly to restraints. Note, however, that these are applied only if the distance is shorter than that specified, so that, unlike restraints, the effect is purely repulsive.

The calculation is in three parts. The first sets up various tables that are required. In particular, it locates and sorts the bonds that exist within and between residues. It also prepares the sorted and weeded atom list, which contains the atoms of one residue of each structure arranged in order of decreasing z coordinate.

The second procedure is the contact search proper. Firstly, if there has been a change in any parameter that might affect the order of the atoms while sorting and weeding, the list is re-sorted. It then considers all contacts between atoms in "source residues" and "target residues". The source unit, which comprises all the source residues, is defined by the user. All the source residues are within one unit cell (strictly, a contiguous volume of equal capacity to the  unit cell). The target unit may contain any of these residues, and more form other cell repeats. Note that an intra-residue search is therefore performed merely by including the same residue in both source and target units. If a possible contact is found, tests are performed to establish whether the atoms are bonded to each other or to a common third atom, in which cases the contact is discarded. Otherwise, it is entered into the "contact list". This list may contain as many entries as specified on the contact option line, after which a non-fatal message will be given and the rest of the search skipped.

Several economies are made in the contact search. Firstly, each pair of residues (one source, one target) is examined to see whether they are close enough to make contacts. This involves checking the distance between their helix axes, and the z separation between the highest atom in the lower of the two and the lowest in the higher ("up" being towards higher z). If these tests succeed, a "reject reason" of "axes", "high" or "low" is given, and the search continues with the next pair of residues. If the pair is not thus rejected, use is made of the sorted atom list to establish the most efficient order in which to search, in particular, whether each residue should best be scanned up or down (the list).

The third procedure applies the contact list. Because the search is relatively time-consuming, it is not necessarily performed in every cycle of refinement. Instead, the distances between the pairs of atoms in the contact list are recalculated, and applied only if they are still short contacts. During the first scan of the list, a check is made to establish whether any of the contacts are invariant with respect to the parameters being varied. Not considered as data, such contacts are marked as invariant and ignored thereafter.

The contact search procedure tests each residue pair to find whether the relationship exists that atom A in the source and atom B in the target are separated (systematically) the same distance as atom B in the source and atom A in the target. If this is true, only the unique distances are checked, and the effect of each contact found is doubled. This is indicated by "diad" in the "relationship" column of the search printout. In these cases, however, if atom A and B are the same atom on different residues, the doubling is not applied, and if they are the same atom on the same residue, the contact is ignored.

A little thought (some say a lot) is required to establish the best choice of source and target residues. Often it will be possible to specify as source something much less than a unit cell full of atoms. In practice, strive to obtain a situation where no contact in the list is merely a symmetry related duplicate of any other. The multiplicity parameter of the target unit specification allows the specification of interactions that occur, because of symmetry, different number of times.

4.5 Uncertainties

WinLALS provides some estimates of uncertainties (errors) in final parameters and coordinates. These should be treated with some caution and in most cases are probably best regarded more as a guide to sources of ill-definition than true estimates of parameter variances.

Least-squares uncertainties are a function of the number of data used. Clearly, the more data, the more certain the results, provided that the data are consistent and have only random errors. Generally, the number of data used is easily reckoned, but for the contact calculation a special problem arises. Do we consider relieved non-contacts as data, or not. LALS compromises by using either the number of actual contacts, or the number of atoms in the source unit, whichever is larger. However, if desired, use the ND parameter to alter this. Using SH = 2, the uncertainties in final parameter values are calculated. Note that these are only meaningful for a converged run, and a warning to that effect will be issued if SH = 2 is used for a cycle where the shifts are significant.

As well as giving estimates in the standard deviations of parameters, SH = 2 gives the correlation coefficient matrix (normalized covariance matrix). If two parameters have a correlation coefficient whose absolute value is close to 1.0, the parameters are highly correlated, i.e. they affect the calculated properties of the model in a very similar way, and hence any error in one may be offset by a corresponding change in the other. They are helpful when convergence is difficult. A possible solution is to vary only one of two highly correlated parameters in any given cycle.

After a SH = 2 cycle, the consequent uncertainties in coordinates may be calculated using the CE parameter. The numbers printed under the coordinates are their standard errors, and an estimate  is also given the root mean square uncertainty in the atomic position.

## References

1) P. J. Campbell Smith, LALS users manual (1978 August)

2) P. J. Campbell Smith, R. Chandrasekaran, LALS Users Manual (1983)

3) R. Chandrasekaran, LALS Users Manual, Whistler Center for Carbohydrate Research,

Purdue University, West Lafayette, IN 47907, USA  (2000.5.19)

Appendix

## A Summary of Cycle Control Data

Cycle Control Data                                  Action Page Number

AC=n  	Apply constraints                   	30, 33

AL  	Same as CN, EL, CS, XR, XS, MA     	30

BO  	Bonding details                      	30

CC=n  	Create coordinates on a file         	31

CD=n  	Contact details                    	30

CE  	Uncertainties in coordinates       	32

CL=n  	Run to convergence with a limit of n cycles 	29

CN 	Constraints and restraints print     	30

CO  	Coordinate calculation               	29

CR=n  	Print coordinates                    	30

CS  	Contact statistics                   	30

CX  	Till contact calculation             	29

CY=n  	Run n cycles                         	29

DG=n  	Various diagnostics print            	30

EH=n 	Elasticate thetas                    	29, 32

EL  	Print elastic parameters             	30

EO=n  	Elasticate other parameters          	29, 32

ET=n  	Elasticate taus                      	29

FC  	Fourier coefficients                 	31

FI  	Filtering details                    	31, 32,33

FO  	Force continuation even if diverging 	31

LI=n  	Contact search after every n cycles  	32

MA 	Normal matrix summary                	30

MF=n  	Model fit data to be used            	32

NC  	No contacts                          	32

ND=n  	Number of data for error calculation 	32

NF  	No forcing                           	32

NH  	No heading                           	31

NS  	Normal shifts                        	31

NX  	No X-ray data                        	32

OP  	Suppress printing of current values  	30

OS  	Optimized shifts                     	31

OZ  	Omit zero occupancy atoms            	31

PO  	Make this file                       	31

PU  	Copy lines to update files           	29

SC=n  	Write the update file                	30

SF=n  	Structure factor calculation         	29

SH=n  	Print eigenvalues                   	30

SP  	Suppress print                       	32

ST=n  	Step parameters                      	29, 32,36

TA=n	Tabulate function                    	32

TB=n  	Tabulate function                    	32

VH=n  	Vary thetas                          	29, 32

VN  	Vary nothing                         	30

VO=n  	Vary other parameters                	29, 33

VT=n  	Vary taus                            	29

XR  	X-ray data included                  	30

XS  	X-ray statistics to be printed       	30

YC  	Contacts to be included              	32

YX  	X-ray to be included                 	32

ZZ  	Terminator                           	29

These Codes are stored in KODE(i) . The i is the index of Control Code as follows.

Index order:

1=CY    11=VH    21=SH    31=OZ    41=YX    51=BO

2=CO    12=AC    22=SC    32=PO    42=NC    52=MF

3=SF    13=AL    23=CC    33=..    43=YC    53=DG

4=RE    14=OS    24=VT    34=FI    44=CD    54=OP

5=CX    15=CR    25=VO    35=FO    45=CS    55=..

6=ZZ    16=CN    26=EH    36=NS    46=TB    56=..

7=CL    17=XR    27=ET    37=NF    47=SP    57=..

8=PU    18=XS    28=EO    38=NH    48=VN    58=..

9=DU    19=MA    29=EL    39=LI    49=ND    59=..

10=ST    20=TA    30=FC    40=NX    50=CE    60=..

Alphabetical order:

AC=12    CS=45    FI=34    NX=40    SP=47    YC=43

AL=13    CX= 5    FO=35    OP=54    ST=10    YX=41

BO=51    CY= 1    LI=39    OS=14    TA=20    ZZ= 6

CC=23    DG=53    MA=19    OZ=31    TB=46    ..=33

CD=44    DU= 9    MF=52    PO=32    VH=11    ..=55

CE=50    EH=26    NC=42    PU= 8    VN=48    ..=56

CL= 7    EL=29    ND=49    RE= 4    VO=25    ..=57

CN=16    EO=28    NF=37    SC=22    VT=24    ..=58

CO= 2    ET=27    NH=38    SF= 3    XR=17    ..=59

CR=15    FC=30    NS=36    SH=21    XS=18    ..=60

## B Examples of Cycle Control Data

B.1----*----1----*----2----*----3----*----4----*----5----*----6-----*---

CY = 1, VT = 4, AC = 2, AL.

4     6     7     8

1     2

CL = 6, CN, SH.

CO, AL.

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This performs one cycle with full printout, refining 4 Taus and applying two constraints, followed by up to 6 cycles refining the same parameters and  applying the same constraints with less print, followed by a coordinate calculation.

B.2----*----1----*----2----*----3----*----4----*----5----*----6-----*---

CY, VH = 3, EH = 1, VO = 4, VT = 7, AL, FI, OS, NX, LI = 2, AC = 9.

10    15   17

12

37.2

0.04

7     8     9    10   11   12   13

1     2     5     6

1     2     5     3    4    9     8      7    6

10.0      1.0       1.0       3.0

CL = 10, CD, SH, CN.

CL = 10, YX, XR, CD, SH, CN.

CO, AL, CC = 2, SC.

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This performs 1 cycle with full print, refining 4 thetas against (presumably) contact data, one of which is bound to a standard value of 37.2 with weight 0.04. Four other parameters and 7 Taus are also refined, and 9 constraints are applied. Optimized shifts are used, and the contact table research frequency is set to every 2 cycles. The shift maxima are reset to 10.0 for angles and 1.0 Angstrom for distances, and 3 eigenvalues are to be filtered. Ten (at most) cycles of the same  follow, with less print. Refinement against x-ray data (as well as contacts) is then switched on, and a further 10 cycle refinement performed. Finally, the coordinates are calculated, copied to an updating file, along with updating lines.

B.3 ----*----1----*----2----*----3----*----4----*----5----*----6----*---

CL = 3, VH = 4, AC = 5, CN, SH.

8     9    10     7

1     2     3     4    5

CL = 3, VH = 2, AC = 5, CN, SH.

8    10

1     2     3    4     5

SF, AL.

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This performs a maximum of 3 cycles (using default convergence criteria) refining 4 thetas, applying 5 constraints, and printing only the constraints and shifts. A similar operation is then performed varying only 2 thetas. A structure factor calculation is then made.

B.4 ----*----1----*----2----*----3----*----4----*----5----*----6----*---

ST = 2, TB = 4, SP.

3     7   0.0        0.0        10.0       60.0

3    10   0.4        0.0         0.05       0.70

999999999

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This is a stepping run. ST = 2 implies that the contact calculation is performed, TB = 4 implies that the contact sigma is to be tabulated with intermediate print suppressed (SP). The two variables stepped are other(7) from 0.0 to 60.0 in steps of 10.0 and other(10) from 0.4 to 0.7 in steps of 0.05. Thus, 49 cycles will be executed.

B.5 ----*----1----*----2----*----3----*----4----*----5----*----6----*---

CL = 5, MF, VH = 3, VO = 3.

7    8     9

5    6     7

1     MOL

C1      1.234   2.345   3.456   .01

C2      7.890   8.901   9.012   .01

C3      3.456   4.567   5.678   .01

99999

CO, AL.

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This is a model fitting run. Three atoms in molecule 1 are fitted to the coordinates given here, with weight each of 0.01. Three thetas and 3 other parameters are varied in order to achieve this.

B.6----*----1----*----2----*----3----*----4----*----5----*----6----*---

RE=7.

CL=4,VH=6,CN,SH,AC=2,LI=2.

4     6     8    10    12   14

1    9

CX,AL.

----*----1----*----2----*----3----*----4----*----5----*----6----*---

This is a restart run. The dump file of some previous run is scanned and the parameter values at the end of cycle 7 of that run are read. A maximum of four additional cycles are then performed with the specified variables and constraints, and a final contact calculation. Is made.

## C Error Messages

Certain error conditions cause abandonment of the whole run. These generate a message, possibly accompanied by certain useful numbers, denoted I1, I2, r1 etc.

For example,

*** WinLALS error 9      Bessel fun arg too big ***

*** I1 = 8,                I2 = 53

*** r1 = 3.26e+01,         r2 = 7.01e+00.

The errors and meanings of the values given are as follows.

1   There have been I1 errors in the data, denoted by a suitable message.

2   The cycle control line has zero or more than one operation code.

3   CY or CL has been specified but no variable parameters.

4   I1 errors have been detected in the supplied indices (listed already).

5   Transform I1 was requested in constraint specification I3 but only I2 transforms have been defined so far.

6   Atom I1 was requested in constraint specification I3 but it is not in structure I2.

7   Continuous transform requested but the helix axis of molecule I1 is not at (0, 0).

8   No Bessel function orders contribute to spot I1, reflection I2.

9   The Bessel function argument is out of range (currently 0 to 30), for atom I1, reflection I2. The argument is r1 and the reciprocal r is r2.

10  The matrix inversion has failed for reason I1. This is unlikely, see the appropriate listings for details.

11  No excess of data over variables - there are I1 data of which I2 are rejected spots, I3 applied constraints and I4 net variables.

12  The theta equiv no I1 puts theta I3 equivalent to theta I1 which is non-existent.

13  An array specifier is too small - its name is given.

14  Constraint specification I3 calls for molecule I1, but there are only I2 molecules defined.

15  A Cruickshank spot weight is negative, viz. r1, for spot I1. Fo scaled is r2.

16  An atom type I1 has been given for which no scattering factor code is supplied. It is atom I2.

17  A scattering factor is out of the interpolating range. For spot I1, rho squared is r1.

18  The refinement appears to be diverging, or something is very wrong. Either the constraint sum (r1) is greater than 50, or the weighted R-factor (r2) is greater than 100, or the shift scaling factor (r3) is less than 0.001. This can be overridden by the cycle control line option FO, if desired.

- The bond length to atom I1 is zero or negative.

20  The parameter index in a COO or PAR constraint or restraint is out of the valid range. It is I1. If I2 is 1 (2, or 3) it is a theta (Tau, or other).

21  The x-ray scale factor is too small (less than 1.0e-10).

22  In a constraint, 2 points are too close.

23  In an angle constraint, the angle is too close to zero or 180 degrees.

24  In a dihedral constraint, the angle is ill-defined. Either two consecutive points are too close, or three are too nearly collinear.

25  In a contact unit specification, specifier I1 gives molecules I2 to I3, one of which is outside the defined range.

26  In a contact unit specification, the residue or molecule numbers are the wrong way round. Its specifier I1 is source if I2 = 1 or target if I2 = 2.

27  A bond has been explicitly specified to a redundant atom. I1 and I2 are the atoms, I3 and I4 are their Lsat values (see below)

28  There are no atoms in the sorted and weeded atom list

29  Theta equivalence error - atom I1 has a non-zero derivative with respect to theta I2.

30  The cycle specified by re parameter does not correspond to any record on the dump file.

31  The m=0 Bessel order option has been selected for TRIG.

32  This atom, I1, is already redundant to something other than atom I2. Its Lsat value (see below) is I3, and this is redundancy line I4.

33  This atom, I1, is specified as being redundant to atom I2, which is itself redundant, having an Lsat value (see below) of I3. If I4 = 1, this was detected in the type I bond detection cycle; if I4 = 2, it was in a precursion search; if I4=3 or 4 it was in the type II bond cycle or precursion test.

34  The parameter corresponding to matrix row I1 has no non-zero weighted derivative.

35  The constraint corresponding to matrix row I1 has no non-zero derivative.

36  There is a negative number of degrees of freedom in the system. There are I1 variables and I2 constraints upon them.

37  Atom I1 in the sorted and weeded atom list has an illegal type, I2.

38  The unit cell volume is zero or negative.

39  No eigenvalues are left after filtering. There are I1 equations and I2 eigenvalues have been accepted.

40  Illegal molecule definition, sequence no I1. One or more of the position, equivalence or structure numbers is less than 0, equal to 0, or greater than the number defined.

41  Illegal set or step parameter. Either the type (I1) is not 1, 2, or 3, or the index (I2) is out of range.

42  Too many variables are being stepped (more than 4).

43  Source or target molecule I1 derives from structure I2, but all its atoms have been removed for some previous reason.

44  The angle-dependence parameters for attractive bond type I1 are faulty. Either angle is not in the range 90 - 180, or deviation is not in the range 1 - 90.

45  The attractive interaction type I1 parameters are faulty. The distances RMI, REQ, RMA, CLIM are not in increasing order. The values given as r1, r2 and r3 are RMI, REQ and RMA.

46  In a degenerate structure (less than 4 atoms) an attempt has been  made to refine an illegal parameter. It is structure I1, varied other no I2, other parameter I3.

47  The stepping interval is not in the direction starting towards finishing value. It is the I1th stepped variable.

48  The cycle control line contains a syntactic error at or to the right of the underlined character(s).

49  The donor atom (I1) in an angle-dependent interaction has no precursor. This is interaction type I2.

50  For attractive interaction type I1, the donor and/or acceptor list is empty.

51  Atom I1, involved in attractive interaction type I2, is not in the sorted and weeded atom list (swal).

52  An illegal T-code (I1) has been given for target I2. Probably it is too large.

53  Equivalence I1 is illegal. It has non-zero elements in positions 3, 6, 7 or 8. Molecule I2 was being processed.

54  Equivalence I1 is illegal. It does not specify a rotation in the  given unit cell. If the equivalence is correct, check that the unit cell parameters are correct for the space group. Molecule I2 was being processed.

55  An illegal scattering factor code (I1) has been given for atom type I2.

56  Molecule I1 does not have helix symmetry (Bessel only).

57  (Unused)

58  Coordinate error (CE) estimation has been requested in the wrong context (see the description of CE under cycle control data).

59  An illegal function (I1 or I2) has been specified for tabulation. Note - this will only be detected if very wrong. Some undefined numbers will produce tabulation of use only to the programmer.

60  Some number has exceeded its maximum possible value, dictated by word-packing  considerations. The exact parameter is given by I1 as follows.

I1 = 1  Number of atoms in input list            	maximum 999

= 2  Position number in molecule definition   	maximum 99

= 3  Structure number in molecule definition  	maximum 99

= 4  Equivalence number in molecule definition	maximum 99

= 5  Indices h, k or l                          	minimum -100, maximum 99

= 6  Multiplicity of reflection               	maximum 99

= 7  Mol no in contact specification            	minimum -499, maximum 499

= 8  a, b or c steps in targets                	minimum -4, maximum 5

= 9  Residue numbers in contacts              	minimum -49, maximum 50

Since these are independent of array specifiers, the only way around is reprogramming.

61  MF = 2 has been specified, but no model fitting data have previously  been read.

62  Faulty model fitting data. Either the first line is not a molecule line, or an atom name (given) is not in the input list.

63  The first stepped parameter has been specified as linked.

64  In scattering factor specification I1, the van der Waals radius or the h--a bond length has been given as zero but is needed.

65  Illegal molecule number, I1, in model fitting data.

66  Conflict between atom number (I1) and molecule number (I3) in  model fitting data. Atom I1 is on structure I2 that is not  used for molecule I3.

The following errors are compiler dependent.

98  Fortran run time error no I1. See the Fortran manual if the provided information is not sufficient.

99  System interrupt for the reason given (code I1). For time, IOU or track limits, no special action is taken. For output limit, a error file message gives the file (output) affected.

D WinLALS Source Files, Subroutine Name and Structure

a) Filename      	Description

cntcts.f       	dimensioned source for contacts subroutines

common.f      	common utilities for WinLALS

coords.f       	dimensioned source for coordinate calculation subroutines

filmat.f       	dimensioned source for matrix filling subroutines

main.f         	dimensioned source for main, input and utility routines

sfbesl.f       	dimensioned source for Bessel scattering factor routines

sftrig.f       	dimensioned source for trig scattering factor routines

solve.f        	dimensioned source for matrix solution routines

b) Subroutine names

cntcts.f                       	contacts subroutines

SUBROUTINE CNTCTS   	driver for contact calc subroutines

SUBROUTINE CONSET   	set up necessary tables etc for contact calc

SUBROUTINE CONTAB  	do contact search

SUBROUTINE CONUSE  	apply contacts from list

SUBROUTINE ISORT    	Shell sort multiple word records

SUBROUTINE MOVE2V  	calc vector

common.f                   	common routines

SUBROUTINE CPUTIM  	CPU time routine

SUBROUTINE LOWTOUP 	transform lower to upper case characters

SUBROUTINE OPNUNI   	open all logical unit for WinLALS

coords.f               	coordinate calculation subroutines

SUBROUTINE APPLY    	common call to con- or restraint application

SUBROUTINE CONELA   	apply con/restraints (CONSTR), elastication (ELASTC)  model fitting (MODEFT) as appropriate

SUBROUTINE CONIN    	apply constraint if user desires it

SUBROUTINE CONSTR  	apply constraints and restraints

SUBROUTINE COORDS  	generate coordinates (P) and derivatives (DP) from stereochemistry input

SUBROUTINE DERIV    	calc derivative of error vector and write other param

SUBROUTINE DIHED    	calc dihedral angle

SUBROUTINE ELASTC   	apply parameter elastication

SUBROUTINE MATMUL 	multiply matrices

SUBROUTINE MODLFT  	apply model-fitting if appropriate

SUBROUTINE REDANG  	calc reducing angles (defined in Manual)

SUBROUTINE RESIN    	apply restraint if desired

SUBROUTINE TRANSP  	transpose DP into DPT

filmat.f                       matrix filling subroutines

SUBROUTINE FILMAT   	complete normal matrix, print summary of it

SUBROUTINE SFSTAT   	calc Cruickshank weight parms

main.f                        main, input and utility routines

PROGRAM LALS        	main, linked-atom Least-squares refinement system

SUBROUTINE ABSMAX  	error exit for numbers too big to pack

SUBROUTINE ARRAYS  	check array size sufficiency

BLOCK DATA          	common data statements

SUBROUTINE CHAT     	check atom number/name correspondence

SUBROUTINE CIATOM   	constraint atom input

SUBROUTINE CIBUFF 	constraint real parm input transfer

SUBROUTINE CONCRD 	read and parse cycle control card

SUBROUTINE CROSS    	cross product of two vectors, R3 = R１x R2

SUBROUTINE CYCLER   	control flow based on cycle control

SUBROUTINE ERROR    	flag error and stop

SUBROUTINE INPUT1   	input, checking, preprocessing of main data

SUBROUTINE INPUT2   	more of same

SUBROUTINE LAYOUT   	provide page handling and subheading services

SUBROUTINE MOVE1    	premultply vector V(1-3) by matrix A(1-9)

SUBROUTINE MOVE2    	premultply vector V(1-3) by matrix A(1-9) and vector A(10-12)

SUBROUTINE NOW      	get time and date from system

SUBROUTINE PARMIN   	sort out input parameter numbers, check validity

SUBROUTINE PROGRS   	summary file manager

SUBROUTINE SCOUT    	generate SC card output

SUBROUTINE SECOND   	measure machine CPU time

SUBROUTINE SSP      	reset parameter and report on printout

SUBROUTINE SYSUSG   	call operating system and request resource usage information, the print it

SUBROUTINE TABULE   	print tabulated functions after stepping

SUBROUTINE TRAPIT   	system dependent routine supplied by installer to set error traps

SUBROUTINE VERSHN   	print version identifier

SUBROUTINE WARN     	log and count warning messages

SUBROUTINE WHISPR   	used to inform console, monitor terminal, etc. of progress of job

open.for                     	Input/Output units handling routines

SUBROUTINE CPUTIM   	CPU time routine

SUBROUTINE LOWTOUP 	transform lower chars into upper chars

SUBROUTINE OPENUNI 	Input/Output unit handling routine

sfbesl.f                       	Bessel scattering factor routines

SUBROUTINE SFBESL   	calc Bessel structure factors and apply

SUBROUTINE BESJ     	calc Bessel functions of the first kind of positive integral order

SUBROUTINE BESSET   	set up table of Bessel fns, orders BOMAX & BOMAX+1

sftrig.f                       	trig scattering factor routines

SUBROUTINE SFTRIG   	calc Trig structure factors and apply

SUBROUTINE MATMPY  	premultiply S(1-9) by R(1-9)

solve.f                        matrix solution routines

SUBROUTINE IMTQL2V  	find the eigenvalues and eigenvectors of a symmetric  trigonometric matrix by the implicit QL method

SUBROUTINE OPTSCL  	determine optimum shift scaling for minimizing constraint sum

FUNCTION   PYTHAG  	for DSQRT(A*A + B*B)

SUBROUTINE RSWZR    	calc normal and residual

SUBROUTINE SOLVE    	solve the normal equations and update parameters

SUBROUTINE SWCH     	switch I and J

SUBROUTINE TRED2V   	reduce a real symmetric matrix to a symmetric tridiagonal matrix using and accumulating orthogonal similarity transformations

SUBROUTINE WRSHFT  	print shift etc and std err if requested

c) Structure

WinLALS-------OPNUNI

|---SECOND

|---INPUT1-----------NOW--------------GETTIM

|              |---VERSHN

|              |---ABSMAX

|              |---ARRAYS

|              |---CHAT

|              |---WARN

|              |---LOWTOUP

|              |---CIBUFF----------ARRAYS

|              |---CIATOM----------CHAT

|                             |---ARRAYS

|---INPUT2-----------ARRAYS

|              |---ABSMAX

|              |---CHAT

|---CYCLER----------PROGRS

|              |---WHISPR

|              |---SCOUT

|              |---CONCRD

|              |---PROGRS--------WARN

|              |---ARRAYS

|              |---PARMIN

|              |---CHAT

|              |---SSP

|              |---TABULE

|              |---SECOND

|---COORDS--------ARRAYS

|              |---MOVE2

|              |---REDANG

|              |---WARN

|              |---MATMUL

|              |---CROSS

|              |---TRANSP

|              |---MOVE1

|              |---DERIV-----------MOVE1

|---CONELA--------CONSTR----------APPLY-----------CONIN

|              |                           |---RESIN

|              |---ELASTC

|              |---MODFTC

|              |---MODLFT--------MOVE1

|                             |---MOVE2

|                             |---WARN

|---CNTCTS--------CONSET------------ABSMAX

|              |              |---ARRAYS

|              |---CONTAB----------MOVE2V

|              |              |---MOVE2

|              |              |---ARRAYS

|              |              |---ISORT

|              |              |---WARN

|              |---CONUSE---------ABSMAX

|                             |---ARRAYS

|                             |---WARN

|                             |---MOVE2

|                             |---MOVE1

|---SFBESL---------BESJ

|              |---BESSET

|              |---WARN

|              |---ARRAYS

|---SFTRIG-----------MATMPY

|              |---WARN

|---SYSUSG

|---FILMAT-----------SFDTAT--------WARN

|              |---WARN

|---SOLVE------------TRED2V

|---IMTQL2V

|---RSWZR

|---OPTSCL--------WARN

|             |---SWCH

|---WARN

|---WRSHFT--------WARN

ERROR--------SECOND

|---SCOUT

|---PROGRS

|---ARRAYS

## E Array Specification and COMMON Header files

a) Array Specification

WinLALS contains various arrays whose minimum size is problem dependent. it checks all its array usage against these values, printing an error message if any is exceeded.

Array   Dim.   meaning

ADEL 	150 	Number of atom redundancies in contact input.

APPR 	600	Number of constraints and restraints simultaneously applied.

ATOM	3000	Number of atoms input.

BNDI	6000	2 * no. of type I bonds + 3 * no. of type II bonds + 9 * no. of type III bonds + no. of atoms in all attractive bond donor/acceptor lists, including the 2 terminating dummies for each type.

BNDL	10000	Bond storage work array length, needed for contact calculation. Needs to be about 3 * no. of atoms plus a bit.

CLIS	10000	Contact list length. Should not be less than LIS1 on the contact option line (2.26).

CMOL	500	Number of molecules or bits of molecules used in source and target descriptions, after expanding the -inclusive molecule no.- Specifications.

COND	600	Number of constraints and restraints defined. Note that some codes (e.g. COI) generate more than one constraint or restraint.

CONP	2000	Length of constraint parameter list, as follows

13 for every TRA or EQU

4 for every COI or GAM

5 for every DIS or COL

6 for every ANG

7 for every DIH

3 + 2 * NP for a PAR or COO involving NP parameters or coordinates

NM + 1 if NM molecules are involved in model fitting,

4 * NA if NA atoms are involved in model fitting.

CONT	 50	Number of TRA and EQU constraint transform specifications.

EPAR	410	Number of elastically bound parameters.

EQIV	10	Number of equivalences specified (for molecule definition).

EQUN	999	Number of normal equations, being the sum of the number of varied parameters and the number of constraints (not restraints) applied.

HBND	2000	Number of types of attractive interaction.

MOLC	5000	Number of molecules defined (by combination of structure, equivalence and position)

ORDR	100	Largest number of Bessel orders contributing to one layer line. This will not exceed (40/NRES) + 1. (ignored by TRIG)

POSN	100	Number of positions defined (sets of Mu, u, v, w).

REFL	30000	Number of X-ray reflections input. If continuous transform data is input, add NL + 1 where NL is the number of layers requested.

SCAT	50	Number of species in the scattering factor definitions, including the 8 pre-defined ones.

SOUM 	500	Number of source definition lines for contacts.

SPOT 	15000	Number of X-ray spots input. If continuous transform data is input, add 3.

STRU	100	Number of structures defined.

TARM	2000	Number of target definition lines input (including lines defining all a, b, c translations)

THEQ	1000	Number of theta equivalences defined.

TYPE	50	Larger of number of types of atomic scattering factors defined, and number of sets of contact distances input.

VARY	500	Maximum of VH + EH, VT + ET, VO + EO on a cycle control line.

VPAR	500	Number of varied parameters.

VSKL	500	Number of varied skeletal parameters, these being thetas, taus, epsilons and S's.

Array Specifiers are changed

ADEL   150,  APPR   600,  ATOM  3000,  BNDI  6000,  BNDL  4000,  CLIS 10000,  CMOL   500,

COND   600,  CONP  2000,  CONT    50,  EPAR   410,  EQIV    10,  EQUN   999,  HBND  2000,

MOLC  5000,  ORDR   100,  POSN   100,  REFL 30000,  SCAT    50,  SOUM   500,  SPOT 15000,

STRU   100,  TARM  2000,  THEQ  1000,  TYPE    50,  VARY   500,  VPAR   500,  VSKL   500.

b) Summary of Include files used in WinLALS

file name           	COMMON name

'CMATOMS.H'      	/ATOMS/

'CMBLANK.H'      	/      /

'CMCCDATA.H'     	/CCDATA/

'CMCDERIV.H'     	/CDERIV/

'CMCELLCO.H'     	/CELLCO/

'CMCONS.H'     	/CONS/

'CMCONTAX.H'     	/CONTAX/

'CMDATA0.H'      	/DATA0/

'CMDATA2.H'      	/DATA2/

'CMDATA3.H'      	/DATA3/

'CMDATA5.H'      	/DATA5/

'CMDATA6.H'      	/DATA6/

'CMHEAD.H'       	/HEAD/

'CMHOLD.H'       	/HOLD/

'CMLOCAL1.H'     	/LOCAL1/

'CMNONST.H'      	/NONST/

'CMNUMBERS.H'    	/NUMBERS/

'CMPARAMS.H'     	/PARAMS/

'CMSCRTCH.H'     	/SCRTCH/

'CMSUMMRY.H'     	/SUMMRY/

'CMTABLE.H'      	/TABLE/

'CMVALUES.H'     	/VALUES/

'CMWATER.H'      	/WATER/

'CMUNIT.H'       	/COMUNIn/

c) Details of include files

CMATOMS.H                                                               CMATOMS

COMMON /ATOMS/ CRYSTR(18),DELPHI,DELZ,P(5, 3000),TRMOL(30, 5000)  CMATOMS

CMBLANK.H                                                               CMBLANK

COMMON /     / A(998001),B(  999),C(  999)  !999*999=998001       CMBLANK

COMMON /11111/ SHARE(4500000)               !=3*3000*500          CMBLANK

COMMON /22222/ SHARE2(4500000)              !=3*500*3000          CMBLANK

CMCCDATA.H                                                              CMCCDATA

COMMON /CCDATA/ IFK(60)                                           CMCCDATA

CMCDERIV.H                                                              CMCDERIV

COMMON /CDERIV/ PX(3),IAT,IMOL,IPOX,IZ,LESK,LETT      !/CDERIV/   CMCDERIV

CMCELLCO.H                                                              CMCELLCO

COMMON /CELLCO/ ANG,ASTAR,BSTAR,CA,CAS,CB,CBS,CC,CCS,CPS,CSTAR,   CMCELLCO

*                SA,SB,SC,SCS,VOL                                  CMCELLCO

CMCONS.H                                                                CMCONS

COMMON /CONS/ BADG,DG(  500),G,SUMF,SUMGA,SUMGB,WD,WEI,IFAPP,     CMCONS

*              INUMC,ISTOR,MV,MW                                   CMCONS

CMCONTAX.H                                                              CMCONTAX

COMMON /CONTAX/ AXSEPL,VSEPL,VSEPL2,ICOM(  500),IGEN(  500),      CMCONTAX

*                IXBD(3, 3000),LBOND( 6000),LSAT( 3000),           CMCONTAX

*                LSWIND(  100),LTAB(2,10000),NBND,NPH,NSGEN,NTAB,  CMCONTAX

*                NTGEN,NTIII                                       CMCONTAX

COMMON /CONTAXL/ QBOND(10)                                        CMCONTAX

LOGICAL          QBOND                                            CMCONTAX

CMDATA0.H                                                               CMDATA0

COMMON /DATA0/ D( 3001),ELAWT,OCC( 3001),OTHER( 1000),            CMDATA0

*               OUNIT( 1000),SOCC(  101),SSYM(12,   11),TAU( 3001),CMDATA0

*               THETA( 3001),UCELL(6),ISNS(  501),JHAND,JMFA,JMFT, CMDATA0

*               KOID( 1000),LCW(  600),LMIND( 2000),LP( 3001),     CMDATA0

*               LQP(3, 1000),LSIND(2,  100),LSTRC( 3000),          CMDATA0

*               LTYPE( 3001),NCNP,NQP,NRES,NTRN                    CMDATA0

COMMON /DATA0C/ CHKNAM( 3001),CHKOID( 1000)                       CMDATA0

CHARACTER       CHKNAM*4,     CHKOID*4                            CMDATA0

CMDATA2.H                                                               CMDATA2

COMMON /DATA2/ CNP( 2000)                                         CMDATA2

CMDATA3.H                                                               CMDATA3

COMMON /DAT31/ CRUIK(5),SCTB(11,  50),LABN,LREJ,LREJT,LSCT(  50), CMDATA3

*               NHEL,NHKL,NSCT,NTYPE,ITLHH(30000),ITLHK(30000)     CMDATA3

COMMON /DAT32/ ITLHL(30000),ITLHM(30000),LOV(30000) !,LHKL( 2000) CMDATA3

COMMON /DAT33/ FOBS(30000),FWT(30000)                             CMDATA3

CMDATA5.H                                                               CMDATA5

COMMON /DATA5/ CONLIM,DAMP,FLTR,SHFTMX(2),MTEST,NFLTR             CMDATA5

CMDATA6.H                                                               CMDATA6

COMMON /DATA6/ HBPAR(6, 2001),VRAD(   55),WCON,WRAD(   55),       CMDATA6

*               LADEL(3,  150),LBDIN( 6000),LSU(4, 2001),          CMDATA6

*               LTU(9, 2001),MHBA,MHBB,MINCP,NADEL,NBI,NBII,NBIII, CMDATA6

*               NHB,NMAXC,NSM,NTM,NTYPX                            CMDATA6

CMHEAD.H                                                                CMHEAD

COMMON /HEAD/ TIM(9),TIM0,ILAY,IPNO,JFLAG                         CMHEAD

COMMON /HEADC/ CHKAP(22)                                          CMHEAD

CHARACTER      CHKAP*4                                            CMHEAD

CMHOLD.H                                                                CMHOLD

COMMON /HOLD/ CRU(3,10),FR,SIGO,SO(3),STEP(6,4),ICTAB,ICYC,IENT,  CMHOLD

*              IFG,IFL1,IFR,IFRQC,IFSET,IGO(4),INOPRT,ISAV(7),     CMHOLD

*              ITB1,ITB2,ITM1,ITM2,LTLOC(   50),NAPCX,NFOUT,NLOOP, CMHOLD

*              NSAT,NSTV,NTFM,NVOTHX,NVTAUX,NVTHTX                 CMHOLD

COMMON /HOLDL/ QFRCE,QNSH,QOPT,QSTEP                              CMHOLD

LOGICAL        QFRCE,QNSH,QOPT,QSTEP                              CMHOLD

CMLOCAL1.H                                                              CMNONST

COMMON /LOCAL1/ BUF(6),LBUF(20)                                   CMLOCAL1

COMMON /LOCAL1C/ CHLBUF(20)                                       CMLOCAL1

CHARACTER        CHLBUF*4                                         CMLOCAL1

CMNONST.H                                                               CMNONST

COMMON /NONST/ NNONST                                             CMNONST

CMNUMBRS.H                                                              CMNUMBERS

COMMON /NUMBRS/ NAPPC,NAPRL,NAT,NCON,NDATA,NEMIS,NEPAR,NEQUIV,    CMNUMBERS

*                NEQS,NGAP,NMOL,NOTH,NPOS,NRPAR,NSPOT,NSTRC,NVPAR, CMNUMBERS

*                NVSK,KSPOT                                        CMNUMBERS

CMPARAMS.H                                                              CMPARAMS

COMMON /PARAMS/ EPAR(  410),SIGE,WEPAR(  410),INDEX(2,  500),     CMPARAMS

*                LAPRL(  600),NEOTH,NETAU,NETHT,NVOTH,NVTAU,NVTHT  CMPARAMS

COMMON /PARAMSL/ QCONX,QXRAY                                      CMPARAMS

LOGICAL          QCONX,QXRAY                                      CMPARAMS

CMSCRTCH.H                                                              CMSCRTCH

COMMON /SCRTCH/ DUMMY(25)                                         CMSCRTCH

CMSUMMRY.H                                                              CMSUMMRY

COMMON /SUMMRY/ SSS(32),ISSS(21)                                  CMSUMMRY

CMTABLE.H                                                               CMTABLE

COMMON /TABLE/ FUNTAB( 1001,2)                                    CMTABLE

CMVALUES.H                                                              CMVALUES

COMMON /VALUES/ DEGREE,FIVEPI,HAFPI,PI,RADIAN,TWOPI               CMVALUES

CMWATER.H                                                               CMWATER

COMMON /WATER/ FUDGE                                              CMWATER

CMUNIT.H                                                                CMUNIT

COMMON /CMUNTC1/ TITLE(11),LAYHX(9),CCARD(100)                    CMUNIT

CHARACTER        TITLE*10,LAYHX*6,CCARD*10                        CMUNIT

COMMON /CMUNIT1/ NSCR1,NSCR2,NSCR3,NFL9,NFL10,NFL11,NFL12,NFL14,  CMUNIT

*                 NFL15,NFL16,NFL17,NFL20,NPLOT,NPF                CMUNIT

COMMON /CMUNIT2/ IRDI,IRDO,IRDPRN                                 CMUNIT

COMMON /CMUNTC2/ CFL9,CFL10,CFL11,CFL12,CFL14,CFL15,CFL16,CFL17,  CMUNIT

*                 CFL20,CFL44,CFL55                                CMUNIT

CHARACTER*40     CFL9,CFL10,CFL11,CFL12,CFL14,CFL15,CFL16,CFL17,  CMUNIT

*                 CFL20,CFL44,CFL55                                CMUNIT

d) Other common used in WinLALS

cntcts.f

COMMON /CONTAB1/ X(3),W(3),ISTAT(7),JMPSS(4),KREAS(4),KEY(2),     2001/5/2

*                 KDI(3),KSCAN(2)                                  2001/5/2

COMMON /CONTAB2/ BIGRD(  100),IXRES( 500),IXAT(4000)              2001/5/2

COMMON /CONTAB3/ DZ( 3000),DQ( 3000),PT( 3000,3),LTYT( 3000)      2001/5/2

COMMON /CONTAB4/ PS(4000,3)                                       2001/5/2

COMMON /CONUSE1M/ X(3,3),DX(3, 3000,2),DG( 3000)                  2001/5/2

COMMON /CONUSE2/ BIT1(  999),QBIT2(  999),DGT(  999)              2001/5/2

LOGICAL          BIT1,       QBIT2                                2001/5/2

sfbesl.f

COMMON /SFBES1/ FF(   50),FZ(   50),FW(   50),KBUF(20),SUMS(28),  2001/5/2

*                BJ(83),BK(42),LROTH(2,4),BL(42)                   2001/5/2

COMMON /SFBES2/ DFP(3, 3000),DFPX(3,3000)                         2001/5/2

COMPLEX*16      DFP                                               2001/5/2

COMMON /SFBES3/ TELS(100),TWAS(100),CORIG(3, 5000),ORMOL( 5000)   2001/5/2

COMMON /SFBES4/ DFO(  400),DFOX(  400),DFOT(  400),DF(  450)      2001/5/2

COMPLEX*16      DFO                                               2001/5/2

COMMON /SFBES5/ G(100,100),DGP(3, 3000,100),X2V(3, 3000)          2001/5/2

COMPLEX*16      G,         DGP,           X2V                     2001/5/2

sftrig.f

COMMON /SFTRIG1/ KANN(4),SUMS(14),KBUF(20),LROTH(2,4),            2001/5/2

*                 T3(9),T4(9),S(3),FF(   50),FZ(   50),FW(   50)   2001/5/2

COMMON /SFTRIG2/ IATVS(100),IATVL(100),SITEM(100),IEQM(100),      2001/5/2

*                 IHM(100),IFATM(100),ILATM(100),LROTHM(2,4,100),  2001/5/2

*                 CORIG(3,  5000)                                  2001/5/2

COMMON /SFTRIG3/ DFO(  400),DFOX(  400),DF(  450)                 2001/5/2

COMPLEX*16       DFO                                              2001/5/2

COMMON /SFTRIG4/ T33(500,9),T44(500,9),T44V(500,3),T33V(500,3)    2001/5/2

COMMON /SFTRIG5/ IXDFPT( 3000),DFP(3, 3000),DFPX(3, 3000),        2001/5/2

*                 X1V(3, 3000),PTR( 3000,3),PTRT( 3000,3),         2001/5/2

*                 TEMP( 3000)                                      2001/5/2

COMMON /SFTRIG6/ IATV(5000),ITYPEV(5000)                          2001/5/2

COMMON /SFTRIG7/ PM(32000,3)                                      2001/5/2

COMMON /SFTRIG8/ X22(32000),X2(32000),FAC(32000)                  2001/5/2

COMPLEX*16       X22,       X2                                    2001/5/2

COMMON /SFTRIG9/ CTEMP( 3000),DFPT( 3000,3)                       2001/5/2

COMPLEX*16       CTEMP,       DFPT                                2001/5/2

COMMON /SFTRIG10/ QATM( 3000)                                     2001/5/2

LOGICAL           QATM                                            2001/5/2

e) Variables used in WinLALS

A    	= Normal Matrix

AXSEPL 	= Maximum Axis Separation between two Molecules which still allows contacts

B,C    	= Normal Equation is A*C=B (solve for C)

CHKAP  	= Title line

CRUIK  	= Overall Weights for X-ray data

DELPHI 	= Rotation per Residue

DELZ   	= Z-translation per Residue

FOBS   	= Observed X-ray data

FWT    	= Individual Reflection Weights

IFK    	= Control Card data (see Subr CONCRD)

INDEX  	= Map between Parameter Number (Normal Matrix Row/Column) and Parameter Number (as in Tau number 5, etc)

ITLHH  	= -.

ITLHK  	= |individual Miller Index and Multiplicity for each reflection

ITLHL  	= |(replace LHKL)

ITLHM  	= -'

KNAM  	= Atom Names

KOID   	= *OTHERS* identification

LBOND 	= List of Atom Bonds (?)

LHKL 	= Miller Indices of each Reflection

LOV  	= Number of Reflections in each Spot

LP     	= List of Atom Precursors (?)

LSAT   	= List of Sorted Atoms Table (?)

LTYPE  	= Atom Types

NAC    	= Number of Applied Constraints and Restraints (AC)

NAPPC 	= Number of Constraints

NAPRL 	= Number of Applied Constraints and Restraints

NAT   	= Number of Structures, Sets of Stereochemistry (3)

Number of Atoms

NBND   	= Number of Bonds (?)

NCON   	= Number of Constraints

NDA    	= Number of Attractive Bond Donors and Accepters (40)

NDATA  	= Total number of data (Varied Params, Applied Cnstrnts, EignVals Filtered, Data Excess and Degs of Freedom)

NE...  	= Number of Elasticated ...

NEMIS  	= Number of Elasticated Restraints

NEQUIV 	= Number of Equivalent Positions in the Unit Cell (11)

NEQS   	= Number of Equations (Dimension of Normal Matrix)

NEQV   	= Number of Equivalent Positions in the Unit Cell (11)

NETH   	= Number of Elasticated Thetas (EH)

NGAP   	= Number of Non-Structural Parameters being Varied

NHEL   	= Number of Helix having a dyad along the molecular x-azis (2)

NIB    	= Number of Type I Bond List (33)

NIIB   	= Number of Type II Bond List (35)

NIIIB  	= Number of Type III Bond List (37)

NNONST	= Number of Non-Structural Parameters. This can be increased if more refinable parameters are added along with the Scale and Attenuation parameters.

NMOL   	= Number of Position Definition (9)

Number of Generated Molecule definitions (13)

NMOL   	= Number of Molecules

NOTH   	= Number of *OTHERS*

NPOS   	= Number of Positions

NRED   	= Number of Atom Redundancy (31)

NREFL  	= Number of X-ray Diffraction data (21)

NRES   	= Number of Residues per c-repeat (2)

NSGEN  	= Number of Source Molecules generated

NSPOT  	= Number of Spots (X-ray data)

NSS    	= Number of Source Unit Specifications (27)

NSTRC  	= Number of Structures

NSV    	= Number of Set and Step data (ST)

NTGEN 	= Number of Target Molecules generated

NTHE  	= Number of Theta Equivalence (6)

NTRN  	= Number of Helix turns per c-repeat (2)

NTS   	= Number of Target unit Specifications (29)

NTYP   = Number of Scattering Factor Code (20)

NTYPE 	= Number of Contact Limit Table (42)

Number of Individual Contact Weights (45)

NV...  	= Number of Varied ...

NVPAR  	= Number of Varied Parameters

NVSKL  	= Number of Varied Skeletals: Taus, Thetas, Epsilons, S

NVTH   	= Number of Varied Thetas (VH)

OUNIT  	= UNITS OF *OTHER* PARAMETERS

P      	= Coordinates of Input Atoms

QCONX 	= True if NC control code is in Effect (?)

QXRAY 	= True if NX control code is in Effect (?)

SHARE 	= Shared between DP array and EIGVEC array

SHARE2 	= Used for DPT array

TRMOL 	= Positioning Matrices for Molecules

VSEPL 	= Maximum Separation between Atoms which may involve Contact

VSEPL2 	= VSEPL**2

## F Sample Input/Output

a) Input file 1: WinLALS.dat

Blackwell valonia cellulose  PARALLEL down P21   April 1997 Finkenstadt 1.title

1    1    2    1                                                    2.mol. sym.

Z-matrix (Internal coordinates) of WinLALS                              3-1.stereo chem., title

O5A*               3     .5000     .0000     .0000     .0000            3-2.stereo chem, Z-matrix

C1A*      O5A*     2     .5000    1.4290     .0000     .0000

O4A       C1A*     3     .5000    1.3890  107.3000     .0000

C4A       O4A      2     .5000    1.4260  115.9053  -95.9226

C5A       C4A      2     .5000    1.5250  107.8399 -143.7740

C3A.      C4A      2     .5000    1.5230  110.4000   95.3360

C6A       C5A      2    1.0000    1.5140  112.6980   64.2936

O6A       C6A      3    1.0000    1.4270  111.7990  -75.8291

O5A       C5A      3     .5000    1.4360  110.0923 -176.4164

C1A       O5A      2     .5000    1.4290  112.8223   60.6002

O4A*      C1A      3     .5000    1.3890  107.3000 -179.2821

C2A       C1A      2    1.0000    1.5230  109.5898  -61.9221

O2A       C2A      3    1.0000    1.4230  109.3000 -179.4653

C3A       C2A      2     .5000    1.5210  110.0307   58.3247

O3A       C3A      3    1.0000    1.4290  109.6020 -175.7593

C4A.      C3A      2     .5000    1.5230  109.5189  -54.8793

C5A.      C4A.     2     .5000    1.5250  110.5649   53.6467

H1A       C1A      1    1.0000    1.1000  109.4800   58.0479

H2A       C2A      1    1.0000    1.1000  109.4800  -62.3453

H3A       C3A      1    1.0000    1.1000  109.4800   65.6607

H4A       C4A      1    1.0000    1.1000  109.4800  -24.2840

H5A       C5A      1    1.0000    1.1000  109.4800  -55.9764

H61A      C6A      1    1.0000    1.1000  109.4800   46.6109

H62A      C6A      1    1.0000    1.1000  109.4800  161.7309

O5A         120.3109   60.3294  -81.8869     .9858            4-1.root def

1    2    3                                                         4-2.

99999                                                                   5terminator

C3A.      C5A       C6A       O5A       O4A*      C2A              6.theta equiv.

O2A       C3A       O3A       C4A.      H1A       C2A

H2A       C3A       H3A       C4A.      H4A       C5A

H5A       O5A       H61A      O6A       H62A      O6A

99999                                                                   7.terminator

2.7315    4.9918     .0000     .0000    1.0000                      8.X-ray refinables

-32.2576     .0000     .0000   .000000    1.0000    1.0000            9.pos. def.

-29.0224     .5000     .5000  -.255872    1.0000    1.0000

99999                                                                   10.terminator

99999                                                                   12.terminator

1    1    1    0                                                    13.gen. mol. def.

2    1    1    0

99999                                                                   14.terminator

TRA              0.0       0.0       1.0                                15.trans. const. & restr.

COI                                                                        ANG, TRE

O5A*     0    1     O5A      1    1                         EQU, GAM

COI                                                                        COI, COO, COL

C1A*     0    1     C1A      1    1                         DIS, DIH, PAR

COI

O4A      0    1     O4A*     1    1

COI

C3A      0    1     C3A.     0    1

COI

C4A      0    1     C4A.     0    1

COI

C5A      0    1     C5A.     0    1

PAR      2       0.0       0.0

OT       9    -1.000

OT      13     1.000

99999                                                                   16.terminator

8.17      7.86     10.38     90.00     90.00     97.00            17.unit cell

48.0                                                                    19.Cruick. weights, A,B,C,D,SUB

0001 0002 0004   -1                                                    20.scatt. fact. codes,

0    1    0    2     3.000        -1                                21.X-ray data,

1    0    0    2     3.000        -1

1   -1    0    2    39.000         1

1    1    0    2    27.000         1

2    0    0    2     5.000        -1

0    2    0    2

2   -1    0    2   100.000         1

1   -2    0    2     3.000        -1

2    1    0    2     3.000        -1

1    2    0    2     3.000        -1

2   -2    0    2     3.000        -1

3    0    0    2     3.000        -1

3   -1    0    2

2    2    0    2    14.000         1

0    3    0    2

1   -3    0    2    15.000         1

3    1    0    2    12.000         1

1    3    0    2

3   -2    0    2    11.000         1

2   -3    0    2     4.000        -1

3    2    0    2     4.000        -1

2    3    0    2     4.000        -1

4    0    0    2

4   -1    0    2    14.000         1

3   -3    0    2     9.000        -1

1   -4    0    2

0    4    0    2    36.000         1

4    1    0    2     9.000        -1

4   -2    0    2     9.000        -1

1    0    1    2

0    1    1    2     8.000         1

1   -1    1    2     5.000        -1

1    1    1    2     6.000         1

2    0    1    2     7.000         1

0    2    1    2     5.000        -1

2   -1    1    2

1   -2    1    2    11.000         1

2    1    1    2     5.000        -1

1    2    1    2     5.000        -1

2   -2    1    2     5.000        -1

3    0    1    2    25.000         1

3   -1    1    2

2    2    1    2    23.000         1

0   -3    1    2

-1    3    1    2    24.000         1

3    1    1    2    15.000         1

1    3    1    2     7.000         1

3   -2    1    2

2   -3    1    2    18.000         1

3    2    1    2     9.000        -1

2    3    1    2     9.000        -1

4    0    1    2

4   -1    1    2     9.000        -1

0    0    2    1   999.000        -1

1    0    2    2

0    1    2    2    19.000         1

1   -1    2    2     5.000        -1

1    1    2    2     5.000        -1

2    0    2    2     5.000        -1

0    2    2    2     5.000        -1

2   -1    2    2

1   -2    2    2    22.000         1

2    1    2    2

1    2    2    2    23.000         1

2   -2    2    2     6.000        -1

3    0    2    2

3   -1    2    2

2    2    2    2    10.000         1

0    3    2    2     6.000        -1

1   -3    2    2     6.000        -1

3    1    2    2     6.000        -1

1    3    2    2     6.000        -1

3   -2    2    2

2   -3    2    2    25.000         1

3    2    2    2     9.000        -1

2    3    2    2     9.000        -1

4    0    2    2

4   -1    2    2    10.000         1

1    0    3    2

0    1    3    2    18.000         1

1   -1    3    2    13.000         1

1    1    3    2    13.000         1

2    0    3    2

0    2    3    2    27.000         1

2   -1    3    2

1   -2    3    2    23.000         1

2    1    3    2

1    2    3    2    14.000         1

2   -2    3    2     6.000        -1

3    0    3    2

3   -1    3    2

2    2    3    2    24.000         1

0    3    3    2

1   -3    3    2    22.000         1

3    1    3    2    14.000         1

0    0    4    1

0    1    4    2

1    0    4    2   999.000        -1

1   -1    4    2

1    1    4    2     8.000         1

2    0    4    2

0    2    4    2

2   -1    4    2

1   -2    4    2    28.000         1

2    1    4    2     9.000        -1

1    2    4    2     9.000        -1

2   -2    4    2     9.000        -1

3    0    4    2     9.000        -1

3   -1    4    2

2    2    4    2    18.000         1

99999                                                                   22.terminator

100    1                                                              23.reject fac. & select

0.045                                                                 24.overall elastic weight

500   40                                                              26.contact opt.

1    1    0    0                                                    27.source unit spec.

99999                                                                   28.terminator

1    1    0    1    0    0    0    1         1                      29.target unit spec.

1    1   -5    5    1    0    0    1         1

1    1   -5    5    0    1    0    1         1

2    2   -5    5    0    0    0    1         1

2    2   -5    5   -1    0    0    1         1

2    2   -5    5    0   -1    0    1         1

2    2   -5    5   -1   -1    0    1         1

99999                                                                   30.terminator

O4A*      O4A     -1                                               31.atom redundancy

O5A*      O5A      1

C1A*      C1A      1

C3A.      C3A

C4A.      C4A

C5A.      C5A

99999                                                                   32.terminator

99999                                                                   34.terminator

99999                                                                   36.terminator

99999                                                                   38.terminator

2.50      3.10      2.90      5.00    120.00     20.00            39.attractive bond spec

O6A       O2A       O3A                 O6A       O2A              40.attractive bond donors and accepters

O3A       O5A       O4A

99999                                                                   41.terminator

2.60                                                              42.contact limit table

3.10      3.60

2.92      3.42      3.24

99999                                                                   43.terminator

6.0                                                                     44.ovalall contact weights

1.04                                                              45.indiv. contact weights

0.99      1.03

1.14      1.17      1.33

CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.               46.cycle control,

4    5    8    9   10   12   14   16   17

-90.0    -120.0     -60.0    -177.0      62.4     -62.8

57.5     -53.2      52.2

.1        .1        .1       .63      0.69       .92

.63       .92       .92

4    5    9   10   12   14   16   17

116.50    108.60    110.16    112.27    109.34    110.53

110.30    110.17

2.5       2.8       5.9      10.0       8.3       5.9

5.1       5.1

5    6    7    8   13    1    9   16    2

1    2    3    4    5    6    7    8    9   10   11   12

13   14   15   16   17   18

20.0       1.0       0.5        5.     0.001

ZZ.

CL=12,EH=1,EO=5,XR,CD=1,CS,CN,SH=1,FI,OP.

8

-60

.1

1    2    9   13   16

400       400       400       400       400

.1        .1       5.1       5.1       5.1

20.0       1.0       1.0        0.     0.001

CL=1,CR=2.

ZZ.

original from cellulose2

CL=3,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

4    5    8    9   10   12   14   16   17

-90.0    -120.0     -60.0    -177.0      62.4     -62.8

57.5     -53.2      52.2

.1        .1        .1       .63      0.69       .92

.63       .92       .92

4    5    9   10   12   14   16   17

116.50    108.60    110.16    112.27    109.34    110.53

110.30    110.17

2.5       2.8       5.9      10.0       8.3       5.9

5.1       5.1

5    6    7    8   13    1    9   16    2

1    2    3    4    5    6    7    8    9   10   11   12

13   14   15   16   17   18

20.0       1.0       0.5        5.     0.001

ZZ.

#EOR

b) Output 1 of Input file 1: WinLALS.prn

OO   Win     OO            * * * * * * * * * * * * * * * * * *   OO   Win     OO

OO           OO            *                                 *   OO           OO

OO   OOOOOO  OO   OOOOOO   *   Linked Atom Least Squares     *   OO   OOOOOO  OO   OOOOOO

OO   OO  OO  OO   OO  OO   *           (LALS)                *   OO   OO  OO  OO   OO  OO

OO       OO  OO   OO       *   Structure Refinement System   *   OO       OO  OO   OO

OO   OOOOOO  OO   OOOOOO   *      for Helical Polymers       *   OO   OOOOOO  OO   OOOOOO

OO   OO  OO  OO       OO   *      - WinLALS ver. 1.006-      *   OO   OO  OO  OO       OO

OO   OO  OO  OO   OO  OO   *                                 *   OO   OO  OO  OO   OO  OO

OOOO OOOOOOO OOOO OOOOOO   * * * * * * * * * * * * * * * * * *   OOOO OOOOOOO OOOO OOOOOO

-Blackwell valonia cellulose  PARALLEL down P21   April 1997 Finkenstadt

Run of 2001-11- 6 (14:39:17)

---------------------------------------------------------------------------------------------------

-===  Main Data Input  ===

-Windows Trig Version of 2001/7/10.

-Helix Properties

Single, RIGHT-Handed,  2 Residues in  1 Turns

-Geometry and Atomic Properties of Structure  1

Title: Z-matrix (Internal coordinates) of WinLALS

Atom   Precursor    Type   Occupancy     B        Tau      Theta

1(O5A*)                3      0.500

2(C1A*)   1(O5A*)      2      0.500     1.4290

3(O4A )   2(C1A*)      3      0.500     1.3890    107.30

4(C4A )   3(O4A )      2      0.500     1.4260    115.91    -95.92

5(C5A )   4(C4A )      2      0.500     1.5250    107.84   -143.77

6(C3A.)   4(C4A )      2      0.500     1.5230    110.40     95.34

7(C6A )   5(C5A )      2      1.000     1.5140    112.70     64.29

8(O6A )   7(C6A )      3      1.000     1.4270    111.80    -75.83

9(O5A )   5(C5A )      3      0.500     1.4360    110.09   -176.42

10(C1A )   9(O5A )      2      0.500     1.4290    112.82     60.60

11(O4A*)  10(C1A )      3      0.500     1.3890    107.30   -179.28

12(C2A )  10(C1A )      2      1.000     1.5230    109.59    -61.92

13(O2A )  12(C2A )      3      1.000     1.4230    109.30   -179.47

14(C3A )  12(C2A )      2      0.500     1.5210    110.03     58.32

15(O3A )  14(C3A )      3      1.000     1.4290    109.60   -175.76

16(C4A.)  14(C3A )      2      0.500     1.5230    109.52    -54.88

17(C5A.)  16(C4A.)      2      0.500     1.5250    110.56     53.65

18(H1A )  10(C1A )      1      1.000     1.1000    109.48     58.05

19(H2A )  12(C2A )      1      1.000     1.1000    109.48    -62.35

20(H3A )  14(C3A )      1      1.000     1.1000    109.48     65.66

21(H4A )   4(C4A )      1      1.000     1.1000    109.48    -24.28

22(H5A )   5(C5A )      1      1.000     1.1000    109.48    -55.98

23(H61A)   7(C6A )      1      1.000     1.1000    109.48     46.61

24(H62A)   7(C6A )      1      1.000     1.1000    109.48    161.73

Root Atom  Epsln-X   Epsln-Y   Epsln-Z      S

9(O5A )   120.31     60.33    -81.89    0.9858

Other ( 5)      ( 6)      ( 7)      ( 8)

Epsilons are applied in the order X, Y, Z

(INPUT  0)  Page   2

-Equivalenced Thetas

6(C3A.) to   5(C5A )           7(C6A ) to   9(O5A )          11(O4A*) to  12(C2A )

13(O2A ) to  14(C3A )          15(O3A ) to  16(C4A.)          18(H1A ) to  12(C2A )

19(H2A ) to  14(C3A )          20(H3A ) to  16(C4A.)          21(H4A ) to   5(C5A )

22(H5A ) to   9(O5A )          23(H61A) to   8(O6A )          24(H62A) to   8(O6A )

-X-ray Data-Fitting Refinables

Bragg      Scale (K) = Other(1)    2.732

Atten (B) = Other(2)    4.992

Continuous Scale (K) = Other(3)    0.000

Atten (B) = Other(4)    0.000

Water Density = 0.2984 *    1.00 el/A. cube

-Generation Details for Molecule Positions

Posn       Mu       U        V        W    Occupancy  Sense

1       -32.26  0.0000   0.0000   0.0000   1.000     UP

Other (  9)  ( 10)    ( 11)    ( 12)

2       -29.02  0.5000   0.5000  -0.2559   1.000     UP

Other ( 13)  ( 14)    ( 15)    ( 16)

-Identity Equivalent Position Generated

-Molecule Definitions

Molecule  Position  Equvlnce  Strcture   Helix

1         1         1         1       YES

2         2         1         1       YES

-Constraint and Restraint Specifications

Number   Type         Details

TRA           0.0000    0.0000    1.0000

1-  3   COI           0.0000

1(O5A*)    0    1   9(O5A )    1    1

4-  6   COI           0.0000

2(C1A*)    0    1  10(C1A )    1    1

7-  9   COI           0.0000

3(O4A )    0    1  11(O4A*)    1    1

10- 12   COI           0.0000

14(C3A )    0    1   6(C3A.)    0    1

13- 15   COI           0.0000

4(C4A )    0    1  16(C4A.)    0    1

16- 18   COI           0.0000

5(C5A )    0    1  17(C5A.)    0    1

19- 19   PAR      2    0.0000    0.0000

OTHR(  9)   -1.0000

OTHR( 13)    1.0000

(INPUT  0)  Page   3

-Unit Cell Parameters

A         B         C         Alpha     Beta      Gamma

8.1700    7.8600   10.3800     90.00     90.00     97.00

-Cruickshank Weighting Parameters (Bragg)

A         B         C

4.80E+01  0.00E+00  0.00E+00

Reciprocal Weight for Continuous X-ray Data is  0.000

-Division Factor for Structure Factor Stats is 2.00

-Scattering Factor Codes

1=    1   2=    2   3=    4

-Indices and Observed Structure Factor Amplitudes

Spot   Reflection    H    K    L    M   F-obs    Weight

1         1       0    1    0    2    3.00    -1.000

2         2       1    0    0    2    3.00    -1.000

3         3       1   -1    0    2   39.00     1.000

4         4       1    1    0    2   27.00     1.000

5         5       2    0    0    2    5.00    -1.000

6         6       0    2    0    2

6         7       2   -1    0    2  100.00     1.000

7         8       1   -2    0    2    3.00    -1.000

8         9       2    1    0    2    3.00    -1.000

9        10       1    2    0    2    3.00    -1.000

10        11       2   -2    0    2    3.00    -1.000

11        12       3    0    0    2    3.00    -1.000

12        13       3   -1    0    2

12        14       2    2    0    2   14.00     1.000

13        15       0    3    0    2

13        16       1   -3    0    2   15.00     1.000

14        17       3    1    0    2   12.00     1.000

15        18       1    3    0    2

15        19       3   -2    0    2   11.00     1.000

16        20       2   -3    0    2    4.00    -1.000

17        21       3    2    0    2    4.00    -1.000

18        22       2    3    0    2    4.00    -1.000

19        23       4    0    0    2

19        24       4   -1    0    2   14.00     1.000

20        25       3   -3    0    2    9.00    -1.000

21        26       1   -4    0    2

21        27       0    4    0    2   36.00     1.000

22        28       4    1    0    2    9.00    -1.000

23        29       4   -2    0    2    9.00    -1.000

24        30       1    0    1    2

24        31       0    1    1    2    8.00     1.000

(INPUT  0)  Page   4

25        32       1   -1    1    2    5.00    -1.000

26        33       1    1    1    2    6.00     1.000

27        34       2    0    1    2    7.00     1.000

28        35       0    2    1    2    5.00    -1.000

29        36       2   -1    1    2

29        37       1   -2    1    2   11.00     1.000

30        38       2    1    1    2    5.00    -1.000

31        39       1    2    1    2    5.00    -1.000

32        40       2   -2    1    2    5.00    -1.000

33        41       3    0    1    2   25.00     1.000

34        42       3   -1    1    2

34        43       2    2    1    2   23.00     1.000

35        44       0   -3    1    2

35        45      -1    3    1    2   24.00     1.000

36        46       3    1    1    2   15.00     1.000

37        47       1    3    1    2    7.00     1.000

38        48       3   -2    1    2

38        49       2   -3    1    2   18.00     1.000

39        50       3    2    1    2    9.00    -1.000

40        51       2    3    1    2    9.00    -1.000

41        52       4    0    1    2

41        53       4   -1    1    2    9.00    -1.000

42        54       0    0    2    1  999.00    -1.000

43        55       1    0    2    2

43        56       0    1    2    2   19.00     1.000

44        57       1   -1    2    2    5.00    -1.000

45        58       1    1    2    2    5.00    -1.000

46        59       2    0    2    2    5.00    -1.000

47        60       0    2    2    2    5.00    -1.000

48        61       2   -1    2    2

48        62       1   -2    2    2   22.00     1.000

49        63       2    1    2    2

49        64       1    2    2    2   23.00     1.000

50        65       2   -2    2    2    6.00    -1.000

51        66       3    0    2    2

51        67       3   -1    2    2

51        68       2    2    2    2   10.00     1.000

52        69       0    3    2    2    6.00    -1.000

53        70       1   -3    2    2    6.00    -1.000

54        71       3    1    2    2    6.00    -1.000

55        72       1    3    2    2    6.00    -1.000

56        73       3   -2    2    2

56        74       2   -3    2    2   25.00     1.000

57        75       3    2    2    2    9.00    -1.000

58        76       2    3    2    2    9.00    -1.000

59        77       4    0    2    2

59        78       4   -1    2    2   10.00     1.000

60        79       1    0    3    2

60        80       0    1    3    2   18.00     1.000

61        81       1   -1    3    2   13.00     1.000

62        82       1    1    3    2   13.00     1.000

63        83       2    0    3    2

63        84       0    2    3    2   27.00     1.000

64        85       2   -1    3    2

64        86       1   -2    3    2   23.00     1.000

65        87       2    1    3    2

(INPUT  0)  Page   5

65        88       1    2    3    2   14.00     1.000

66        89       2   -2    3    2    6.00    -1.000

67        90       3    0    3    2

67        91       3   -1    3    2

67        92       2    2    3    2   24.00     1.000

68        93       0    3    3    2

68        94       1   -3    3    2   22.00     1.000

69        95       3    1    3    2   14.00     1.000

70        96       0    0    4    1

70        97       0    1    4    2

70        98       1    0    4    2  999.00    -1.000

71        99       1   -1    4    2

71       100       1    1    4    2    8.00     1.000

72       101       2    0    4    2

72       102       0    2    4    2

72       103       2   -1    4    2

72       104       1   -2    4    2   28.00     1.000

73       105       2    1    4    2    9.00    -1.000

74       106       1    2    4    2    9.00    -1.000

75       107       2   -2    4    2    9.00    -1.000

76       108       3    0    4    2    9.00    -1.000

77       109       3   -1    4    2

77       110       2    2    4    2   18.00     1.000

-Rejection Factor for All Spots is   100, for Unobserved Spots is     1

-Relative Weighting of Elastic Data is  4.500E-02

-Short Contact Calculation Data

-Contact List Length  500, Min Diff for Printing   40 Hundredths

-Source Unit Specifications

From       To       From       To

Molecule  Molecule   Residue   Residue   S-Code

1         1         0         0         1

-Target Unit Specifications

From       To       From       To         Units of Displacement      Multi-

Molecule  Molecule   Residue   Residue   Along A   Along B   Along C   Plicity   T-Code

1         1         0         1         0         0         0         1         1

1         1        -5         5         1         0         0         1         1

1         1        -5         5         0         1         0         1         1

2         2        -5         5         0         0         0         1         1

2         2        -5         5        -1         0         0         1         1

2         2        -5         5         0        -1         0         1         1

2         2        -5         5        -1        -1         0         1         1

(INPUT  0)  Page   6

-Atom Redundancy List

11(O4A*) is the same as   3(O4A ) with -1 Helix Ops

1(O5A*) is the same as   9(O5A ) with  1 Helix Ops

2(C1A*) is the same as  10(C1A ) with  1 Helix Ops

6(C3A.) is the same as  14(C3A )

16(C4A.) is the same as   4(C4A )

17(C5A.) is the same as   5(C5A )

-Attractive Interaction Parameters

Type   1 - R1 = 2.50  R2 = 3.10  D0 = 2.90  W =   5.00  Ang = 120.00  Dev =  20.00

Donor/Acceptor List      8(O6A )   13(O2A )   15(O3A )   -1(    )    8(O6A )   13(O2A )

Donor/Acceptor List     15(O3A )    9(O5A )    3(O4A )   -1(    )

-Contact Limit Table

1      2      3

1   2.60

2   3.10   3.60

3   2.92   3.42   3.24

-Overall Contact Distance Weight is 6.00E+00

-Contact Weight Table

1      2      3

1   1.04

2   0.99   1.03

3   1.14   1.17   1.33

(CYCLE  1)  Page   7

-                              -----  Start of Cycle  1  (used   1.0 Seconds)  -----

-===  Current Values of Parameters  ===

1         2         3         4         5         6         7         8         9        10

Theta      0.000     0.000     0.000   -95.923  -143.774    95.336    64.294   -75.829  -176.416    60.600

Tau        0.000     0.000   107.300   115.905   107.840   110.400   112.698   111.799   110.092   112.822

Other    2.73150   4.99180   0.00000   0.00000 120.31090  60.32940 -81.88690   0.98580 -32.25760   0.00000

SCLE( 0)  ATTN( 0)  SCLE( 1)  ATTN( 1)  EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 1)     U( 1)

11        12        13        14        15        16        17        18        19        20

Theta   -179.282   -61.922  -179.465    58.325  -175.759   -54.879    53.647    58.048   -62.345    65.661

Tau      107.300   109.590   109.300   110.031   109.602   109.519   110.565   109.480   109.480   109.480

Other    0.00000   0.00000 -29.02240   0.50000   0.50000  -0.25587

V( 1)     W( 1)    MU( 2)     U( 2)     V( 2)     W( 2)

21        22        23        24

Theta    -24.284   -55.976    46.611   161.731

Tau      109.480   109.480   109.480   109.480

-===  Cycle Control Data  ===

-Cycle Control Card -

--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

Elastically Bound Thetas

Theta  Bound    Wt       Theta  Bound    Wt       Theta  Bound    Wt       Theta  Bound    Wt

4   -90.00   0.10        5  -120.00   0.10        8   -60.00   0.10        9  -177.00   0.63

10    62.40   0.69       12   -62.80   0.92       14    57.50   0.63       16   -53.20   0.92

17    52.20   0.92

Elastically Bound Taus

Tau    Bound    Wt       Tau    Bound    Wt       Tau    Bound    Wt       Tau    Bound    Wt

4   116.50   2.50        5   108.60   2.80        9   110.16   5.90       10   112.27  10.00

12   109.34   8.30       14   110.53   5.90       16   110.30   5.10       17   110.17   5.10

Refined Other Parameters

5         6         7         8        13         1         9        16         2

EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 2)  SCLE( 0)    MU( 1)     W( 2)  ATTN( 0)

(CYCLE  1)  Page   8

Applied Constraints and Restraints

1    2    3    4    5    6    7    8    9   10   11   12

13   14   15   16   17   18

( 18 are Constraints,  0 are Restraints)

Max Angle Shift      20.0

Max Dist Shift       1.00

Damping Factor       0.50

Filter Number           5

Convergency Limit  0.0010

---  Run of  6 Cycles  ---

-===  Molecule Building  ===

-Equivalenced Theta Implementation

Thta(  6) Tied to Thta(  5), Difference  120.89

Thta( 21) Tied to Thta(  5), Difference -119.49

Thta( 23) Tied to Thta(  8), Difference -122.44

Thta( 24) Tied to Thta(  8), Difference  122.44

Thta(  7) Tied to Thta(  9), Difference  119.29

Thta( 22) Tied to Thta(  9), Difference -120.44

Thta( 11) Tied to Thta( 12), Difference  117.36

Thta( 18) Tied to Thta( 12), Difference -119.97

Thta( 13) Tied to Thta( 14), Difference -122.21

Thta( 19) Tied to Thta( 14), Difference  120.67

Thta( 15) Tied to Thta( 16), Difference  120.88

Thta( 20) Tied to Thta( 16), Difference -120.54

-Coordinates of Base Residue

Atom       X         Y         Z             R        Phi        Z

1(O5A*)  -0.9858    0.0000    5.1900        0.9858   -180.00    5.1900

2(C1A*)   0.3541    0.0677    4.6980        0.3606     10.83    4.6980

3(O4A )   0.3210    0.7802    3.5061        0.8436     67.63    3.5061

4(C4A )   0.2555   -0.0011    2.3150        0.2555     -0.24    2.3150

5(C5A )   1.0861    0.7037    1.2477        1.2942     32.94    1.2477

6(C3A.)  -1.1892   -0.1501    1.8566        1.1986   -172.81    1.8566

7(C6A )   2.5570    0.7686    1.6005        2.6700     16.73    1.6005

8(O6A )   2.8205    1.7580    2.5944        3.3235     31.94    2.5944

9(O5A )   0.9858    0.0000    0.0000        0.9858      0.00    0.0000

10(C1A )  -0.3541   -0.0678   -0.4920        0.3606   -169.17   -0.4920

11(O4A*)  -0.3210   -0.7802   -1.6839        0.8436   -112.36   -1.6839

12(C2A )  -1.2234   -0.8324    0.4976        1.4797   -145.77    0.4976

13(O2A )  -2.5530   -0.9068   -0.0040        2.7092   -160.44   -0.0040

14(C3A )  -1.1892   -0.1501    1.8566        1.1986   -172.81    1.8566

15(O3A )  -1.9266   -0.9276    2.8020        2.1383   -154.29    2.8020

16(C4A.)   0.2555   -0.0011    2.3150        0.2555     -0.24    2.3150

17(C5A.)   1.0861    0.7037    1.2477        1.2942     32.94    1.2477

18(H1A )  -0.7513    0.9505   -0.6159        1.2116    128.32   -0.6159

19(H2A )  -0.8529   -1.8639    0.5906        2.0498   -114.59    0.5906

20(H3A )  -1.6620    0.8403    1.7819        1.8624    153.18    1.7819

21(H4A )   0.6687   -1.0018    2.5094        1.2045    -56.28    2.5094

(CYCLE  1)  Page   9

22(H5A )   0.7209    1.7333    1.1192        1.8772     67.42    1.1192

23(H61A)   2.8948   -0.2189    1.9481        2.9030     -4.33    1.9481

24(H62A)   3.1431    0.9756    0.6930        3.2911     17.24    0.6930

-Coordinates of Molecule  1, Residue  0  -  Helix Axis at X =   0.0000, Y =   0.0000

C a r t e s i a n                F r a c t i o n a l              C y l i n d r i c a l

Atom       X         Y         Z             X         Y         Z             R        Phi        Z          Atom

1(O5A*)  -0.8336    0.5261    5.1900      -0.10280   0.05392   0.50000        0.9858    147.74    5.1900      1(O5A*)

2(C1A*)   0.3356   -0.1317    4.6980       0.04139  -0.01152   0.45260        0.3606    -21.43    4.6980      2(C1A*)

3(O4A )   0.6879    0.4884    3.5061       0.08483   0.07289   0.33778        0.8436     35.38    3.5061      3(O4A )

4(C4A )   0.2155   -0.1373    2.3150       0.02658  -0.01410   0.22302        0.2555    -32.50    2.3150      4(C4A )

5(C5A )   1.2941    0.0154    1.2477       0.15958   0.02218   0.12020        1.2942      0.68    1.2477      5(C5A )

6(C3A.)  -1.0858    0.5078    1.8566      -0.13390   0.04764   0.17886        1.1986    154.94    1.8566      6(C3A.)

7(C6A )   2.5726   -0.7148    1.6005       0.31724  -0.05076   0.15419        2.6700    -15.53    1.6005      7(C6A )

8(O6A )   3.3235   -0.0187    2.5944       0.40984   0.04954   0.24994        3.3235     -0.32    2.5944      8(O6A )

9(O5A )   0.8336   -0.5261    0.0000       0.10280  -0.05392   0.00000        0.9858    -32.26    0.0000      9(O5A )

10(C1A )  -0.3356    0.1317   -0.4920      -0.04139   0.01151  -0.04740        0.3606    158.57   -0.4920     10(C1A )

11(O4A*)  -0.6879   -0.4884   -1.6839      -0.08482  -0.07289  -0.16222        0.8436   -144.62   -1.6839     11(O4A*)

12(C2A )  -1.4788   -0.0509    0.4976      -0.18237  -0.02958   0.04794        1.4797   -178.03    0.4976     12(C2A )

13(O2A )  -2.6429    0.5957   -0.0040      -0.32592   0.03450  -0.00038        2.7092    167.30   -0.0040     13(O2A )

14(C3A )  -1.0858    0.5078    1.8566      -0.13390   0.04764   0.17886        1.1986    154.94    1.8566     14(C3A )

15(O3A )  -2.1243    0.2438    2.8020      -0.26197  -0.00216   0.26994        2.1383    173.45    2.8020     15(O3A )

16(C4A.)   0.2155   -0.1373    2.3150       0.02658  -0.01410   0.22302        0.2555    -32.50    2.3150     16(C4A.)

17(C5A.)   1.2941    0.0154    1.2477       0.15958   0.02218   0.12020        1.2942      0.68    1.2477     17(C5A.)

18(H1A )  -0.1280    1.2048   -0.6159      -0.01579   0.15128  -0.05934        1.2116     96.07   -0.6159     18(H1A )

19(H2A )  -1.7161   -1.1210    0.5906      -0.21163  -0.16943   0.05689        2.0498   -146.85    0.5906     19(H2A )

20(H3A )  -0.9571    1.5976    1.7819      -0.11802   0.18831   0.17166        1.8624    120.92    1.7819     20(H3A )

21(H4A )   0.0308   -1.2041    2.5094       0.00380  -0.15271   0.24176        1.2045    -88.53    2.5094     21(H4A )

22(H5A )   1.5347    1.0811    1.1192       0.18926   0.16151   0.10783        1.8772     35.16    1.1192     22(H5A )

23(H61A)   2.3311   -1.7301    1.9481       0.28747  -0.18371   0.18767        2.9030    -36.58    1.9481     23(H61A)

24(H62A)   3.1787   -0.8525    0.6930       0.39200  -0.05880   0.06676        3.2911    -15.01    0.6930     24(H62A)

-Coordinates of Molecule  2, Residue  0  -  Helix Axis at X =   4.0546, Y =   3.4322

C a r t e s i a n                F r a c t i o n a l              C y l i n d r i c a l

Atom       X         Y         Z             X         Y         Z             R        Phi        Z          Atom

1(O5A*)   3.1925    3.9104    2.5340       0.39370   0.54738   0.24413        0.9858    150.98    2.5340      1(O5A*)

2(C1A*)   4.3971    3.3196    2.0421       0.54224   0.49103   0.19673        0.3606    -18.19    2.0421      2(C1A*)

3(O4A )   4.7138    3.9586    0.8502       0.58129   0.57728   0.08190        0.8436     38.61    0.8502      3(O4A )

4(C4A )   4.2775    3.3073   -0.3410       0.52749   0.48759  -0.03285        0.2555    -29.26   -0.3410      4(C4A )

5(C5A )   5.3457    3.5206   -1.4082       0.65922   0.53142  -0.13567        1.2942      3.92   -1.4082      5(C5A )

6(C3A.)   2.9419    3.8778   -0.7994       0.36278   0.53932  -0.07701        1.1986    158.17   -0.7994      6(C3A.)

7(C6A )   6.6634    2.8637   -1.0555       0.82171   0.46843  -0.10168        2.6700    -12.29   -1.0555      7(C6A )

8(O6A )   7.3738    3.6011   -0.0616       0.90932   0.57334  -0.00593        3.3235      2.91   -0.0616      8(O6A )

9(O5A )   4.9166    2.9539   -2.6560       0.60630   0.45262  -0.25587        0.9858    -29.02   -2.6560      9(O5A )

10(C1A )   3.7120    3.5447   -3.1479       0.45776   0.50897  -0.30327        0.3606    161.81   -3.1479     10(C1A )

11(O4A*)   3.3954    2.9057   -4.3398       0.41871   0.42272  -0.41810        0.8436   -141.39   -4.3398     11(O4A*)

12(C2A )   2.5810    3.2978   -2.1583       0.31828   0.45989  -0.20793        1.4797   -174.79   -2.1583     12(C2A )

13(O2A )   1.3822    3.8778   -2.6599       0.17045   0.51495  -0.25626        2.7092    170.53   -2.6599     13(O2A )

14(C3A )   2.9418    3.8778   -0.7994       0.36278   0.53932  -0.07701        1.1986    158.17   -0.7994     14(C3A )

15(O3A )   1.9199    3.5557    0.1461       0.23675   0.48237   0.01407        2.1383    176.69    0.1461     15(O3A )

16(C4A.)   4.2775    3.3073   -0.3410       0.52749   0.48759  -0.03285        0.2555    -29.26   -0.3410     16(C4A.)

17(C5A.)   5.3457    3.5206   -1.4082       0.65922   0.53142  -0.13567        1.2942      3.92   -1.4082     17(C5A.)

18(H1A )   3.8587    4.6278   -3.2719       0.47585   0.64906  -0.31521        1.2116     99.30   -3.2719     18(H1A )

19(H2A )   2.4045    2.2161   -2.0654       0.29651   0.31951  -0.19898        2.0498   -143.61   -2.0654     19(H2A )

(CYCLE  1)  Page  10

20(H3A )   3.0089    4.9732   -0.8741       0.37105   0.67973  -0.08421        1.8624    124.16   -0.8741     20(H3A )

21(H4A )   4.1533    2.2317   -0.1465       0.51217   0.34882  -0.01411        1.2045    -85.30   -0.1465     21(H4A )

22(H5A )   5.5258    4.5981   -1.5367       0.68143   0.67132  -0.14805        1.8772     38.40   -1.5367     22(H5A )

23(H61A)   6.4796    1.8363   -0.7079       0.79905   0.33485  -0.06820        2.9030    -33.35   -0.7079     23(H61A)

24(H62A)   7.2763    2.7604   -1.9630       0.89730   0.46486  -0.18911        3.2911    -11.78   -1.9630     24(H62A)

-===  Constraints and Restraints  ===

No   Type  Applied  Value   Weight    W*D**2    Want    Have    Atom    TFM Mol    X        Y        Z

TFM  1 Defined,     0.0*A    0.0*B    1.0*H

1  COI CON  YES   0.0000    0.000  0.00E+00 X                  1(O5A*)   0   1  -0.8336   0.5261   5.1900

2  COI CON  YES   0.0000    0.000  0.00E+00 Y                  9(O5A )   1   1  -0.8336   0.5261   5.1900

3  COI CON  YES   0.0000    0.000  0.00E+00 Z

4  COI CON  YES   0.0000    0.000  0.00E+00 X                  2(C1A*)   0   1   0.3356  -0.1317   4.6980

5  COI CON  YES   0.0000    0.000  0.00E+00 Y                 10(C1A )   1   1   0.3356  -0.1317   4.6980

6  COI CON  YES   0.0000    0.000  0.00E+00 Z

7  COI CON  YES   0.0000    0.000  0.00E+00 X                  3(O4A )   0   1   0.6879   0.4884   3.5061

8  COI CON  YES   0.0000    0.000  0.00E+00 Y                 11(O4A*)   1   1   0.6879   0.4884   3.5061

9  COI CON  YES   0.0000    0.000  0.00E+00 Z

10  COI CON  YES   0.0000    0.000  0.00E+00 X                 14(C3A )   0   1  -1.0858   0.5078   1.8566

11  COI CON  YES   0.0000    0.000  0.00E+00 Y                  6(C3A.)   0   1  -1.0858   0.5078   1.8566

12  COI CON  YES   0.0000    0.000  0.00E+00 Z

13  COI CON  YES   0.0000    0.000  0.00E+00 X                  4(C4A )   0   1   0.2155  -0.1373   2.3150

14  COI CON  YES   0.0000    0.000  0.00E+00 Y                 16(C4A.)   0   1   0.2155  -0.1373   2.3150

15  COI CON  YES   0.0000    0.000  0.00E+00 Z

16  COI CON  YES   0.0000    0.000  0.00E+00 X                  5(C5A )   0   1   1.2941   0.0154   1.2477

17  COI CON  YES   0.0000    0.000  0.00E+00 Y                 17(C5A.)   0   1   1.2941   0.0154   1.2477

18  COI CON  YES   0.0000    0.000  0.00E+00 Z

19  PAR CON  NO    0.0565    0.000  0.00E+00    0.00    3.24  OTHR(  9)   OTHR( 13)

* -1.0000   *  1.0000

Sum of Moduli of Lagrange Constraints - All   0.0566, Applied   0.0001

-Elastically Bound Parameters

Parameter            Current    Bound      Diff     W*D**2

THTA(  4)            -95.923   -90.000     5.923   1.58E-01

THTA(  5)           -143.774  -120.000    23.774   2.54E+00

THTA(  8)            -75.829   -60.000    15.829   1.13E+00

THTA(  9)           -176.416  -177.000    -0.584   9.66E-03

THTA( 10)             60.600    62.400     1.800   1.01E-01

THTA( 12)            -61.922   -62.800    -0.878   3.19E-02

THTA( 14)             58.325    57.500    -0.825   1.93E-02

THTA( 16)            -54.879   -53.200     1.679   1.17E-01

THTA( 17)             53.647    52.200    -1.447   8.66E-02

TAU(  4)            115.905   116.500     0.595   3.98E-02

TAU(  5)            107.840   108.600     0.760   7.28E-02

TAU(  9)            110.092   110.160     0.068   1.22E-03

(CYCLE  1)  Page  11

TAU( 10)            112.822   112.270    -0.552   1.37E-01

TAU( 12)            109.590   109.340    -0.250   2.33E-02

TAU( 14)            110.031   110.530     0.499   6.62E-02

TAU( 16)            109.519   110.300     0.781   1.40E-01

TAU( 17)            110.565   110.170    -0.395   3.58E-02

CNTCTS91:IFK(44)= 1, IFK(51)= 0

-===  Contact Search Setup ===

-Source Molecules Generated

Source   Original    From       To

Mol No    Mol No    Residue   Residue   S-Code

S    1         1         0         0         1

-Target Molecules Generated

Target   Original    From       To         Units of Displacement      Multi-    Combines With

Mol No    Mol No    Residue   Residue   Along A   Along B   Along C   Plicity   Source Mols ...

T    1         1         0         1         0         0         0         1       1

T    2         1        -5         5         1         0         0         1       1

T    3         1        -5         5         0         1         0         1       1

T    4         2        -5         5         0         0         0         1       1

T    5         2        -5         5        -1         0         0         1       1

T    6         2        -5         5         0        -1         0         1       1

T    7         2        -5         5        -1        -1         0         1       1

-===  Contact List Creation  ===

--------  S o u r c e  ---------  --------  T a r g e t  ---------   Axis   Reject  Multi-  Rela-     Contacts     Min

Mol  Res    Z-Low   Z-High  Scan  Mol  Res    Z-Low   Z-High  Scan  Separn  Reason Plicity  Tion   Tested  Found   Dis

S 1   0   -0.6159   3.5061   UP   T  1  0   -0.6159   3.5061   UP     0.00             1    SAME     169     34    2.41

S 1   0   -0.6159   3.5061   UP   T  1  1    4.5741   8.6961   UP     0.00             1              72     12    2.07

S 1   0   -0.6159   3.5061   UP   T  2  0   -0.6159   3.5061   UP     8.17             1             322      3    2.43

S 1   0   -0.6159   3.5061   UP   T  4  0   -3.2719   0.8502   UP     5.31             1             228      1    2.69

S 1   0   -0.6159   3.5061   UP   T  4  1    1.9181   6.0402   UP     5.31             1             248      4    2.53

S 1   0   -0.6159   3.5061   UP   T  5  0   -3.2719   0.8502   UP     6.00             1             228      4    2.22

S 1   0   -0.6159   3.5061   UP   T  5  1    1.9181   6.0402   UP     6.00             1             248      1    2.54

S 1   0   -0.6159   3.5061   UP   T  6  0   -3.2719   0.8502   UP     6.00             1             228      1    2.57

S 1   0   -0.6159   3.5061   UP   T  6  1    1.9181   6.0402   UP     6.00             1             248      1    2.45

S 1   0   -0.6159   3.5061   UP   T  7  0   -3.2719   0.8502   UP     5.31             1             228      5    2.44

S 1   0   -0.6159   3.5061   UP   T  7  1    1.9181   6.0402   UP     5.31             1             248      1    2.50

..Residue Pairs..   ...Atom Pairs...    Bonds     Common    Coinci-  Contacts

Total    Searched   Total    Searched   Found     Bonds     Dences    Found

68        16     22032      3055        19        38        18        67

-===  Contact List Application  ===

--------------  S o u r c e  ---------------  --------------  T a r g e t  ---------------

Mol  Res  Atom       X        Y        Z      Mol  Res  Atom       X        Y        Z       Dist Limit  Diff Mul W*D**2

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  0   7(C6A )   2.5726  -0.7148   1.6005   2.94  3.42  0.48  1 1.63E+00

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  0   8(O6A )   3.3235  -0.0187   2.5944   2.83  3.24  0.41  1 1.31E+00

ADI Type  1, Donor on TRGT, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is   79.75

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  1  13(O2A )   2.6429  -0.5957   5.1860   2.80  3.24  0.44  1 1.57E+00

(CYCLE  1)  Page  12

ADI Type  1, Donor on TRGT, Prcrsr is  12(C2A ) at    1.4788   0.0509   5.6876, Angle is   57.56

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  0  10(C1A )  -0.3356   0.1317  -0.4920   2.87  3.60  0.73  1 3.27E+00

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  0  23(H61A)   2.3311  -1.7301   1.9481   2.67  3.10  0.43  1 1.08E+00

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  1  18(H1A )   0.1280  -1.2048   4.5741   2.50  3.10  0.60  1 2.14E+00

S 1   0   5(C5A )   1.2941   0.0154   1.2477  T  1  0  12(C2A )  -1.4788  -0.0509   0.4976   2.87  3.60  0.73  1 3.26E+00

S 1   0   5(C5A )   1.2941   0.0154   1.2477  T  1  0  18(H1A )  -0.1280   1.2048  -0.6159   2.63  3.10  0.47  1 1.32E+00

S 1   0   8(O6A )   3.3235  -0.0187   2.5944  T  1  1  13(O2A )   2.6429  -0.5957   5.1860   2.74  2.90+ 0.16  1 7.59E-01

ADI Type  1, Donor on SRCE, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is  115.16

S 1   0   8(O6A )   3.3235  -0.0187   2.5944  T  2  0  15(O3A )   5.9848  -0.7518   2.8020   2.77  2.90+ 0.13  1 5.21E-01

ADI Type  1, Donor on SRCE, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is  115.40

S 1   0  14(C3A )  -1.0858   0.5078   1.8566  T  1  0   9(O5A )   0.8336  -0.5261   0.0000   2.86  3.42  0.56  1 2.17E+00

S 1   0  15(O3A )  -2.1243   0.2438   2.8020  T  1  1   9(O5A )  -0.8336   0.5261   5.1900   2.73  2.90+ 0.17  1 8.76E-01

ADI Type  1, Donor on SRCE, Prcrsr is  14(C3A ) at   -1.0858   0.5078   1.8566, Angle is  102.48

S 1   0  15(O3A )  -2.1243   0.2438   2.8020  T  7  1  19(H2A )  -2.4045  -2.2161   3.1246   2.50  2.92  0.42  1 1.23E+00

S 1   0  19(H2A )  -1.7161  -1.1210   0.5906  T  7  0   3(O4A )  -3.3953  -2.9057   0.8502   2.46  2.92  0.46  1 1.42E+00

S 1   0  19(H2A )  -1.7161  -1.1210   0.5906  T  7  0   8(O6A )  -0.7353  -3.2632  -0.0616   2.44  2.92  0.48  1 1.55E+00

S 1   0  21(H4A )   0.0308  -1.2041   2.5094  T  1  1  10(C1A )   0.3356  -0.1317   4.6980   2.46  3.10  0.64  1 2.46E+00

S 1   0  21(H4A )   0.0308  -1.2041   2.5094  T  1  1  18(H1A )   0.1280  -1.2048   4.5741   2.07  2.60  0.53  1 1.77E+00

S 1   0  22(H5A )   1.5347   1.0811   1.1192  T  1  0  10(C1A )  -0.3356   0.1317  -0.4920   2.64  3.10  0.46  1 1.23E+00

S 1   0  23(H61A)   2.3311  -1.7301   1.9481  T  6  1   9(O5A )   3.1925  -3.9496   2.5340   2.45  2.92  0.47  1 1.50E+00

S 1   0  24(H62A)   3.1787  -0.8525   0.6930  T  1  0   9(O5A )   0.8336  -0.5261   0.0000   2.47  2.92  0.45  1 1.40E+00

S 1   0  24(H62A)   3.1787  -0.8525   0.6930  T  2  0  13(O2A )   5.4662  -0.4000  -0.0040   2.43  2.92  0.49  1 1.62E+00

Contacts   Invar-   Contacts  Shortest   Worst

In List    Iants    Applied   Contact     Diff

67         0        67      2.07      0.73

-===  X-ray Data Application ===

Scattering Types

Atom   Core H  W        Atom   Core H  W        Atom   Core H  W        Atom   Core H  W

1(O5A*)  O              7(C6A )  C             13(O2A )  O             19(H2A )  H

2(C1A*)  C              8(O6A )  O             14(C3A )  C             20(H3A )  H

3(O4A )  O              9(O5A )  O             15(O3A )  O             21(H4A )  H

4(C4A )  C             10(C1A )  C             16(C4A.)  C             22(H5A )  H

5(C5A )  C             11(O4A*)  O             17(C5A.)  C             23(H61A)  H

6(C3A.)  C             12(C2A )  C             18(H1A )  H             24(H62A)  H

Structure Factor Amplitudes - Trig

Spot Refl     H   K   L        A       B     Mod     Phase  Multy  F-calc   F-obs    Diff     Weight     W*D**2   Flag

1    1     0   1   0     0.22    0.00    0.31      0.00     2     0.30    8.19    7.89   2.08E-02   1.74E-01   UR

2    2     1   0   0     0.97    0.00    1.37      0.00     2     1.34    8.19    6.85   2.08E-02   1.31E-01   UR

3    3     1  -1   0    76.99    0.00  108.88      0.00     2   105.17  106.53    1.36   2.08E-02   5.13E-03

4    4     1   1   0    71.63    0.00  101.29      0.00     2    96.91   73.75  -23.16   2.08E-02   1.50E+00

5    5     2   0   0   -11.54    0.00   16.33   -180.00     2    15.13   13.66   -1.48   2.08E-02   6.08E-03   U

6    6     0   2   0   205.78    0.00  291.01      0.00     2

7     2  -1   0     1.82    0.00    2.58      0.00     2   266.83  273.15    6.32   2.08E-02   1.12E-01

(CYCLE  1)  Page  13

7    8     1  -2   0   -11.64    0.00   16.46    180.00     2    15.02    8.19   -6.83   2.08E-02   1.30E-01   U

8    9     2   1   0    -1.94    0.00    2.75    180.00     2     2.47    8.19    5.72   2.08E-02   9.14E-02   UR

9   10     1   2   0    10.15    0.00   14.36      0.00     2    12.85    8.19   -4.66   2.08E-02   6.06E-02   U

10   11     2  -2   0   -11.49    0.00   16.25    180.00     2    14.15    8.19   -5.95   2.08E-02   9.89E-02   U

11   12     3   0   0     0.12    0.00    0.17      0.00     2     0.15    8.19    8.05   2.08E-02   1.81E-01   UR

12   13     3  -1   0    25.96    0.00   36.71      0.00     2

14     2   2   0   -13.54    0.00   19.15   -180.00     2    34.68   38.24    3.56   2.08E-02   3.54E-02

13   15     0   3   0     1.56    0.00    2.21      0.00     2

16     1  -3   0    28.68    0.00   40.56      0.00     2    33.62   40.97    7.35   2.08E-02   1.51E-01

14   17     3   1   0    16.00    0.00   22.63      0.00     2    18.42   32.78   14.36   2.08E-02   5.76E-01

15   18     1   3   0    25.21    0.00   35.65      0.00     2

19     3  -2   0    -3.06    0.00    4.33   -180.00     2    28.71   30.05    1.34   2.08E-02   4.99E-03

16   20     2  -3   0     6.09    0.00    8.61      0.00     2     6.83   10.93    4.09   2.08E-02   4.68E-02   UR

17   21     3   2   0    -0.65    0.00    0.91   -180.00     2     0.69   10.93   10.24   2.08E-02   2.93E-01   UR

18   22     2   3   0    -1.11    0.00    1.58    180.00     2     1.18   10.93    9.75   2.08E-02   2.65E-01   UR

19   23     4   0   0   -34.00    0.00   48.08   -180.00     2

24     4  -1   0     0.52    0.00    0.74      0.00     2    35.45   38.24    2.79   2.08E-02   2.18E-02

20   25     3  -3   0    29.33    0.00   41.48      0.00     2    30.36   24.58   -5.78   2.08E-02   9.32E-02   U

21   26     1  -4   0   -12.26    0.00   17.34    180.00     2

27     0   4   0    77.84    0.00  110.08      0.00     2    80.27   98.33   18.07   2.08E-02   9.12E-01

22   28     4   1   0     2.87    0.00    4.05      0.00     2     2.87   24.58   21.71   2.08E-02   1.32E+00   UR

23   29     4  -2   0   -47.60    0.00   67.32   -180.00     2    47.57   24.58  -22.99   2.08E-02   1.48E+00   U

24   30     1   0   1     0.27  -16.81   23.78    -89.08     2

31     0   1   1    -1.52  -15.15   21.53    -95.71     2    31.07   21.85   -9.22   2.08E-02   2.37E-01

25   32     1  -1   1    -7.05    0.04    9.98    179.68     2     9.53   13.66    4.13   2.08E-02   4.77E-02   UR

26   33     1   1   1   -19.49   -4.85   28.41   -166.04     2    26.86   16.39  -10.47   2.08E-02   3.06E-01

27   34     2   0   1    10.87   -5.29   17.10    -25.94     2    15.66   19.12    3.46   2.08E-02   3.34E-02

28   35     0   2   1   -23.59    6.70   34.69    164.15     2    31.59   13.66  -17.93   2.08E-02   8.98E-01   U

29   36     2  -1   1     6.70   16.88   25.69     68.35     2

37     1  -2   1     1.41    3.21    4.96     66.32     2    23.60   30.05    6.44   2.08E-02   1.16E-01

30   38     2   1   1    -1.63    4.53    6.81    109.84     2     6.05   13.66    7.61   2.08E-02   1.62E-01   UR

(CYCLE  1)  Page  14

31   39     1   2   1     7.69  -16.67   25.96    -65.25     2    22.98   13.66   -9.32   2.08E-02   2.43E-01   U

32   40     2  -2   1    18.42   -5.49   27.18    -16.59     2    23.39   13.66   -9.73   2.08E-02   2.64E-01   U

33   41     3   0   1   -33.30  -32.98   66.28   -135.28     2    55.23   68.29   13.06   2.08E-02   4.76E-01

34   42     3  -1   1   -38.15   35.84   74.02    136.79     2

43     2   2   1     2.28    8.26   12.11     74.59     2    62.11   62.82    0.72   2.08E-02   1.44E-03

35   44     0  -3   1     4.23   27.13   38.83     81.13     2

45    -1   3   1   -12.20    1.01   17.31    175.25     2    34.78   65.56   30.77   2.08E-02   2.64E+00

36   46     3   1   1   -20.41   27.93   48.93    126.16     2    39.37   40.97    1.60   2.08E-02   7.17E-03

37   47     1   3   1   -14.33  -17.88   32.40   -128.72     2    25.75   19.12   -6.63   2.08E-02   1.23E-01

38   48     3  -2   1   -36.12  -35.80   71.92   -135.26     2

49     2  -3   1    -0.03   23.09   32.65     90.08     2    61.94   49.17  -12.77   2.08E-02   4.55E-01

39   50     3   2   1   -21.08   -8.60   32.20   -157.81     2    24.02   24.58    0.57   2.08E-02   8.94E-04   UR

40   51     2   3   1    -8.93   -8.98   17.92   -134.85     2    13.26   24.58   11.32   2.08E-02   3.58E-01   UR

41   52     4   0   1   -13.25    1.70   18.89    172.68     2

53     4  -1   1    -5.29  -25.37   36.65   -101.77     2    30.04   24.58   -5.46   2.08E-02   8.32E-02   U

42   54     0   0   2    -1.52   -0.54    1.62   -160.32     1     1.54 2728.77 2727.22   2.08E-02   2.08E+04   UR

43   55     1   0   2    20.94   -9.45   32.50    -24.29     2

56     0   1   2   -11.27   34.01   50.66    108.33     2    56.30   51.90   -4.40   2.08E-02   5.41E-02

44   57     1  -1   2     1.95   -0.89    3.03    -24.40     2     2.80   13.66   10.86   2.08E-02   3.29E-01   UR

45   58     1   1   2     0.39    3.13    4.46     82.82     2     4.07   13.66    9.59   2.08E-02   2.57E-01   UR

46   59     2   0   2     1.39   -0.19    1.98     -7.75     2     1.75   13.66   11.91   2.08E-02   3.96E-01   UR

47   60     0   2   2    -4.32   -1.43    6.44   -161.67     2     5.66   13.66    8.00   2.08E-02   1.78E-01   UR

48   61     2  -1   2    -1.14  -35.78   50.63    -91.83     2

62     1  -2   2    -2.68  -15.99   22.93    -99.52     2    48.43   60.09   11.66   2.08E-02   3.80E-01

49   63     2   1   2   -12.31  -28.19   43.50   -113.58     2

64     1   2   2    31.19    7.81   45.47     14.07     2    53.79   62.82    9.04   2.08E-02   2.28E-01

50   65     2  -2   2    -1.80   -0.02    2.55   -179.42     2     2.12   16.39   14.27   2.08E-02   5.68E-01   UR

51   66     3   0   2   -10.05  -23.58   36.25   -113.08     2

67     3  -1   2     2.32   -0.85    3.50    -20.03     2

68     2   2   2     0.70    0.35    1.11     26.23     2    29.14   27.32   -1.83   2.08E-02   9.31E-03

52   69     0   3   2     2.77   17.29   24.76     80.91     2    19.66   16.39   -3.27   2.08E-02   2.98E-02   U

53   70     1  -3   2     4.66   -2.40    7.42    -27.27     2     5.86   16.39   10.52   2.08E-02   3.09E-01   UR

(CYCLE  1)  Page  15

54   71     3   1   2     1.06    0.96    2.03     42.30     2     1.57   16.39   14.81   2.08E-02   6.13E-01   UR

55   72     1   3   2     3.20    3.48    6.69     47.37     2     5.14   16.39   11.25   2.08E-02   3.53E-01   UR

56   73     3  -2   2   -34.90  -30.93   65.94   -138.45     2

74     2  -3   2    12.88  -14.51   27.43    -48.41     2    54.09   68.29   14.20   2.08E-02   5.63E-01

57   75     3   2   2    25.83    0.62   36.54      1.38     2    26.32   24.58   -1.74   2.08E-02   8.41E-03   U

58   76     2   3   2    -4.95   -2.12    7.61   -156.81     2     5.44   24.58   19.14   2.08E-02   1.02E+00   UR

59   77     4   0   2     1.88    0.57    2.78     16.83     2

78     4  -1   2   -29.87  -29.23   59.11   -135.62     2    41.64   27.32  -14.33   2.08E-02   5.73E-01

60   79     1   0   3   -23.38   -5.05   33.82   -167.81     2

80     0   1   3     4.10   -2.15    6.55    -27.69     2    30.41   49.17   18.75   2.08E-02   9.82E-01

61   81     1  -1   3     7.58  -17.56   27.05    -66.64     2    23.54   35.51   11.97   2.08E-02   4.00E-01

62   82     1   1   3     1.37  -33.98   48.10    -87.68     2    41.46   35.51   -5.95   2.08E-02   9.88E-02

63   83     2   0   3   -45.74   26.05   74.44    150.33     2

84     0   2   3     8.64   12.32   21.28     54.95     2    64.26   73.75    9.49   2.08E-02   2.51E-01

64   85     2  -1   3    17.62   37.63   58.76     64.91     2

86     1  -2   3   -13.48   -5.84   20.77   -156.55     2    51.25   62.82   11.57   2.08E-02   3.74E-01

65   87     2   1   3    24.44   32.52   57.53     53.08     2

88     1   2   3   -29.12    2.34   41.31    175.40     2    57.13   38.24  -18.89   2.08E-02   9.96E-01

66   89     2  -2   3   -27.28   12.94   42.70    154.63     2    33.49   16.39  -17.10   2.08E-02   8.17E-01   U

67   90     3   0   3    37.71   12.03   55.97     17.69     2

91     3  -1   3     0.77   29.67   41.97     88.51     2

92     2   2   3   -32.64   21.77   55.49    146.30     2    67.39   65.56   -1.84   2.08E-02   9.42E-03

68   93     0   3   3     9.09   -2.89   13.49    -17.65     2

94     1  -3   3    10.50   -9.88   20.39    -43.26     2    18.23   60.09   41.86   2.08E-02   4.89E+00

69   95     3   1   3   -16.18   49.20   73.25    108.21     2    53.73   38.24  -15.48   2.08E-02   6.69E-01

70   96     0   0   4    67.95   -8.56   68.48     -7.18     1

97     0   1   4     0.97    5.51    7.91     79.96     2

98     1   0   4     0.97   -1.38    2.39    -54.74     2    56.23 2728.77 2672.53   2.08E-02   1.99E+04   UR

71   99     1  -1   4   -11.84    9.36   21.34    141.66     2

100     1   1   4    -7.35  -26.16   38.43   -105.70     2    34.94   21.85  -13.09   2.08E-02   4.78E-01

72  101     2   0   4     1.78  -22.81   32.35    -85.54     2

102     0   2   4    56.65   -6.84   80.69     -6.88     2

103     2  -1   4     3.18   -2.62    5.83    -39.45     2

104     1  -2   4    -7.30   -0.29   10.33   -177.76     2    66.53   76.48    9.95   2.08E-02   2.76E-01

73  105     2   1   4     1.14    0.85    2.01     36.75     2     1.50   24.58   23.08   2.08E-02   1.49E+00   UR

(CYCLE  1)  Page  16

74  106     1   2   4     4.52    0.97    6.54     12.14     2     4.86   24.58   19.72   2.08E-02   1.09E+00   UR

75  107     2  -2   4   -33.22    3.60   47.26    173.82     2    34.18   24.58   -9.60   2.08E-02   2.57E-01   U

76  108     3   0   4     2.50    1.54    4.16     31.57     2     2.91   24.58   21.67   2.08E-02   1.31E+00   UR

77  109     3  -1   4    24.21  -17.26   42.04    -35.48     2

110     2   2   4    25.31  -41.58   68.84    -58.67     2    56.14   49.17   -6.97   2.08E-02   1.36E-01

-X-ray Data Statistics

Excl Rej-Unobs    Excl Unobs

No of Spots                 51            37

Sum of Scale*Fo        2264.41       2029.50

Sum of Fc              2281.57       1924.84

Sum of Abs Delta        516.53        394.71

R-Factors

Normal                 22.81         19.45

Weighted Quadratic     21.54         19.63

Weighted Normal        22.81         19.45

spots Rejected from Least Squares --   0 Observed,  26 Unobserved

-Normal Scattering from Unit Cell --    332.0 Electrons

-===  Solution of Normal Equations  ===

-Eigen Values

1.35E+04  7.31E+03  6.16E+03  4.34E+03  1.81E+03  1.54E+03  1.05E+03  1.04E+03  7.69E+02  7.61E+02

6.52E+02  5.86E+02  4.48E+02  3.81E+02  3.40E+02  2.45E+02  2.19E+02  1.53E+02  1.40E+02  1.22E+02

7.65E+01  6.01E+01  3.80E+01  2.55E+01  2.02E+01 -1.81E-01  1.69E-01 -1.34E-01 -9.40E-02 -4.96E-02

-3.63E-02 -3.05E-02 -2.65E-02 -1.07E-02 -9.29E-03 -5.26E-03 -3.97E-03 -2.71E-03 -9.92E-04  1.83E-12

-5.84E-13 -2.38E-13 -8.26E-14  3.42E-14

Last  5 Filteres - Goodliness is   0.00 - Residual is  5.42E-16

-  No of     Varied   Applied   EignVals    Data    Degs of

Data     Params   Cnstrnts  Filtered   Excess   Freedom

135        26        18         5       122        13

Weight Analysis      Restnts   El Pare    Bragg   Continuous Contacts  Overall

Sigam (WT*Delta**2)  0.00E+00  4.71E+00  2.36E+01  0.00E+00  5.55E+01  8.38E+01

Data Contributrion          0        17        51         0        67       135

Mean (WT*Delta**2)   0.00E+00  2.77E-01  4.62E-01  0.00E+00  8.29E-01  6.21E-01

(CYCLE  1)  Page  17

-Shifts in Parameters - Shift Scaling Factor is 0.500

Parameter             Shift      New      Bound

THTA(  4)              0.000   -95.923   -90.000

THTA(  5)             -0.002  -143.776  -120.000

THTA(  8)              0.004   -75.825   -60.000

THTA(  9)             -0.011  -176.428  -177.000

THTA( 10)              0.009    60.609    62.400

THTA( 12)              0.000   -61.922   -62.800

THTA( 14)             -0.010    58.315    57.500

THTA( 16)              0.008   -54.871   -53.200

THTA( 17)              0.003    53.650    52.200

TAU(  4)              0.002   115.907   116.500

TAU(  5)              0.002   107.842   108.600

TAU(  9)             -0.005   110.087   110.160

TAU( 10)             -0.001   112.822   112.270

TAU( 12)              0.002   109.592   109.340

TAU( 14)              0.002   110.032   110.530

TAU( 16)              0.001   109.520   110.300

TAU( 17)             -0.001   110.564   110.170

OTHR(  1) SCLE( 0)    0.0000    2.7315

OTHR(  2) ATTN( 0)    0.0061    4.9979

OTHR(  5) EP-X( 1)   -0.0025  120.3084

OTHR(  6) EP-Y( 1)   -0.0049   60.3245

OTHR(  7) EP-Z( 1)    0.0033  -81.8836

OTHR(  8)    S( 1)   -0.0001    0.9857

OTHR(  9)   MU( 1)    0.0087  -32.2489

OTHR( 13)   MU( 2)    0.0033  -29.0191

OTHR( 16)    W( 2)    0.0000   -0.2558

----  Dump Record Made ( 1)  ---

-Convergence Test, Mean Shift is  0.000 - YES

-===  CPU Seconds for Cycle  1  ===

Cycler    Coords    Contrr    Elastic   Cntcts    Sfactor   Filmat     Solve     Total

0.00      0.00      0.00      0.00      0.00      0.00      0.00      0.00      0.00

(CYCLE  2)  Page  18

-                              -----  Start of Cycle  2  (used   1.0 Seconds)  -----

-===  Current Values of Parameters  ===

1         2         3         4         5         6         7         8         9        10

Theta      0.000     0.000     0.000   -95.923  -143.776    95.334    64.282   -75.825  -176.428    60.609

Tau        0.000     0.000   107.300   115.907   107.842   110.400   112.698   111.799   110.087   112.822

Other    2.73150   4.99791   0.00000   0.00000 120.30840  60.32446 -81.88365   0.98573 -32.24894   0.00000

SCLE( 0)  ATTN( 0)  SCLE( 1)  ATTN( 1)  EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 1)     U( 1)

11        12        13        14        15        16        17        18        19        20

Theta   -179.282   -61.922  -179.475    58.315  -175.751   -54.871    53.650    58.048   -62.355    65.669

Tau      107.300   109.592   109.300   110.032   109.602   109.520   110.564   109.480   109.480   109.480

Other    0.00000   0.00000 -29.01912   0.50000   0.50000  -0.25583

V( 1)     W( 1)    MU( 2)     U( 2)     V( 2)     W( 2)

21        22        23        24

Theta    -24.286   -55.988    46.615   161.735

Tau      109.480   109.480   109.480   109.480

-===  Cycle Control Data  ===

-Cycle Control Card -

--- ZZ.

----  SC Output at Cycle  2  ---

(CYCLE  2)  Page  19

-===  Run Summary  ===

-  Cycle     Sum of    Worst    Elastic   Contacts  Contacts  Contact

Number   Constrts  Constrt    Sigma    In Table  Applied    Sigma

--        --        --        --        --        --        --

Bragg   Continuous Overall    No of    Net Vard   Shift    Time for

R-Factor  R-factor   Sigma      Data     Params    Scaler    Cycle

--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

1    0.0001    0.0000  4.71E+00        67        67  5.55E+01

22.81      0.00  8.38E+01       135        13     0.500      0.00

--- ZZ.

-===  Array Specifiers Used/Given  ===

ADEL    6/  150, APPR   18/  600, ATOM   24/ 3000, BNDI   10/ 6000, BNDL   88/ 4000,

CLIS   67/10000, CMOL    8/  500, COND   19/  600, CONP   44/ 2000, CONT    1/   50,

EPAR   17/  410, EQIV    1/   10, EQUN   44/  999, HBND    1/ 2000, MOLC    2/ 5000,

ORDR    1/  100, POSN    2/  100, REFL  110/30000, SCAT    8/   50, SOUM    1/  500,

SPOT   77/15000, STRU    1/  100, TARM    7/ 2000, THEQ   12/ 1000, TYPE    3/   50,

VARY    9/  500, VPAR   26/  500, VSKL   21/  500,

-===   WinLALS Happy Ending   ===

Total CPU Time =      1 seconds ( =  0 H  0 M  1 S)

End of This Job

c) Output 2 of Input file 1: WinLALS.pnc

Cycle=  2///////////////  WinLALS 14:39:17 01-11- 6  ///////////////SC=  1

L down P21   April 1997 Finkenstadt

O5A*               3    0.5000    0.0000    0.0000    0.0000

C1A*      O5A*     2    0.5000    1.4290    0.0000    0.0000

O4A       C1A*     3    0.5000    1.3890  107.3000    0.0000

C4A       O4A      2    0.5000    1.4260  115.9074  -95.9230

C5A       C4A      2    0.5000    1.5250  107.8422 -143.7763

C3A.      C4A      2    0.5000    1.5230  110.4000   95.3337

C6A       C5A      2    1.0000    1.5140  112.6980   64.2823

O6A       C6A      3    1.0000    1.4270  111.7990  -75.8247

O5A       C5A      3    0.5000    1.4360  110.0871 -176.4277

C1A       O5A      2    0.5000    1.4290  112.8217   60.6092

O4A*      C1A      3    0.5000    1.3890  107.3000 -179.2821

C2A       C1A      2    1.0000    1.5230  109.5922  -61.9221

O2A       C2A      3    1.0000    1.4230  109.3000 -179.4755

C3A       C2A      2    0.5000    1.5210  110.0323   58.3145

O3A       C3A      3    1.0000    1.4290  109.6020 -175.7511

C4A.      C3A      2    0.5000    1.5230  109.5203  -54.8711

C5A.      C4A.     2    0.5000    1.5250  110.5637   53.6498

H1A       C1A      1    1.0000    1.1000  109.4800   58.0479

H2A       C2A      1    1.0000    1.1000  109.4800  -62.3555

H3A       C3A      1    1.0000    1.1000  109.4800   65.6689

H4A       C4A      1    1.0000    1.1000  109.4800  -24.2863

H5A       C5A      1    1.0000    1.1000  109.4800  -55.9877

H61A      C6A      1    1.0000    1.1000  109.4800   46.6153

H62A      C6A      1    1.0000    1.1000  109.4800  161.7353

O5A         120.3084   60.3245  -81.8836    0.9857

2.7315    4.9979    0.0000    0.0000    1.0000

-32.2489    0.0000    0.0000  0.000000    1.0000    1.0000

-29.0191    0.5000    0.5000 -0.255835    1.0000    1.0000

d) Output 3 of Input file 1: WinLALS.err

WinLALS Run of 2001-11- 6 (14:39:17)

Blackwell valonia cellulose  PARALLEL down P21   April 1997 Finkenstadt

.--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

.        1    0.0001    0.0000  4.71E+00        67        67  5.55E+01

.    22.81      0.00  8.38E+01       135        13     0.500      0.00

.--- ZZ.

----  SC Output at Cycle  2  ---

e) Input file 2-1: LALS2.dat   The first character of Title line has ‘*’.

* pba( sin(th)/ramuda < 0.28 ), 3+4,21 and 46-51 are separate files *   1.title

1    1    2    1                                                    2.mol. sym.

99999                                                                   5.terminator

hc4       c5        hc5       c1        hc6       c7               6.theta equiv.

c6        c2        hn        c3        o         n

99999                                                                   7.terminator

1.0000    8.0000                                                    8.X-ray refinables

0.0000    0.0000    0.0000    0.0000    1.0000    1.0000            9.pos. defi.

99999                                                                   10.terminator

1.0000    0.0000    0.0000    0.0000    1.0000    0.0000            11.equiv. pos.

0.0000    0.0000    1.0000    0.0000    0.0000    0.0000

1.0000    0.0000    0.0000    0.0000   -1.0000    0.0000

0.0000    0.0000   -1.0000    0.5000    0.5000    0.0000

99999                                                                   12.terminator

1    1    1    0                                                    13.gen. mole. def.

1    2    1    0

99999                                                                   14.terminator

tra       0.0       0.0       1.0                                       15.trans. const. & restr.

coi       0.

3         0    1   12         1    1

coi       0.

1         0    1    7         1    1

coi       0.

2         0    1   10         1    1

par      2       0.0       0.0

th       5       1.0

th      12       1.0

99999                                                                   16.terminator

7.740     5.270     12.82      90.0     90.0      90.0             17.unit cell

1.0       0.0       0.0       0.0                                19.Cruick. weights

0002 0003 0004 0001   -1                                               20.scatt. fact. codes

99999                                                                   22.terminator

100    1    0                                                         23.reject fac. & select

10.0                                                              24.overall elastic weight

500                                                                   26.contact opt.

1    0    0    0                                                    27.source unit spec.

99999                                                                   28.terminator

2    2   -4    1    0    0    0    1         0                      29.target unit spec.

2    2   -4    1   -1    0    0    1         0

2    2   -4    1    0   -1    0    1         0

2    2   -4    1   -1   -1    0    1         0

1    1   -4    3    1    0    0    1         0

1    1   -4    3   -1    0    0    1         0

1    1   -4    3    0    1    0    1         0

99999                                                                   30.terminator

c1.       c1       1                                               31.atom redundancy

c2.       c2       1

n.        n        1

99999                                                                   32.terminator

c3        c7                                                       33.type I bond list

99999                                                                   34.terminator

99999                                                                   36.terminator

99999                                                                   38.terminator

2.50      3.27      2.9       10.0      120       60                    39.attractive bond spec.

n                   o                                              40.attract. bond donors and accepters

1.50      2.92      1.7       10.0      180       60

hn                  o

99999                                                                   41.terminator

3.60                                                              42.contact limit table

3.45      3.30

3.42      3.27      3.24

3.10      2.95      2.92      2.60

99999                                                                   43.terminator

1.0                                                              44.ovarall contact weights

1.03                                                              45.indiv. contact weights

1.31      1.71

1.17      1.48      1.33

0.99      1.29      1.14      1.04

f) Input file 2-2: LALS2Atom.dat

Z-matrix (Internal coordinates) of WinLALS                              3-1.local title

c1.                1    0.5000    0.0000    0.0000    0.0000            3-2.stereo chem.

c2.       c1.      1    0.5000    1.5300    0.0000    0.0000              (KAT(i),IPR(i),KPR(i),ITYP(i),

n.        c2.      2    0.5000    1.3200  114.0624    0.0000               OCC(i),B(i),TAU(i),THETA(i),i=1,NAT)

c3        n.       1    1.0000    1.4700  123.2306  180.0000

c4        c3       1    1.0000    1.3800  121.2152    0.0000

c5        c4       1    1.0000    1.3800  120.0000  180.0000

c1        c5       1    0.5000    1.3800  120.0000    0.0000

c6        c1       1    1.0000    1.3800  120.0000    0.0000

c7        c6       1    1.0000    1.3800  120.0000    0.0000

c2        c1       1    0.5000    1.5300  122.9189  180.0000

o         c2       3    1.0000    1.2400  121.0000  180.0000

n         c2       2    0.5000    1.3200  114.0624    0.0000

hn        n.       4    1.0000    1.0000  123.0000    0.0000

hc4       c4       4    1.0000    1.0840  120.0000    0.0000

hc5       c5       4    1.0000    1.0840  120.0000 -180.0000

hc6       c6       4    1.0000    1.0840  120.0000  180.0000

hc7       c7       4    1.0000    1.0840  120.0000  180.0000

n            0.0000    0.0000    0.0000    0.0000             4.root drf., IRUT,KRUT,EPX,EPY,EPZ,S,

1    2    3                                                                       I1,I2,I3

99999

g) Input file 2-3: LALS2Fobs.dat

1    1    0    2    31.610       1.0                                21.X-ray data

2    0    0    1    48.670       1.0

2    1    0    2    16.760       1.0

0    2    0    1     2.990      -1.0

1    2    0    2     4.130       1.0

3    1    0    2    10.220       1.0

2    2    0    2     3.370      -1.0

4    0    0    1                 0.0

3    2    0    2                 0.0

4    1    0    2    12.570       1.0

1    0    1    1     1.670      -1.0

0    1    1    1     8.640       1.0

1    1    1    2    10.340       1.0

2    0    1    1     6.780       1.0

2    1    1    2    19.400       1.0

0    2    1    1                 0.0

3    0    1    1                 0.0

1    2    1    2     8.820       1.0

3    1    1    2                 0.0

2    2    1    2     6.680       1.0

4    0    1    1     3.640      -1.0

3    2    1    2                 0.0

4    1    1    2     9.750       1.0

1    0    2    1     1.670      -1.0

0    1    2    1     3.800       1.0

1    1    2    2     5.390       1.0

2    0    2    1     7.450       1.0

2    1    2    2    11.350       1.0

0    2    2    1                 0.0

3    0    2    1                 0.0

1    2    2    2     7.050       1.0

3    1    2    2     3.250      -1.0

2    2    2    2     3.380      -1.0

4    0    2    1    10.030       1.0

1    0    3    1     1.630      -1.0

0    1    3    2     8.440       1.0

2    0    3    1     2.410      -1.0

2    1    3    2     9.040       1.0

0    2    3    1    12.080       1.0

3    0    3    1     3.050      -1.0

1    2    3    2                 0.0

3    1    3    2                 0.0

2    2    3    2    12.120       1.0

1    0    4    1                 0.0

0    1    4    1     6.460       1.0

1    1    4    2                 0.0

2    0    4    1     7.570       1.0

2    1    4    2                 0.0

0    2    4    1                 0.0

3    0    4    1     6.430       1.0

1    2    4    2                 0.0

3    1    4    2                 0.0

2    2    4    2                 0.0

4    0    4    1     4.450       1.0

1    0    5    1     1.620      -1.0

0    1    5    1                 0.0

1    1    5    2     5.020       1.0

2    0    5    1     2.290      -1.0

2    1    5    2                 0.0

0    2    5    1                 0.0

3    0    5    1     5.660       1.0

1    0    6    1                 0.0

0    1    6    1    12.000       1.0

1    1    6    2     1.620      -1.0

2    0    6    1                 0.0

2    1    6    2     4.610       1.0

h) Input file 2-4: LALS2Cont.dat

cl=20,vo=4,eo=4,ac=9,nx,nc,cn,el,sh,fi,fo.                             46.cycle control, CL=n,CY=n,CO,SF=n,CX,ST=n,ZZ,PU.

5    6    7    8                                                   47.A (IUTH(i),i=1,NCTH)

5    6    7    8                                                   47.B (IETH(i),i=1,NCTH)

400.0     400.0     400.0     400.0                                    (BUTH(i),i=1,NCTH)

50.0      50.0      50.0      50.0                                    (WTH(i),i=1,NCTH)

1    2    3    4    5    6    7    8    9

50.0       1.0       1.0       5.0

zz.

cl=8,vh=3,eh=3,vt=2,et=2,vo=8,eo=4,ac=10,cn,el,xr,xs,cd,cs,sh,fi,fo,ma.

4    5   12

4    5   12

400.0     400.0     400.0

1.0       5.0       5.0

5   10    3    4   12

5   10    3    4   12

-400.0     122.5     400.0     400.0     400.0

50.0       1.0      50.0      50.0      50.0

3    4    5    6    7   10    1    2

3    4    5    6

400.0     400.0     400.0     400.0

30.0      30.0      30.0      30.0

1    2    3    4    5    6    7    8    9   10

20.0       1.0       1.0       4.0

zz.

cl=5,vo=1,xr,xs,cs,cd,sh.

1

zz.

i) Output 4 of Input file 2: LALS2.prn

OO   Win     OO            * * * * * * * * * * * * * * * * * *   OO   Win     OO

OO           OO            *                                 *   OO           OO

OO   OOOOOO  OO   OOOOOO   *   Linked Atom Least Squares     *   OO   OOOOOO  OO   OOOOOO

OO   OO  OO  OO   OO  OO   *           (LALS)                *   OO   OO  OO  OO   OO  OO

OO       OO  OO   OO       *   Structure Refinement System   *   OO       OO  OO   OO

OO   OOOOOO  OO   OOOOOO   *      for Helical Polymers       *   OO   OOOOOO  OO   OOOOOO

OO   OO  OO  OO       OO   *       - WinLALS ver. 1.0-       *   OO   OO  OO  OO       OO

OO   OO  OO  OO   OO  OO   *                                 *   OO   OO  OO  OO   OO  OO

OOOO OOOOOOO OOOO OOOOOO   * * * * * * * * * * * * * * * * * *   OOOO OOOOOOO OOOO OOOOOO

-Blackwell valonia cellulose  PARALLEL down P21   April 1997 Finkenstadt

Run of 2001-11- 6 (14:59:17)

---------------------------------------------------------------------------------------------------

-===  Main Data Input  ===

-Windows Trig Version of 2001/7/10.

-Helix Properties

Single, RIGHT-Handed,  2 Residues in  1 Turns

-Geometry and Atomic Properties of Structure  1

Title: Z-matrix (Internal coordinates) of WinLALS

Atom   Precursor    Type   Occupancy     B        Tau      Theta

1(O5A*)                3      0.500

2(C1A*)   1(O5A*)      2      0.500     1.4290

3(O4A )   2(C1A*)      3      0.500     1.3890    107.30

4(C4A )   3(O4A )      2      0.500     1.4260    115.91    -95.92

5(C5A )   4(C4A )      2      0.500     1.5250    107.84   -143.77

6(C3A.)   4(C4A )      2      0.500     1.5230    110.40     95.34

7(C6A )   5(C5A )      2      1.000     1.5140    112.70     64.29

8(O6A )   7(C6A )      3      1.000     1.4270    111.80    -75.83

9(O5A )   5(C5A )      3      0.500     1.4360    110.09   -176.42

10(C1A )   9(O5A )      2      0.500     1.4290    112.82     60.60

11(O4A*)  10(C1A )      3      0.500     1.3890    107.30   -179.28

12(C2A )  10(C1A )      2      1.000     1.5230    109.59    -61.92

13(O2A )  12(C2A )      3      1.000     1.4230    109.30   -179.47

14(C3A )  12(C2A )      2      0.500     1.5210    110.03     58.32

15(O3A )  14(C3A )      3      1.000     1.4290    109.60   -175.76

16(C4A.)  14(C3A )      2      0.500     1.5230    109.52    -54.88

17(C5A.)  16(C4A.)      2      0.500     1.5250    110.56     53.65

18(H1A )  10(C1A )      1      1.000     1.1000    109.48     58.05

19(H2A )  12(C2A )      1      1.000     1.1000    109.48    -62.35

20(H3A )  14(C3A )      1      1.000     1.1000    109.48     65.66

21(H4A )   4(C4A )      1      1.000     1.1000    109.48    -24.28

22(H5A )   5(C5A )      1      1.000     1.1000    109.48    -55.98

23(H61A)   7(C6A )      1      1.000     1.1000    109.48     46.61

24(H62A)   7(C6A )      1      1.000     1.1000    109.48    161.73

Root Atom  Epsln-X   Epsln-Y   Epsln-Z      S

9(O5A )   120.31     60.33    -81.89    0.9858

Other ( 5)   ( 6)      ( 7)      ( 8)

Epsilons are applied in the order X, Y, Z

(INPUT  0)  Page   2

-Equivalenced Thetas

6(C3A.) to   5(C5A )           7(C6A ) to   9(O5A )          11(O4A*) to  12(C2A )

13(O2A ) to  14(C3A )          15(O3A ) to  16(C4A.)          18(H1A ) to  12(C2A )

19(H2A ) to  14(C3A )          20(H3A ) to  16(C4A.)          21(H4A ) to   5(C5A )

22(H5A ) to   9(O5A )          23(H61A) to   8(O6A )          24(H62A) to   8(O6A )

-X-ray Data-Fitting Refinables

Bragg      Scale (K) = Other(1)    2.732

Atten (B) = Other(2)    4.992

Continuous Scale (K) = Other(3)    0.000

Atten (B) = Other(4)    0.000

Water Density = 0.2984 *    1.00 el/A. cube

-Generation Details for Molecule Positions

Posn       Mu       U        V        W    Occupancy  Sense

1       -32.26  0.0000   0.0000   0.0000   1.000     UP

Other (  9)  ( 10)    ( 11)    ( 12)

2       -29.02  0.5000   0.5000  -0.2559   1.000     UP

Other ( 13)  ( 14)    ( 15)    ( 16)

-Identity Equivalent Position Generated

-Molecule Definitions

Molecule  Position  Equvlnce  Strcture   Helix

1         1         1         1       YES

2         2         1         1       YES

-Constraint and Restraint Specifications

Number   Type         Details

TRA           0.0000    0.0000    1.0000

1-  3   COI           0.0000

1(O5A*)    0    1   9(O5A )    1    1

4-  6   COI           0.0000

2(C1A*)    0    1  10(C1A )    1    1

7-  9   COI           0.0000

3(O4A )    0    1  11(O4A*)    1    1

10- 12   COI           0.0000

14(C3A )    0    1   6(C3A.)    0    1

13- 15   COI           0.0000

4(C4A )    0    1  16(C4A.)    0    1

16- 18   COI           0.0000

5(C5A )    0    1  17(C5A.)    0    1

19- 19   PAR      2    0.0000    0.0000

OTHR(  9)   -1.0000

OTHR( 13)    1.0000

(INPUT  0)  Page   3

-Unit Cell Parameters

A         B         C         Alpha     Beta      Gamma

8.1700    7.8600   10.3800     90.00     90.00     97.00

-Cruickshank Weighting Parameters (Bragg)

A         B         C

4.80E+01  0.00E+00  0.00E+00

Reciprocal Weight for Continuous X-ray Data is  0.000

-Division Factor for Structure Factor Stats is 2.00

-Scattering Factor Codes

1=    1   2=    2   3=    4

-Indices and Observed Structure Factor Amplitudes

Spot   Reflection    H    K    L    M   F-obs    Weight

1         1       0    1    0    2    3.00    -1.000

2         2       1    0    0    2    3.00    -1.000

3         3       1   -1    0    2   39.00     1.000

4         4       1    1    0    2   27.00     1.000

5         5       2    0    0    2    5.00    -1.000

6         6       0    2    0    2

6         7       2   -1    0    2  100.00     1.000

7         8       1   -2    0    2    3.00    -1.000

8         9       2    1    0    2    3.00    -1.000

9        10       1    2    0    2    3.00    -1.000

10        11       2   -2    0    2    3.00    -1.000

11        12       3    0    0    2    3.00    -1.000

12        13       3   -1    0    2

12        14       2    2    0    2   14.00     1.000

13        15       0    3    0    2

13        16       1   -3    0    2   15.00     1.000

14        17       3    1    0    2   12.00     1.000

15        18       1    3    0    2

15        19       3   -2    0    2   11.00     1.000

16        20       2   -3    0    2    4.00    -1.000

17        21       3    2    0    2    4.00    -1.000

18        22       2    3    0    2    4.00    -1.000

19        23       4    0    0    2

19        24       4   -1    0    2   14.00     1.000

20        25       3   -3    0    2    9.00    -1.000

21        26       1   -4    0    2

21        27       0    4    0    2   36.00     1.000

22        28       4    1    0    2    9.00    -1.000

23        29       4   -2    0    2    9.00    -1.000

24        30       1    0    1    2

24        31       0    1    1    2    8.00     1.000

(INPUT  0)  Page   4

25        32       1   -1    1    2    5.00    -1.000

26        33       1    1    1    2    6.00     1.000

27        34       2    0    1    2    7.00     1.000

28        35       0    2    1    2    5.00    -1.000

29        36       2   -1    1    2

29        37       1   -2    1    2   11.00     1.000

30        38       2    1    1    2    5.00    -1.000

31        39       1    2    1    2    5.00    -1.000

32        40       2   -2    1    2    5.00    -1.000

33        41       3    0    1    2   25.00     1.000

34        42       3   -1    1    2

34        43       2    2    1    2   23.00     1.000

35        44       0   -3    1    2

35        45      -1    3    1    2   24.00     1.000

36        46       3    1    1    2   15.00     1.000

37        47       1    3    1    2    7.00     1.000

38        48       3   -2    1    2

38        49       2   -3    1    2   18.00     1.000

39        50       3    2    1    2    9.00    -1.000

40        51       2    3    1    2    9.00    -1.000

41        52       4    0    1    2

41        53       4   -1    1    2    9.00    -1.000

42        54       0    0    2    1  999.00    -1.000

43        55       1    0    2    2

43        56       0    1    2    2   19.00     1.000

44        57       1   -1    2    2    5.00    -1.000

45        58       1    1    2    2    5.00    -1.000

46        59       2    0    2    2    5.00    -1.000

47        60       0    2    2    2    5.00    -1.000

48        61       2   -1    2    2

48        62       1   -2    2    2   22.00     1.000

49        63       2    1    2    2

49        64       1    2    2    2   23.00     1.000

50        65       2   -2    2    2    6.00    -1.000

51        66       3    0    2    2

51        67       3   -1    2    2

51        68       2    2    2    2   10.00     1.000

52        69       0    3    2    2    6.00    -1.000

53        70       1   -3    2    2    6.00    -1.000

54        71       3    1    2    2    6.00    -1.000

55        72       1    3    2    2    6.00    -1.000

56        73       3   -2    2    2

56        74       2   -3    2    2   25.00     1.000

57        75       3    2    2    2    9.00    -1.000

58        76       2    3    2    2    9.00    -1.000

59        77       4    0    2    2

59        78       4   -1    2    2   10.00     1.000

60        79       1    0    3    2

60        80       0    1    3    2   18.00     1.000

61        81       1   -1    3    2   13.00     1.000

62        82       1    1    3    2   13.00     1.000

63        83       2    0    3    2

63        84       0    2    3    2   27.00     1.000

64        85       2   -1    3    2

64        86       1   -2    3    2   23.00     1.000

65        87       2    1    3    2

(INPUT  0)  Page   5

65        88       1    2    3    2   14.00     1.000

66        89       2   -2    3    2    6.00    -1.000

67        90       3    0    3    2

67        91       3   -1    3    2

67        92       2    2    3    2   24.00     1.000

68        93       0    3    3    2

68        94       1   -3    3    2   22.00     1.000

69        95       3    1    3    2   14.00     1.000

70        96       0    0    4    1

70        97       0    1    4    2

70        98       1    0    4    2  999.00    -1.000

71        99       1   -1    4    2

71       100       1    1    4    2    8.00     1.000

72       101       2    0    4    2

72       102       0    2    4    2

72       103       2   -1    4    2

72       104       1   -2    4    2   28.00     1.000

73       105       2    1    4    2    9.00    -1.000

74       106       1    2    4    2    9.00    -1.000

75       107       2   -2    4    2    9.00    -1.000

76       108       3    0    4    2    9.00    -1.000

77       109       3   -1    4    2

77       110       2    2    4    2   18.00     1.000

-Rejection Factor for All Spots is   100, for Unobserved Spots is     1

-Relative Weighting of Elastic Data is  4.500E-02

-Short Contact Calculation Data

-Contact List Length  500, Min Diff for Printing   40 Hundredths

-Source Unit Specifications

From       To       From       To

Molecule  Molecule   Residue   Residue   S-Code

1         1         0         0         1

-Target Unit Specifications

From       To       From       To         Units of Displacement      Multi-

Molecule  Molecule   Residue   Residue   Along A   Along B   Along C   Plicity   T-Code

1         1         0         1         0         0         0         1         1

1         1        -5         5         1         0         0         1         1

1         1        -5         5         0         1         0         1         1

2         2        -5         5         0         0         0         1         1

2         2        -5         5        -1         0         0         1         1

2         2        -5         5         0        -1         0         1         1

2         2        -5         5        -1        -1         0         1         1

(INPUT  0)  Page   6

-Atom Redundancy List

11(O4A*) is the same as   3(O4A ) with -1 Helix Ops

1(O5A*) is the same as   9(O5A ) with  1 Helix Ops

2(C1A*) is the same as  10(C1A ) with  1 Helix Ops

6(C3A.) is the same as  14(C3A )

16(C4A.) is the same as   4(C4A )

17(C5A.) is the same as   5(C5A )

-Attractive Interaction Parameters

Type   1 - R1 = 2.50  R2 = 3.10  D0 = 2.90  W =   5.00  Ang = 120.00  Dev =  20.00

Donor/Acceptor List      8(O6A )   13(O2A )   15(O3A )   -1(    )    8(O6A )   13(O2A )

Donor/Acceptor List     15(O3A )    9(O5A )    3(O4A )   -1(    )

-Contact Limit Table

1      2      3

1   2.60

2   3.10   3.60

3   2.92   3.42   3.24

-Overall Contact Distance Weight is 6.00E+00

-Contact Weight Table

1      2      3

1   1.04

2   0.99   1.03

3   1.14   1.17   1.33

(CYCLE  1)  Page   7

-                              -----  Start of Cycle  1  (used   0.0 Seconds)  -----

-===  Current Values of Parameters  ===

1         2         3         4         5         6         7         8         9        10

Theta      0.000     0.000     0.000   -95.923  -143.774    95.336    64.294   -75.829  -176.416    60.600

Tau        0.000     0.000   107.300   115.905   107.840   110.400   112.698   111.799   110.092   112.822

Other    2.73150   4.99180   0.00000   0.00000 120.31090  60.32940 -81.88690   0.98580 -32.25760   0.00000

SCLE( 0)  ATTN( 0)  SCLE( 1)  ATTN( 1)  EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 1)     U( 1)

11        12        13        14        15        16        17        18        19        20

Theta   -179.282   -61.922  -179.465    58.325  -175.759   -54.879    53.647    58.048   -62.345    65.661

Tau      107.300   109.590   109.300   110.031   109.602   109.519   110.565   109.480   109.480   109.480

Other    0.00000   0.00000 -29.02240   0.50000   0.50000  -0.25587

V( 1)     W( 1)    MU( 2)     U( 2)     V( 2)     W( 2)

21        22        23        24

Theta    -24.284   -55.976    46.611   161.731

Tau      109.480   109.480   109.480   109.480

-===  Cycle Control Data  ===

-Cycle Control Card -

--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

Elastically Bound Thetas

Theta  Bound    Wt       Theta  Bound    Wt       Theta  Bound    Wt       Theta  Bound    Wt

4   -90.00   0.10        5  -120.00   0.10        8   -60.00   0.10        9  -177.00   0.63

10    62.40   0.69       12   -62.80   0.92       14    57.50   0.63       16   -53.20   0.92

17    52.20   0.92

Elastically Bound Taus

Tau    Bound    Wt       Tau    Bound    Wt       Tau    Bound    Wt       Tau    Bound    Wt

4   116.50   2.50        5   108.60   2.80        9   110.16   5.90       10   112.27  10.00

12   109.34   8.30       14   110.53   5.90       16   110.30   5.10       17   110.17   5.10

Refined Other Parameters

5         6         7         8        13         1         9        16         2

EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 2)  SCLE( 0)    MU( 1)     W( 2)  ATTN( 0)

(CYCLE  1)  Page   8

Applied Constraints and Restraints

1    2    3    4    5    6    7    8    9   10   11   12

13   14   15   16   17   18

( 18 are Constraints,  0 are Restraints)

Max Angle Shift      20.0

Max Dist Shift       1.00

Damping Factor       0.50

Filter Number           5

Convergency Limit  0.0010

---  Run of  6 Cycles  ---

-===  Molecule Building  ===

-Equivalenced Theta Implementation

Thta(  6) Tied to Thta(  5), Difference  120.89

Thta( 21) Tied to Thta(  5), Difference -119.49

Thta( 23) Tied to Thta(  8), Difference -122.44

Thta( 24) Tied to Thta(  8), Difference  122.44

Thta(  7) Tied to Thta(  9), Difference  119.29

Thta( 22) Tied to Thta(  9), Difference -120.44

Thta( 11) Tied to Thta( 12), Difference  117.36

Thta( 18) Tied to Thta( 12), Difference -119.97

Thta( 13) Tied to Thta( 14), Difference -122.21

Thta( 19) Tied to Thta( 14), Difference  120.67

Thta( 15) Tied to Thta( 16), Difference  120.88

Thta( 20) Tied to Thta( 16), Difference -120.54

-Coordinates of Base Residue

Atom       X         Y         Z             R        Phi        Z

1(O5A*)  -0.9858    0.0000    5.1900        0.9858   -180.00    5.1900

2(C1A*)   0.3541    0.0677    4.6980        0.3606     10.83    4.6980

3(O4A )   0.3210    0.7802    3.5061        0.8436     67.63    3.5061

4(C4A )   0.2555   -0.0011    2.3150        0.2555     -0.24    2.3150

5(C5A )   1.0861    0.7037    1.2477        1.2942     32.94    1.2477

6(C3A.)  -1.1892   -0.1501    1.8566        1.1986   -172.81    1.8566

7(C6A )   2.5570    0.7686    1.6005        2.6700     16.73    1.6005

8(O6A )   2.8205    1.7580    2.5944        3.3235     31.94    2.5944

9(O5A )   0.9858    0.0000    0.0000        0.9858      0.00    0.0000

10(C1A )  -0.3541   -0.0678   -0.4920        0.3606   -169.17   -0.4920

11(O4A*)  -0.3210   -0.7802   -1.6839        0.8436   -112.36   -1.6839

12(C2A )  -1.2234   -0.8324    0.4976        1.4797   -145.77    0.4976

13(O2A )  -2.5530   -0.9068   -0.0040        2.7092   -160.44   -0.0040

14(C3A )  -1.1892   -0.1501    1.8566        1.1986   -172.81    1.8566

15(O3A )  -1.9266   -0.9276    2.8020        2.1383   -154.29    2.8020

16(C4A.)   0.2555   -0.0011    2.3150        0.2555     -0.24    2.3150

17(C5A.)   1.0861    0.7037    1.2477        1.2942     32.94    1.2477

18(H1A )  -0.7513    0.9505   -0.6159        1.2116    128.32   -0.6159

19(H2A )  -0.8529   -1.8639    0.5906        2.0498   -114.59    0.5906

20(H3A )  -1.6620    0.8403    1.7819        1.8624    153.18    1.7819

21(H4A )   0.6687   -1.0018    2.5094        1.2045    -56.28    2.5094

(CYCLE  1)  Page   9

22(H5A )   0.7209    1.7333    1.1192        1.8772     67.42    1.1192

23(H61A)   2.8948   -0.2189    1.9481        2.9030     -4.33    1.9481

24(H62A)   3.1431    0.9756    0.6930        3.2911     17.24    0.6930

-Coordinates of Molecule  1, Residue  0  -  Helix Axis at X =   0.0000, Y =   0.0000

C a r t e s i a n                F r a c t i o n a l              C y l i n d r i c a l

Atom       X         Y         Z             X         Y         Z             R        Phi        Z          Atom

1(O5A*)  -0.8336    0.5261    5.1900      -0.10280   0.05392   0.50000        0.9858    147.74    5.1900      1(O5A*)

2(C1A*)   0.3356   -0.1317    4.6980       0.04139  -0.01152   0.45260        0.3606    -21.43    4.6980      2(C1A*)

3(O4A )   0.6879    0.4884    3.5061       0.08483   0.07289   0.33778        0.8436     35.38    3.5061      3(O4A )

4(C4A )   0.2155   -0.1373    2.3150       0.02658  -0.01410   0.22302        0.2555    -32.50    2.3150      4(C4A )

5(C5A )   1.2941    0.0154    1.2477       0.15958   0.02218   0.12020        1.2942      0.68    1.2477      5(C5A )

6(C3A.)  -1.0858    0.5078    1.8566      -0.13390   0.04764   0.17886        1.1986    154.94    1.8566      6(C3A.)

7(C6A )   2.5726   -0.7148    1.6005       0.31724  -0.05076   0.15419        2.6700    -15.53    1.6005      7(C6A )

8(O6A )   3.3235   -0.0187    2.5944       0.40984   0.04954   0.24994        3.3235     -0.32    2.5944      8(O6A )

9(O5A )   0.8336   -0.5261    0.0000       0.10280  -0.05392   0.00000        0.9858    -32.26    0.0000      9(O5A )

10(C1A )  -0.3356    0.1317   -0.4920      -0.04139   0.01151  -0.04740        0.3606    158.57   -0.4920     10(C1A )

11(O4A*)  -0.6879   -0.4884   -1.6839      -0.08482  -0.07289  -0.16222        0.8436   -144.62   -1.6839     11(O4A*)

12(C2A )  -1.4788   -0.0509    0.4976      -0.18237  -0.02958   0.04794        1.4797   -178.03    0.4976     12(C2A )

13(O2A )  -2.6429    0.5957   -0.0040      -0.32592   0.03450  -0.00038        2.7092    167.30   -0.0040     13(O2A )

14(C3A )  -1.0858    0.5078    1.8566      -0.13390   0.04764   0.17886        1.1986    154.94    1.8566     14(C3A )

15(O3A )  -2.1243    0.2438    2.8020      -0.26197  -0.00216   0.26994        2.1383    173.45    2.8020     15(O3A )

16(C4A.)   0.2155   -0.1373    2.3150       0.02658  -0.01410   0.22302        0.2555    -32.50    2.3150     16(C4A.)

17(C5A.)   1.2941    0.0154    1.2477       0.15958   0.02218   0.12020        1.2942      0.68    1.2477     17(C5A.)

18(H1A )  -0.1280    1.2048   -0.6159      -0.01579   0.15128  -0.05934        1.2116     96.07   -0.6159     18(H1A )

19(H2A )  -1.7161   -1.1210    0.5906      -0.21163  -0.16943   0.05689        2.0498   -146.85    0.5906     19(H2A )

20(H3A )  -0.9571    1.5976    1.7819      -0.11802   0.18831   0.17166        1.8624    120.92    1.7819     20(H3A )

21(H4A )   0.0308   -1.2041    2.5094       0.00380  -0.15271   0.24176        1.2045    -88.53    2.5094     21(H4A )

22(H5A )   1.5347    1.0811    1.1192       0.18926   0.16151   0.10783        1.8772     35.16    1.1192     22(H5A )

23(H61A)   2.3311   -1.7301    1.9481       0.28747  -0.18371   0.18767        2.9030    -36.58    1.9481     23(H61A)

24(H62A)   3.1787   -0.8525    0.6930       0.39200  -0.05880   0.06676        3.2911    -15.01    0.6930     24(H62A)

-Coordinates of Molecule  2, Residue  0  -  Helix Axis at X =   4.0546, Y =   3.4322

C a r t e s i a n                F r a c t i o n a l              C y l i n d r i c a l

Atom       X         Y         Z             X         Y         Z             R        Phi        Z          Atom

1(O5A*)   3.1925    3.9104    2.5340       0.39370   0.54738   0.24413        0.9858    150.98    2.5340      1(O5A*)

2(C1A*)   4.3971    3.3196    2.0421       0.54224   0.49103   0.19673        0.3606    -18.19    2.0421      2(C1A*)

3(O4A )   4.7138    3.9586    0.8502       0.58129   0.57728   0.08190        0.8436     38.61    0.8502      3(O4A )

4(C4A )   4.2775    3.3073   -0.3410       0.52749   0.48759  -0.03285        0.2555    -29.26   -0.3410      4(C4A )

5(C5A )   5.3457    3.5206   -1.4082       0.65922   0.53142  -0.13567        1.2942      3.92   -1.4082      5(C5A )

6(C3A.)   2.9419    3.8778   -0.7994       0.36278   0.53932  -0.07701        1.1986    158.17   -0.7994      6(C3A.)

7(C6A )   6.6634    2.8637   -1.0555       0.82171   0.46843  -0.10168        2.6700    -12.29   -1.0555      7(C6A )

8(O6A )   7.3738    3.6011   -0.0616       0.90932   0.57334  -0.00593        3.3235      2.91   -0.0616      8(O6A )

9(O5A )   4.9166    2.9539   -2.6560       0.60630   0.45262  -0.25587        0.9858    -29.02   -2.6560      9(O5A )

10(C1A )   3.7120    3.5447   -3.1479       0.45776   0.50897  -0.30327        0.3606    161.81   -3.1479     10(C1A )

11(O4A*)   3.3954    2.9057   -4.3398       0.41871   0.42272  -0.41810        0.8436   -141.39   -4.3398     11(O4A*)

12(C2A )   2.5810    3.2978   -2.1583       0.31828   0.45989  -0.20793        1.4797   -174.79   -2.1583     12(C2A )

13(O2A )   1.3822    3.8778   -2.6599       0.17045   0.51495  -0.25626        2.7092    170.53   -2.6599     13(O2A )

14(C3A )   2.9418    3.8778   -0.7994       0.36278   0.53932  -0.07701        1.1986    158.17   -0.7994     14(C3A )

15(O3A )   1.9199    3.5557    0.1461       0.23675   0.48237   0.01407        2.1383    176.69    0.1461     15(O3A )

16(C4A.)   4.2775    3.3073   -0.3410       0.52749   0.48759  -0.03285        0.2555    -29.26   -0.3410     16(C4A.)

17(C5A.)   5.3457    3.5206   -1.4082       0.65922   0.53142  -0.13567        1.2942      3.92   -1.4082     17(C5A.)

18(H1A )   3.8587    4.6278   -3.2719       0.47585   0.64906  -0.31521        1.2116     99.30   -3.2719     18(H1A )

19(H2A )   2.4045    2.2161   -2.0654       0.29651   0.31951  -0.19898        2.0498   -143.61   -2.0654     19(H2A )

(CYCLE  1)  Page  10

20(H3A )   3.0089    4.9732   -0.8741       0.37105   0.67973  -0.08421        1.8624    124.16   -0.8741     20(H3A )

21(H4A )   4.1533    2.2317   -0.1465       0.51217   0.34882  -0.01411        1.2045    -85.30   -0.1465     21(H4A )

22(H5A )   5.5258    4.5981   -1.5367       0.68143   0.67132  -0.14805        1.8772     38.40   -1.5367     22(H5A )

23(H61A)   6.4796    1.8363   -0.7079       0.79905   0.33485  -0.06820        2.9030    -33.35   -0.7079     23(H61A)

24(H62A)   7.2763    2.7604   -1.9630       0.89730   0.46486  -0.18911        3.2911    -11.78   -1.9630     24(H62A)

-===  Constraints and Restraints  ===

No   Type  Applied  Value   Weight    W*D**2    Want    Have    Atom    TFM Mol    X        Y        Z

TFM  1 Defined,     0.0*A    0.0*B    1.0*H

1  COI CON  YES   0.0000    0.000  0.00E+00 X                  1(O5A*)   0   1  -0.8336   0.5261   5.1900

2  COI CON  YES   0.0000    0.000  0.00E+00 Y                  9(O5A )   1   1  -0.8336   0.5261   5.1900

3  COI CON  YES   0.0000    0.000  0.00E+00 Z

4  COI CON  YES   0.0000    0.000  0.00E+00 X                  2(C1A*)   0   1   0.3356  -0.1317   4.6980

5  COI CON  YES   0.0000    0.000  0.00E+00 Y                 10(C1A )   1   1   0.3356  -0.1317   4.6980

6  COI CON  YES   0.0000    0.000  0.00E+00 Z

7  COI CON  YES   0.0000    0.000  0.00E+00 X                  3(O4A )   0   1   0.6879   0.4884   3.5061

8  COI CON  YES   0.0000    0.000  0.00E+00 Y                 11(O4A*)   1   1   0.6879   0.4884   3.5061

9  COI CON  YES   0.0000    0.000  0.00E+00 Z

10  COI CON  YES   0.0000    0.000  0.00E+00 X                 14(C3A )   0   1  -1.0858   0.5078   1.8566

11  COI CON  YES   0.0000    0.000  0.00E+00 Y                  6(C3A.)   0   1  -1.0858   0.5078   1.8566

12  COI CON  YES   0.0000    0.000  0.00E+00 Z

13  COI CON  YES   0.0000    0.000  0.00E+00 X                  4(C4A )   0   1   0.2155  -0.1373   2.3150

14  COI CON  YES   0.0000    0.000  0.00E+00 Y                 16(C4A.)   0   1   0.2155  -0.1373   2.3150

15  COI CON  YES   0.0000    0.000  0.00E+00 Z

16  COI CON  YES   0.0000    0.000  0.00E+00 X                  5(C5A )   0   1   1.2941   0.0154   1.2477

17  COI CON  YES   0.0000    0.000  0.00E+00 Y                 17(C5A.)   0   1   1.2941   0.0154   1.2477

18  COI CON  YES   0.0000    0.000  0.00E+00 Z

19  PAR CON  NO    0.0565    0.000  0.00E+00    0.00    3.24  OTHR(  9)   OTHR( 13)

* -1.0000   *  1.0000

Sum of Moduli of Lagrange Constraints - All   0.0566, Applied   0.0001

-Elastically Bound Parameters

Parameter            Current    Bound      Diff     W*D**2

THTA(  4)            -95.923   -90.000     5.923   1.58E-01

THTA(  5)           -143.774  -120.000    23.774   2.54E+00

THTA(  8)            -75.829   -60.000    15.829   1.13E+00

THTA(  9)           -176.416  -177.000    -0.584   9.66E-03

THTA( 10)             60.600    62.400     1.800   1.01E-01

THTA( 12)            -61.922   -62.800    -0.878   3.19E-02

THTA( 14)             58.325    57.500    -0.825   1.93E-02

THTA( 16)            -54.879   -53.200     1.679   1.17E-01

THTA( 17)             53.647    52.200    -1.447   8.66E-02

TAU(  4)            115.905   116.500     0.595   3.98E-02

TAU(  5)            107.840   108.600     0.760   7.28E-02

TAU(  9)            110.092   110.160     0.068   1.22E-03

(CYCLE  1)  Page  11

TAU( 10)            112.822   112.270    -0.552   1.37E-01

TAU( 12)            109.590   109.340    -0.250   2.33E-02

TAU( 14)            110.031   110.530     0.499   6.62E-02

TAU( 16)            109.519   110.300     0.781   1.40E-01

TAU( 17)            110.565   110.170    -0.395   3.58E-02

CNTCTS91:IFK(44)= 1, IFK(51)= 0

-===  Contact Search Setup ===

-Source Molecules Generated

Source   Original    From       To

Mol No    Mol No    Residue   Residue   S-Code

S    1         1         0         0         1

-Target Molecules Generated

Target   Original    From       To         Units of Displacement      Multi-    Combines With

Mol No    Mol No    Residue   Residue   Along A   Along B   Along C   Plicity   Source Mols ...

T    1         1         0         1         0         0         0         1       1

T    2         1        -5         5         1         0         0         1       1

T    3         1        -5         5         0         1         0         1       1

T    4         2        -5         5         0         0         0         1       1

T    5         2        -5         5        -1         0         0         1       1

T    6         2        -5         5         0        -1         0         1       1

T    7         2        -5         5        -1        -1         0         1       1

-===  Contact List Creation  ===

--------  S o u r c e  ---------  --------  T a r g e t  ---------   Axis   Reject  Multi-  Rela-     Contacts     Min

Mol  Res    Z-Low   Z-High  Scan  Mol  Res    Z-Low   Z-High  Scan  Separn  Reason Plicity  Tion   Tested  Found   Dis

S 1   0   -0.6159   3.5061   UP   T  1  0   -0.6159   3.5061   UP     0.00             1    SAME     169     34    2.41

S 1   0   -0.6159   3.5061   UP   T  1  1    4.5741   8.6961   UP     0.00             1              72     12    2.07

S 1   0   -0.6159   3.5061   UP   T  2  0   -0.6159   3.5061   UP     8.17             1             322      3    2.43

S 1   0   -0.6159   3.5061   UP   T  4  0   -3.2719   0.8502   UP     5.31             1             228      1    2.69

S 1   0   -0.6159   3.5061   UP   T  4  1    1.9181   6.0402   UP     5.31             1             248      4    2.53

S 1   0   -0.6159   3.5061   UP   T  5  0   -3.2719   0.8502   UP     6.00             1             228      4    2.22

S 1   0   -0.6159   3.5061   UP   T  5  1    1.9181   6.0402   UP     6.00             1             248      1    2.54

S 1   0   -0.6159   3.5061   UP   T  6  0   -3.2719   0.8502   UP     6.00             1             228      1    2.57

S 1   0   -0.6159   3.5061   UP   T  6  1    1.9181   6.0402   UP     6.00             1             248      1    2.45

S 1   0   -0.6159   3.5061   UP   T  7  0   -3.2719   0.8502   UP     5.31             1             228      5    2.44

S 1   0   -0.6159   3.5061   UP   T  7  1    1.9181   6.0402   UP     5.31             1             248      1    2.50

..Residue Pairs..   ...Atom Pairs...    Bonds     Common    Coinci-  Contacts

Total    Searched   Total    Searched   Found     Bonds     Dences    Found

68        16     22032      3055        19        38        18        67

-===  Contact List Application  ===

--------------  S o u r c e  ---------------  --------------  T a r g e t  ---------------

Mol  Res  Atom       X        Y        Z      Mol  Res  Atom       X        Y        Z       Dist Limit  Diff Mul W*D**2

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  0   7(C6A )   2.5726  -0.7148   1.6005   2.94  3.42  0.48  1 1.63E+00

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  0   8(O6A )   3.3235  -0.0187   2.5944   2.83  3.24  0.41  1 1.31E+00

ADI Type  1, Donor on TRGT, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is   79.75

S 1   0   3(O4A )   0.6879   0.4884   3.5061  T  1  1  13(O2A )   2.6429  -0.5957   5.1860   2.80  3.24  0.44  1 1.57E+00

(CYCLE  1)  Page  12

ADI Type  1, Donor on TRGT, Prcrsr is  12(C2A ) at    1.4788   0.0509   5.6876, Angle is   57.56

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  0  10(C1A )  -0.3356   0.1317  -0.4920   2.87  3.60  0.73  1 3.27E+00

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  0  23(H61A)   2.3311  -1.7301   1.9481   2.67  3.10  0.43  1 1.08E+00

S 1   0   4(C4A )   0.2155  -0.1373   2.3150  T  1  1  18(H1A )   0.1280  -1.2048   4.5741   2.50  3.10  0.60  1 2.14E+00

S 1   0   5(C5A )   1.2941   0.0154   1.2477  T  1  0  12(C2A )  -1.4788  -0.0509   0.4976   2.87  3.60  0.73  1 3.26E+00

S 1   0   5(C5A )   1.2941   0.0154   1.2477  T  1  0  18(H1A )  -0.1280   1.2048  -0.6159   2.63  3.10  0.47  1 1.32E+00

S 1   0   8(O6A )   3.3235  -0.0187   2.5944  T  1  1  13(O2A )   2.6429  -0.5957   5.1860   2.74  2.90+ 0.16  1 7.59E-01

ADI Type  1, Donor on SRCE, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is  115.16

S 1   0   8(O6A )   3.3235  -0.0187   2.5944  T  2  0  15(O3A )   5.9848  -0.7518   2.8020   2.77  2.90+ 0.13  1 5.21E-01

ADI Type  1, Donor on SRCE, Prcrsr is   7(C6A ) at    2.5726  -0.7148   1.6005, Angle is  115.40

S 1   0  14(C3A )  -1.0858   0.5078   1.8566  T  1  0   9(O5A )   0.8336  -0.5261   0.0000   2.86  3.42  0.56  1 2.17E+00

S 1   0  15(O3A )  -2.1243   0.2438   2.8020  T  1  1   9(O5A )  -0.8336   0.5261   5.1900   2.73  2.90+ 0.17  1 8.76E-01

ADI Type  1, Donor on SRCE, Prcrsr is  14(C3A ) at   -1.0858   0.5078   1.8566, Angle is  102.48

S 1   0  15(O3A )  -2.1243   0.2438   2.8020  T  7  1  19(H2A )  -2.4045  -2.2161   3.1246   2.50  2.92  0.42  1 1.23E+00

S 1   0  19(H2A )  -1.7161  -1.1210   0.5906  T  7  0   3(O4A )  -3.3953  -2.9057   0.8502   2.46  2.92  0.46  1 1.42E+00

S 1   0  19(H2A )  -1.7161  -1.1210   0.5906  T  7  0   8(O6A )  -0.7353  -3.2632  -0.0616   2.44  2.92  0.48  1 1.55E+00

S 1   0  21(H4A )   0.0308  -1.2041   2.5094  T  1  1  10(C1A )   0.3356  -0.1317   4.6980   2.46  3.10  0.64  1 2.46E+00

S 1   0  21(H4A )   0.0308  -1.2041   2.5094  T  1  1  18(H1A )   0.1280  -1.2048   4.5741   2.07  2.60  0.53  1 1.77E+00

S 1   0  22(H5A )   1.5347   1.0811   1.1192  T  1  0  10(C1A )  -0.3356   0.1317  -0.4920   2.64  3.10  0.46  1 1.23E+00

S 1   0  23(H61A)   2.3311  -1.7301   1.9481  T  6  1   9(O5A )   3.1925  -3.9496   2.5340   2.45  2.92  0.47  1 1.50E+00

S 1   0  24(H62A)   3.1787  -0.8525   0.6930  T  1  0   9(O5A )   0.8336  -0.5261   0.0000   2.47  2.92  0.45  1 1.40E+00

S 1   0  24(H62A)   3.1787  -0.8525   0.6930  T  2  0  13(O2A )   5.4662  -0.4000  -0.0040   2.43  2.92  0.49  1 1.62E+00

Contacts   Invar-   Contacts  Shortest   Worst

In List    Iants    Applied   Contact     Diff

67         0        67      2.07      0.73

-===  X-ray Data Application ===

Scattering Types

Atom   Core H  W        Atom   Core H  W        Atom   Core H  W        Atom   Core H  W

1(O5A*)  O              7(C6A )  C             13(O2A )  O             19(H2A )  H

2(C1A*)  C              8(O6A )  O             14(C3A )  C             20(H3A )  H

3(O4A )  O              9(O5A )  O             15(O3A )  O             21(H4A )  H

4(C4A )  C             10(C1A )  C             16(C4A.)  C             22(H5A )  H

5(C5A )  C             11(O4A*)  O             17(C5A.)  C             23(H61A)  H

6(C3A.)  C             12(C2A )  C             18(H1A )  H             24(H62A)  H

Structure Factor Amplitudes - Trig

Spot Refl     H   K   L        A       B     Mod     Phase  Multy  F-calc   F-obs    Diff     Weight     W*D**2   Flag

1    1     0   1   0     0.22    0.00    0.31      0.00     2     0.30    8.19    7.89   2.08E-02   1.74E-01   UR

2    2     1   0   0     0.97    0.00    1.37      0.00     2     1.34    8.19    6.85   2.08E-02   1.31E-01   UR

3    3     1  -1   0    76.99    0.00  108.88      0.00     2   105.17  106.53    1.36   2.08E-02   5.13E-03

4    4     1   1   0    71.63    0.00  101.29      0.00     2    96.91   73.75  -23.16   2.08E-02   1.50E+00

5    5     2   0   0   -11.54    0.00   16.33   -180.00     2    15.13   13.66   -1.48   2.08E-02   6.08E-03   U

6    6     0   2   0   205.78    0.00  291.01      0.00     2

7     2  -1   0     1.82    0.00    2.58      0.00     2   266.83  273.15    6.32   2.08E-02   1.12E-01

(CYCLE  1)  Page  13

7    8     1  -2   0   -11.64    0.00   16.46    180.00     2    15.02    8.19   -6.83   2.08E-02   1.30E-01   U

8    9     2   1   0    -1.94    0.00    2.75    180.00     2     2.47    8.19    5.72   2.08E-02   9.14E-02   UR

9   10     1   2   0    10.15    0.00   14.36      0.00     2    12.85    8.19   -4.66   2.08E-02   6.06E-02   U

10   11     2  -2   0   -11.49    0.00   16.25    180.00     2    14.15    8.19   -5.95   2.08E-02   9.89E-02   U

11   12     3   0   0     0.12    0.00    0.17      0.00     2     0.15    8.19    8.05   2.08E-02   1.81E-01   UR

12   13     3  -1   0    25.96    0.00   36.71      0.00     2

14     2   2   0   -13.54    0.00   19.15   -180.00     2    34.68   38.24    3.56   2.08E-02   3.54E-02

13   15     0   3   0     1.56    0.00    2.21      0.00     2

16     1  -3   0    28.68    0.00   40.56      0.00     2    33.62   40.97    7.35   2.08E-02   1.51E-01

14   17     3   1   0    16.00    0.00   22.63      0.00     2    18.42   32.78   14.36   2.08E-02   5.76E-01

15   18     1   3   0    25.21    0.00   35.65      0.00     2

19     3  -2   0    -3.06    0.00    4.33   -180.00     2    28.71   30.05    1.34   2.08E-02   4.99E-03

16   20     2  -3   0     6.09    0.00    8.61      0.00     2     6.83   10.93    4.09   2.08E-02   4.68E-02   UR

17   21     3   2   0    -0.65    0.00    0.91   -180.00     2     0.69   10.93   10.24   2.08E-02   2.93E-01   UR

18   22     2   3   0    -1.11    0.00    1.58    180.00     2     1.18   10.93    9.75   2.08E-02   2.65E-01   UR

19   23     4   0   0   -34.00    0.00   48.08   -180.00     2

24     4  -1   0     0.52    0.00    0.74      0.00     2    35.45   38.24    2.79   2.08E-02   2.18E-02

20   25     3  -3   0    29.33    0.00   41.48      0.00     2    30.36   24.58   -5.78   2.08E-02   9.32E-02   U

21   26     1  -4   0   -12.26    0.00   17.34    180.00     2

27     0   4   0    77.84    0.00  110.08      0.00     2    80.27   98.33   18.07   2.08E-02   9.12E-01

22   28     4   1   0     2.87    0.00    4.05      0.00     2     2.87   24.58   21.71   2.08E-02   1.32E+00   UR

23   29     4  -2   0   -47.60    0.00   67.32   -180.00     2    47.57   24.58  -22.99   2.08E-02   1.48E+00   U

24   30     1   0   1     0.27  -16.81   23.78    -89.08     2

31     0   1   1    -1.52  -15.15   21.53    -95.71     2    31.07   21.85   -9.22   2.08E-02   2.37E-01

25   32     1  -1   1    -7.05    0.04    9.98    179.68     2     9.53   13.66    4.13   2.08E-02   4.77E-02   UR

26   33     1   1   1   -19.49   -4.85   28.41   -166.04     2    26.86   16.39  -10.47   2.08E-02   3.06E-01

27   34     2   0   1    10.87   -5.29   17.10    -25.94     2    15.66   19.12    3.46   2.08E-02   3.34E-02

28   35     0   2   1   -23.59    6.70   34.69    164.15     2    31.59   13.66  -17.93   2.08E-02   8.98E-01   U

29   36     2  -1   1     6.70   16.88   25.69     68.35     2

37     1  -2   1     1.41    3.21    4.96     66.32     2    23.60   30.05    6.44   2.08E-02   1.16E-01

30   38     2   1   1    -1.63    4.53    6.81    109.84     2     6.05   13.66    7.61   2.08E-02   1.62E-01   UR

(CYCLE  1)  Page  14

31   39     1   2   1     7.69  -16.67   25.96    -65.25     2    22.98   13.66   -9.32   2.08E-02   2.43E-01   U

32   40     2  -2   1    18.42   -5.49   27.18    -16.59     2    23.39   13.66   -9.73   2.08E-02   2.64E-01   U

33   41     3   0   1   -33.30  -32.98   66.28   -135.28     2    55.23   68.29   13.06   2.08E-02   4.76E-01

34   42     3  -1   1   -38.15   35.84   74.02    136.79     2

43     2   2   1     2.28    8.26   12.11     74.59     2    62.11   62.82    0.72   2.08E-02   1.44E-03

35   44     0  -3   1     4.23   27.13   38.83     81.13     2

45    -1   3   1   -12.20    1.01   17.31    175.25     2    34.78   65.56   30.77   2.08E-02   2.64E+00

36   46     3   1   1   -20.41   27.93   48.93    126.16     2    39.37   40.97    1.60   2.08E-02   7.17E-03

37   47     1   3   1   -14.33  -17.88   32.40   -128.72     2    25.75   19.12   -6.63   2.08E-02   1.23E-01

38   48     3  -2   1   -36.12  -35.80   71.92   -135.26     2

49     2  -3   1    -0.03   23.09   32.65     90.08     2    61.94   49.17  -12.77   2.08E-02   4.55E-01

39   50     3   2   1   -21.08   -8.60   32.20   -157.81     2    24.02   24.58    0.57   2.08E-02   8.94E-04   UR

40   51     2   3   1    -8.93   -8.98   17.92   -134.85     2    13.26   24.58   11.32   2.08E-02   3.58E-01   UR

41   52     4   0   1   -13.25    1.70   18.89    172.68     2

53     4  -1   1    -5.29  -25.37   36.65   -101.77     2    30.04   24.58   -5.46   2.08E-02   8.32E-02   U

42   54     0   0   2    -1.52   -0.54    1.62   -160.32     1     1.54 2728.77 2727.22   2.08E-02   2.08E+04   UR

43   55     1   0   2    20.94   -9.45   32.50    -24.29     2

56     0   1   2   -11.27   34.01   50.66    108.33     2    56.30   51.90   -4.40   2.08E-02   5.41E-02

44   57     1  -1   2     1.95   -0.89    3.03    -24.40     2     2.80   13.66   10.86   2.08E-02   3.29E-01   UR

45   58     1   1   2     0.39    3.13    4.46     82.82     2     4.07   13.66    9.59   2.08E-02   2.57E-01   UR

46   59     2   0   2     1.39   -0.19    1.98     -7.75     2     1.75   13.66   11.91   2.08E-02   3.96E-01   UR

47   60     0   2   2    -4.32   -1.43    6.44   -161.67     2     5.66   13.66    8.00   2.08E-02   1.78E-01   UR

48   61     2  -1   2    -1.14  -35.78   50.63    -91.83     2

62     1  -2   2    -2.68  -15.99   22.93    -99.52     2    48.43   60.09   11.66   2.08E-02   3.80E-01

49   63     2   1   2   -12.31  -28.19   43.50   -113.58     2

64     1   2   2    31.19    7.81   45.47     14.07     2    53.79   62.82    9.04   2.08E-02   2.28E-01

50   65     2  -2   2    -1.80   -0.02    2.55   -179.42     2     2.12   16.39   14.27   2.08E-02   5.68E-01   UR

51   66     3   0   2   -10.05  -23.58   36.25   -113.08     2

67     3  -1   2     2.32   -0.85    3.50    -20.03     2

68     2   2   2     0.70    0.35    1.11     26.23     2    29.14   27.32   -1.83   2.08E-02   9.31E-03

52   69     0   3   2     2.77   17.29   24.76     80.91     2    19.66   16.39   -3.27   2.08E-02   2.98E-02   U

53   70     1  -3   2     4.66   -2.40    7.42    -27.27     2     5.86   16.39   10.52   2.08E-02   3.09E-01   UR

(CYCLE  1)  Page  15

54   71     3   1   2     1.06    0.96    2.03     42.30     2     1.57   16.39   14.81   2.08E-02   6.13E-01   UR

55   72     1   3   2     3.20    3.48    6.69     47.37     2     5.14   16.39   11.25   2.08E-02   3.53E-01   UR

56   73     3  -2   2   -34.90  -30.93   65.94   -138.45     2

74     2  -3   2    12.88  -14.51   27.43    -48.41     2    54.09   68.29   14.20   2.08E-02   5.63E-01

57   75     3   2   2    25.83    0.62   36.54      1.38     2    26.32   24.58   -1.74   2.08E-02   8.41E-03   U

58   76     2   3   2    -4.95   -2.12    7.61   -156.81     2     5.44   24.58   19.14   2.08E-02   1.02E+00   UR

59   77     4   0   2     1.88    0.57    2.78     16.83     2

78     4  -1   2   -29.87  -29.23   59.11   -135.62     2    41.64   27.32  -14.33   2.08E-02   5.73E-01

60   79     1   0   3   -23.38   -5.05   33.82   -167.81     2

80     0   1   3     4.10   -2.15    6.55    -27.69     2    30.41   49.17   18.75   2.08E-02   9.82E-01

61   81     1  -1   3     7.58  -17.56   27.05    -66.64     2    23.54   35.51   11.97   2.08E-02   4.00E-01

62   82     1   1   3     1.37  -33.98   48.10    -87.68     2    41.46   35.51   -5.95   2.08E-02   9.88E-02

63   83     2   0   3   -45.74   26.05   74.44    150.33     2

84     0   2   3     8.64   12.32   21.28     54.95     2    64.26   73.75    9.49   2.08E-02   2.51E-01

64   85     2  -1   3    17.62   37.63   58.76     64.91     2

86     1  -2   3   -13.48   -5.84   20.77   -156.55     2    51.25   62.82   11.57   2.08E-02   3.74E-01

65   87     2   1   3    24.44   32.52   57.53     53.08     2

88     1   2   3   -29.12    2.34   41.31    175.40     2    57.13   38.24  -18.89   2.08E-02   9.96E-01

66   89     2  -2   3   -27.28   12.94   42.70    154.63     2    33.49   16.39  -17.10   2.08E-02   8.17E-01   U

67   90     3   0   3    37.71   12.03   55.97     17.69     2

91     3  -1   3     0.77   29.67   41.97     88.51     2

92     2   2   3   -32.64   21.77   55.49    146.30     2    67.39   65.56   -1.84   2.08E-02   9.42E-03

68   93     0   3   3     9.09   -2.89   13.49    -17.65     2

94     1  -3   3    10.50   -9.88   20.39    -43.26     2    18.23   60.09   41.86   2.08E-02   4.89E+00

69   95     3   1   3   -16.18   49.20   73.25    108.21     2    53.73   38.24  -15.48   2.08E-02   6.69E-01

70   96     0   0   4    67.95   -8.56   68.48     -7.18     1

97     0   1   4     0.97    5.51    7.91     79.96     2

98     1   0   4     0.97   -1.38    2.39    -54.74     2    56.23 2728.77 2672.53   2.08E-02   1.99E+04   UR

71   99     1  -1   4   -11.84    9.36   21.34    141.66     2

100     1   1   4    -7.35  -26.16   38.43   -105.70     2    34.94   21.85  -13.09   2.08E-02   4.78E-01

72  101     2   0   4     1.78  -22.81   32.35    -85.54     2

102     0   2   4    56.65   -6.84   80.69     -6.88     2

103     2  -1   4     3.18   -2.62    5.83    -39.45     2

104     1  -2   4    -7.30   -0.29   10.33   -177.76     2    66.53   76.48    9.95   2.08E-02   2.76E-01

73  105     2   1   4     1.14    0.85    2.01     36.75     2     1.50   24.58   23.08   2.08E-02   1.49E+00   UR

(CYCLE  1)  Page  16

74  106     1   2   4     4.52    0.97    6.54     12.14     2     4.86   24.58   19.72   2.08E-02   1.09E+00   UR

75  107     2  -2   4   -33.22    3.60   47.26    173.82     2    34.18   24.58   -9.60   2.08E-02   2.57E-01   U

76  108     3   0   4     2.50    1.54    4.16     31.57     2     2.91   24.58   21.67   2.08E-02   1.31E+00   UR

77  109     3  -1   4    24.21  -17.26   42.04    -35.48     2

110     2   2   4    25.31  -41.58   68.84    -58.67     2    56.14   49.17   -6.97   2.08E-02   1.36E-01

-X-ray Data Statistics

Excl Rej-Unobs    Excl Unobs

No of Spots                 51            37

Sum of Scale*Fo        2264.41       2029.50

Sum of Fc              2281.57       1924.84

Sum of Abs Delta        516.53        394.71

R-Factors

Normal                 22.81         19.45

Weighted Quadratic     21.54         19.63

Weighted Normal        22.81         19.45

spots Rejected from Least Squares --   0 Observed,  26 Unobserved

-Normal Scattering from Unit Cell --    332.0 Electrons

-===  Solution of Normal Equations  ===

-Eigen Values

1.35E+04  7.31E+03  6.16E+03  4.34E+03  1.81E+03  1.54E+03  1.05E+03  1.04E+03  7.69E+02  7.61E+02

6.52E+02  5.86E+02  4.48E+02  3.81E+02  3.40E+02  2.45E+02  2.19E+02  1.53E+02  1.40E+02  1.22E+02

7.65E+01  6.01E+01  3.80E+01  2.55E+01  2.02E+01 -1.81E-01  1.69E-01 -1.34E-01 -9.40E-02 -4.96E-02

-3.63E-02 -3.05E-02 -2.65E-02 -1.07E-02 -9.29E-03 -5.26E-03 -3.97E-03 -2.71E-03 -9.92E-04  1.83E-12

-5.84E-13 -2.38E-13 -8.26E-14  3.42E-14

Last  5 Filteres - Goodliness is   0.00 - Residual is  5.42E-16

-  No of     Varied   Applied   EignVals    Data    Degs of

Data     Params   Cnstrnts  Filtered   Excess   Freedom

135        26        18         5       122        13

Weight Analysis      Restnts   El Pare    Bragg   Continuous Contacts  Overall

Sigam (WT*Delta**2)  0.00E+00  4.71E+00  2.36E+01  0.00E+00  5.55E+01  8.38E+01

Data Contributrion          0        17        51         0        67       135

Mean (WT*Delta**2)   0.00E+00  2.77E-01  4.62E-01  0.00E+00  8.29E-01  6.21E-01

(CYCLE  1)  Page  17

-Shifts in Parameters - Shift Scaling Factor is 0.500

Parameter             Shift      New      Bound

THTA(  4)              0.000   -95.923   -90.000

THTA(  5)             -0.002  -143.776  -120.000

THTA(  8)              0.004   -75.825   -60.000

THTA(  9)             -0.011  -176.428  -177.000

THTA( 10)              0.009    60.609    62.400

THTA( 12)              0.000   -61.922   -62.800

THTA( 14)             -0.010    58.315    57.500

THTA( 16)              0.008   -54.871   -53.200

THTA( 17)              0.003    53.650    52.200

TAU(  4)              0.002   115.907   116.500

TAU(  5)              0.002   107.842   108.600

TAU(  9)             -0.005   110.087   110.160

TAU( 10)             -0.001   112.822   112.270

TAU( 12)              0.002   109.592   109.340

TAU( 14)              0.002   110.032   110.530

TAU( 16)              0.001   109.520   110.300

TAU( 17)             -0.001   110.564   110.170

OTHR(  1) SCLE( 0)    0.0000    2.7315

OTHR(  2) ATTN( 0)    0.0061    4.9979

OTHR(  5) EP-X( 1)   -0.0025  120.3084

OTHR(  6) EP-Y( 1)   -0.0049   60.3245

OTHR(  7) EP-Z( 1)    0.0033  -81.8836

OTHR(  8)    S( 1)   -0.0001    0.9857

OTHR(  9)   MU( 1)    0.0087  -32.2489

OTHR( 13)   MU( 2)    0.0033  -29.0191

OTHR( 16)    W( 2)    0.0000   -0.2558

----  Dump Record Made ( 1)  ---

-Convergence Test, Mean Shift is  0.000 - YES

-===  CPU Seconds for Cycle  1  ===

Cycler    Coords    Contrr    Elastic   Cntcts    Sfactor   Filmat     Solve     Total

0.00      0.00      0.00      0.00      0.00      0.00      0.00      0.00      0.00

(CYCLE  2)  Page  18

-                              -----  Start of Cycle  2  (used   0.0 Seconds)  -----

-===  Current Values of Parameters  ===

1         2         3         4         5         6         7         8         9        10

Theta      0.000     0.000     0.000   -95.923  -143.776    95.334    64.282   -75.825  -176.428    60.609

Tau        0.000     0.000   107.300   115.907   107.842   110.400   112.698   111.799   110.087   112.822

Other    2.73150   4.99791   0.00000   0.00000 120.30840  60.32446 -81.88365   0.98573 -32.24894   0.00000

SCLE( 0)  ATTN( 0)  SCLE( 1)  ATTN( 1)  EP-X( 1)  EP-Y( 1)  EP-Z( 1)     S( 1)    MU( 1)     U( 1)

11        12        13        14        15        16        17        18        19        20

Theta   -179.282   -61.922  -179.475    58.315  -175.751   -54.871    53.650    58.048   -62.355    65.669

Tau      107.300   109.592   109.300   110.032   109.602   109.520   110.564   109.480   109.480   109.480

Other    0.00000   0.00000 -29.01912   0.50000   0.50000  -0.25583

V( 1)     W( 1)    MU( 2)     U( 2)     V( 2)     W( 2)

21        22        23        24

Theta    -24.286   -55.988    46.615   161.735

Tau      109.480   109.480   109.480   109.480

-===  Cycle Control Data  ===

-Cycle Control Card -

--- ZZ.

----  SC Output at Cycle  2  ---

(CYCLE  2)  Page  19

-===  Run Summary  ===

-  Cycle     Sum of    Worst    Elastic   Contacts  Contacts  Contact

Number   Constrts  Constrt    Sigma    In Table  Applied    Sigma

--        --        --        --        --        --        --

Bragg   Continuous Overall    No of    Net Vard   Shift    Time for

R-Factor  R-factor   Sigma      Data     Params    Scaler    Cycle

--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

1    0.0001    0.0000  4.71E+00        67        67  5.55E+01

22.81      0.00  8.38E+01       135        13     0.500      0.00

--- ZZ.

-===  Array Specifiers Used/Given  ===

ADEL    6/  150, APPR   18/  600, ATOM   24/ 3000, BNDI   10/ 6000, BNDL   88/ 4000,

CLIS   67/10000, CMOL    8/  500, COND   19/  600, CONP   44/ 2000, CONT    1/   50,

EPAR   17/  410, EQIV    1/   10, EQUN   44/  999, HBND    1/ 2000, MOLC    2/ 5000,

ORDR    1/  100, POSN    2/  100, REFL  110/30000, SCAT    8/   50, SOUM    1/  500,

SPOT   77/15000, STRU    1/  100, TARM    7/ 2000, THEQ   12/ 1000, TYPE    3/   50,

VARY    9/  500, VPAR   26/  500, VSKL   21/  500,

-===   WinLALS Happy Ending   ===

Total CPU Time =      0 seconds ( =  0 H  0 M  0 S)

End of This Job

j) Output 5 of Input file 2: LALS2.pnc

Cycle=  2///////////////  WinLALS 14:59:17 01-11- 6  ///////////////SC=  1

L down P21   April 1997 Finkenstadt

O5A*               3    0.5000    0.0000    0.0000    0.0000

C1A*      O5A*     2    0.5000    1.4290    0.0000    0.0000

O4A       C1A*     3    0.5000    1.3890  107.3000    0.0000

C4A       O4A      2    0.5000    1.4260  115.9074  -95.9230

C5A       C4A      2    0.5000    1.5250  107.8422 -143.7763

C3A.      C4A      2    0.5000    1.5230  110.4000   95.3337

C6A       C5A      2    1.0000    1.5140  112.6980   64.2823

O6A       C6A      3    1.0000    1.4270  111.7990  -75.8247

O5A       C5A      3    0.5000    1.4360  110.0871 -176.4277

C1A       O5A      2    0.5000    1.4290  112.8217   60.6092

O4A*      C1A      3    0.5000    1.3890  107.3000 -179.2821

C2A       C1A      2    1.0000    1.5230  109.5922  -61.9221

O2A       C2A      3    1.0000    1.4230  109.3000 -179.4755

C3A       C2A      2    0.5000    1.5210  110.0323   58.3145

O3A       C3A      3    1.0000    1.4290  109.6020 -175.7511

C4A.      C3A      2    0.5000    1.5230  109.5203  -54.8711

C5A.      C4A.     2    0.5000    1.5250  110.5637   53.6498

H1A       C1A      1    1.0000    1.1000  109.4800   58.0479

H2A       C2A      1    1.0000    1.1000  109.4800  -62.3555

H3A       C3A      1    1.0000    1.1000  109.4800   65.6689

H4A       C4A      1    1.0000    1.1000  109.4800  -24.2863

H5A       C5A      1    1.0000    1.1000  109.4800  -55.9877

H61A      C6A      1    1.0000    1.1000  109.4800   46.6153

H62A      C6A      1    1.0000    1.1000  109.4800  161.7353

O5A         120.3084   60.3245  -81.8836    0.9857

2.7315    4.9979    0.0000    0.0000    1.0000

-32.2489    0.0000    0.0000  0.000000    1.0000    1.0000

-29.0191    0.5000    0.5000 -0.255835    1.0000    1.0000

k) Output 6 of Input file 2: LALS2.err

WinLALS Run of 2001-11- 6 (14:59:17)

Blackwell valonia cellulose  PARALLEL down P21   April 1997 Finkenstadt

.--- CL=6,EH=9,ET=8,VO=9,AC=18,CR=3,CD=1,CS,XR,XS,SH,EL,FI,CN.

.        1    0.0001    0.0000  4.71E+00        67        67  5.55E+01

.    22.81      0.00  8.38E+01       135        13     0.500      0.00

.--- ZZ.

----  SC Output at Cycle  2  ---

Finish of WinLALS (Version 1.0) user's Guide.